| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Propose MAC PDU Header Format for 802.16m |  |
| Date Submitted | 2008-09-05 |  |
| Source(s) | Xiao Xu, Hua Xu Motorola 1501 West Shure Drive Arlington Heights, IL 60004 | Voice: 1 847 632 2176 E-mail: hua.xu@motorola.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | MAC: Data Plane; in response to the TGm Call for Contributions and Comments 802.16m-08/003r4 for Session 57 |  |
| Abstract | Proposal for IEEE 802.16m MAC PDU Header Format that Reduces Overhead |  |
| Purpose | Discussion and Approval |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Propose MAC PDU Header Format for 802.16m

Xiao Xu, Hua Xu

Motorola

# 1  Instructions

The generic MAC header (GMH) in IEEE 802.16e-2005 reference system is 6 bytes, which needs to be present in every MAC PDU. Lots of other overheads also need to be added, including subheaders, extended subheaders and CRCs. For some applications with small payload sizes such as VoIP, this significantly reduces efficiency and degrades system capacity.

The 16e Generic MAC Header (GMH) is illustrated by Figure 19 below. Careful analysis will show that many of the fields can be reduced or eliminated without functional impairment to the system.

.

# 2   Detailed Discussion of MAC Headers

2.1  Generic MAC Header Field

The following changes will happen to generic MAC header fields:

- Header type (HT) – Removed.

- Message length (LEN) – needed when multiple PDUs are sent in one MAC header; not needed if a HARQ burst only contains one MAC PDU since the size of the HARQ burst is known.

- CID – if the flow ID (transport CID) is used in the resource allocation, we do not need to repeat this every packet unless the allocation is for multiple flows for a single MS. In that case, a shorter CID index can be used to identify flows within one MS.

- EC – can be removed from the header and exchanged during connection establishment process only.  No need to be transmitted in every MAC PDU.

- CI – If only one MAC PDU is sent in each HARQ burst, there is no need of MAC PDU CRC, there for no need for CI bit.

- Type – after changing all the subheaders and extended subheaders to the subheader format, we don’t need these Type bits for existing 5 subheaders. For example we don’t need a bit to indicate whether fragmentation subheader is present or not. It will be covered the the subheader group length and subheader type. Also we are proposing to remove ARQ (C802.16m-08/1068) so the ‘Extended’ and ‘ARQ feedback’ bits are no longer needed.

- SHF – New field to indicate if subheaders are present.

- EKS – If encryption is enabled, this 2 bits is needed for every PDU.  Can be put into the 2 LSB of PN (4 bytes), leaving 30 bits for PN.

- HCS – extra protection for header, if HARQ is enabled this is redundant since HARQ CRC is enough.

- Multiple PDU indicator (MPI): new field to indicate whether there are multiple MAC PDUs in the same HARQ burst.

# Subheaders

In 16e there are 6 subheaders (packing, fragmentation, grant management, etc) and some extended headers. The extended subheaders are organized into a group format while the subheaders are not. There is really no point in differentiating the subheaders from the extended subheaders. They can all be called ‘subheaders’ and organized into a group format that is very similar to the 16e extended subheader group format. This is a more local and uniform way of organizing these subheaders. The order of the subheaders is FFS.

# 3  Proposed SDD Text Changes

Insert the following text into MAC header sub-clause of the SDD [2]:

-------------------------------  Text Start  ---------------------------------------------------

10.x  MAC PDU Header Format

In 802.16m, MAC PDU headers consist of the generic MAC header (GMH) and optionally subheaders. The generic MAC header (GMH) shall always be present and shall precede all subheaders. Depending on if multiple PDUs are present in one HARQ burst, the GMH format shall look like figure 1 or figure 2. The meaning of each field is explained in Table 1.

Figure 1. Generic MAC Header (GMH) Format (multiple PDUs)

Figure 2. Generic MAC Header (GMH) Format (single PDU)

Table 1. Generic MAC Header Fields

10.y  MAC PDU Subheader Format

If SHF = 1 (subheaders are included), GMH shall be followed by subheaders which shall be organized and grouped according to the format in figure 3.

Length of subheader group

Figure 3. Subheader Group Format

-------------------------------  Text End  ---------------------------------------------------

# References

[1] IEEE Std. 802.16e-2005, IEEE Standard for Local and metropolitan area networks, Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems, Amendment 2: Physical and Medium Access Control Layers for Combined Fixed and Mobile Operation in Licensed Bands, and P802.16Rev2/D5 (June 2008).

[2] IEEE 802.16m-08/003r4, “The Draft IEEE 802.16m System Description Document”