| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed SDD Text for UL Pilot |  |
| Date Submitted | 2008-09-05 |  |
| Source(s) | Dongsheng Yu, Mo-Han Fong, Jun Yuan, Sophie Vrzic, Robert Novak, Hosein Nikopourdeilami, Kathiravetpillai Sivanesan Nortel | Email: dongshengyu@nortel.com mhfong@nortel.com |
| Re: | SDD Session 56 Cleanup, Call for PHY details; in response to the TGm Call for Contributions and Comments 802.16m-08/033 for Session 57 |  |
| Abstract | This contribution proposes SDD text for UL pilot structure |  |
| Purpose | For discussion and approval into TGm SDD text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed SDD Text for UL Pilot

# Dongsheng Yu, Mo-Han Fong, Jun Yuan, Sophie Vrzic, Robert Novak, Hosein Nikopourdeilami, Kathiravetpillai Sivanesan

# Nortel Networks

# 1. Introduction

This contribution is to propose pilot structure of UL tile for data transmission. Pilot structure is proposed for up to 4 Tx streams.

2. Pilot structure for UL data transmission

In current SDD (003r4), two types of tile structure is proposed: 6xNsym and 18xNsym, where Nsym=6 for regular subframe.

For tile size of 18xNsym, which is the size of a PRU and is shared by both downlink and uplink data transmission. On the other hand, DL pilot structure is PRU based and has unified pilot pattern for common and dedicated pilots. UL pilot can be viewed as dedicated pilot. Based on consideration above, downlink and uplink should be shared the same pilot pattern for 18xNsym tile.

For uplink tile size of 6xNsym, which is used for uplink diversity transmission, four pilots are allocated for each Tx stream. We recommend the following pilot pattern to support up to 4 Tx streams as required:

The pilot pattern as shown above can facilitate both 2-D and 1-D channel estimation. With 1-D channel estimation, since pilot spacing is already very small (4 subcarriers), channel estimation can be done in frequency direction first in order to retain better quality at either low or high mobility speed.

2. Pilot structure for UL legacy support

When legacy system is in PUSC mode, FDM scheme should be chosen to multiplex with 16m system, such that legacy system can span its channelization to more OFDM symbols and main its uplink power efficiency. In this case, the tile size for 16m should be 4x6. The following is the recommended pilot structure for this tile.

# Text Proposal modification to SDD

------------------------------------ Start of text proposal------------------------------------

(L15, P55 of 003r4)

[Add the following figure at the end of section 11.6.3]

Figure xx Pilot pattern for UL data tile

(L15, P55 of 003r4)

[Add the following figure at the end of section 11.6.4]

Figure xx Pilot pattern for UL 4x6 tile for supporting PUSC legacy mode

------------------------------------End of text proposal-------------------------------------