| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Recommended HARQ Operation for 802.16m |  |
| Date Submitted | 2008-09-05 |  |
| Source(s) | Sean McBeath, Chongli Liu, Huawei Technologies Fang Huiying, Qu Hongyun, Wang Junhu, ZTE Corporation Dong Xiaolu, Du Ying, CATR Liu Jianhua, Yang Hongwei, ASB Yair Bourlas, Nextwave Sungkyung Kim, ETRI Isamu Yoshii*, Ryohei Kimura*, Lei Huang** *Matsushita Electric (Panasonic) **Panasonic Singapore Laboratories | E-mail: smcbeath@huawei.com, liuchongli@huawei.com E-mail: fang.huiying@zte.com.cn E-mail: dongxiaolu@mail.ritt.com.cn, duying@mail.ritt.com.cn E-mail: Hongwei.Yang@alcatel-sbell.com.cn E-mail: YBourlas@nextwave.com Email: cyrano@etri.re.kr Email: Lei.Huang@sg.panasonic.com |
| Re: | MAC: SDD Session 56 Cleanup; in response to the TGm Call for Contributions and Comments 802.16m-08/033 for Session 57 |  |
| Abstract | This contribution recommends that both synchronous and asynchronous HARQ be supported in 802.16m. |  |
| Purpose | To be discussed and adopted by TGm for use in the 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Recommended HARQ Operation for 802.16m

Sean McBeath, Chongli Liu

Huawei Technologies

Fang Huiying, Qu Hongyun,Wang Junhu

ZTE Corporation

Dong Xiaolu, Du Ying

CATR

Liu Jianhua, Yang Hongwei

ASB

Yair Bourlas

Nextwave

Sungkyung Kim

ETRI

Isamu Yoshii*, Ryohei Kimura*, Lei Huang**

*Matsushita Electric (Panasonic)

**Panasonic Singapore Laboratories

# Introduction

Based on the collection of inputs to the HARQ Rapporteur Group, it can be seen that both synchronous and asynchronous HARQ have associated advantages and disadvantages. In addition, these schemes impact capacity and complexity differently, and this impact varies with the type of traffic.  Accordingly, we propose that 802.16m supports both synchronous and asynchronous HARQ for both DL and UL operations.

Proposal

Synchronous and asynchronous HARQ provide benefits in different operating scenarios.  Therefore, it is recommended that 802.16m support both synchronous and asynchronous HARQ (i.e. option 3 in HARQ RG draft).  The 802.16m BS uses control signaling to differentiate between synchronous and asynchronous HARQ.  Such operation provides the 802.16m BS scheduler the ability to use different HARQ operating modes to meet different traffic QoS and system capacity requirements. Moreover, the 802.16m BS scheduler can switch the HARQ operating modes during call connection. For example, VoIP traffic may use synchronous HARQ, while best effort traffic may alternatively use synchronous and asynchronous HARQ.

Text Proposal

Start of Text

## HARQ Functions

HARQ is supported in downlink and uplink packet (re)transmissions in both BS and MS to improve robustness and performance.

The HARQ operation relies on N-process (multi-channel) stop-and-wait protocol.

### HARQ in the Downlink

#### HARQ Timing and Protocol

IEEE 802.16m uses both asynchronous and synchronous HARQ schemes in the downlink.

The following HARQ parameters and their associated values are defined:

- Maximum retransmission delay.: FFS

- Maximum number of retransmissions: FFS

- Maximum number of HARQ processes FFS

- ACK/NACK delay: FFS

[Placeholder for figures illustrating the choice of HARQ scheme(s) in Downlink]

The HARQ ACK/NACK delay is defined for FDD and for each TDD DL/UL ratio and for each mixed mode scenarios

#### HARQ Operation with Persistent Allocation

<Editor’s note: This section is provided as place holder. This section will be filled when details of HARQ operation for persistent allocation is presented and discussed>

#### HARQ Re-transmissions

For synchronous HARQ, resource at the retransmissions in the downlink can be fixed or adaptive according to control signaling. For asynchronous HARQ, IEEE 802.16m uses an adaptive HARQ scheme in the downlink.  In adaptive asynchronous HARQ, the resource allocation and transmission format for the HARQ retransmissions may be different from the initial transmission. In case of retransmission, control signaling is required to indicate the resource allocation and transmission format along with other HARQ necessary parameters.

### HARQ in the Uplink

#### HARQ Timing and Protocol

IEEE 802.16m uses both asynchronous and synchronous HARQ schemes in the uplink.

The following HARQ parameters and their associated values are defined:

- Maximum retransmission delay: FFS

- Maximum number of retransmissions: FFS

- Maximum number of HARQ processes FFS

- ACK/NACK delay: FFS

[Placeholder for figures illustrating the choice of HARQ scheme(s) in Uplink]

#### HARQ Operation with Persistent Allocation

<Editor’s note: This section is provided as place holder. This section will be filled when details of HARQ operation for persistent allocation is presented and discussed>

#### HARQ Re-transmissions

For synchronous HARQ, resource at the retransmissions in the uplink can be fixed or adaptive according to control signaling. For asynchronous HARQ, IEEE 802.16m uses an adaptive HARQ scheme in the uplink.  In adaptive asynchronous HARQ, the resource allocation and transmission format for the HARQ retransmissions may be different from the initial transmission. In case of retransmission, control signaling is required to indicate the resource allocation and transmission format along with other HARQ necessary parameters.

### HARQ and ARQ Interactions

<Editor’s note: This section is provided as place holder. This section will be filled when details of HARQ and ARQ interactions is presented and discussed>

End of Text