| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | 2nd Draft HO SDD Text (HO AHG) |  |
| Date Submitted | 2008-09-05 |  |
| Source(s) | Mary Chion HO AHG Chair | mchion@zteusa.com |
| Re: | MAC:HO |  |
| Abstract | This contribution provides 2nd draft for HO SDD Text. |  |
| Purpose | Draft HO SDD text for discussion |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

2nd Draft of HO SDD Text

Mary Chion

HO Ad-HOC Group Chair

# Introduction

This document contains the 2nd draft of SDD text related to HO section. This document is used as a starting point to further developing of consensus SDD text related to HO. The proposed draft is based on current version of Draft IEEE 802.16m System Description Document [1].

The 2nd draft of SDD text only includes intra-RAT 16m HO. There are still many open issues/sections, this contribution provided some text as well as list of issues need to be resolved. In addition, this is not the final SDD text for HO. The text shown as consensus text is determined by AHG chair as “reasonably consensus” and can be discussed and modified further.

# List of Contributions

This section contains a list of contributions which are submitted by the AHG members to comment on the 1st draft of SDD text proposed by HO AHG chair. After reviewing the contributions, the AHG chair provides modification to the 1st draft of SDD shown in section 3. It is the intention of AHG chair to include all comments/contributions into the ToC as long as there is a reasonable consensus shown among the members. Some comments were deferred to later date since HO AHG chair decided that only intra-16m HO related text will be finalized prior to Session #57.

Also please note that AHG chair defines “reasonable consensus” as there is not major conflicts shown in different proposals. Since AHG chair did not receive any comments from members on the contributions themselves, it is not possible to determine if there is any opposition on the proposals otherwise. In addition, AHG chair has also included text that is no concerns or issues raised by members or the chair.

There is NO consensus reached for most part of section 10.3.2 Handover Process. There are a few contributions providing different SDD texts to that section, but they can not be harmonized at the moment. AHG chair decided to not include any text that has not reach consensus, instead, will use to next week to reach consensus. So, several contributions are deferred.

| Name | Author(s) | Comment |
| --- | --- | --- |
| C80216m-MAC-08_024r1 | Haihong Zheng Shashikant Maheshwari Yousuf Saifullah NSN Zexian Li Nokia Yung-Han Chen Fang-Ching (Frank) Ren Richard Li ITRI Wern-Ho Sheen NCTU/ITRI | Deferred to later |
| C80216m-MAC-08_027r1 | Kanchei(Ken) Loa Yi-Hsueh Tsai Chih-Wei Su Chun-Yen Hsu Yung-Ting Lee Youn-Tai Lee Hua-Chiang Yin Institute for Information Industry Shiann-Tsong Sheu National Central University Yih-Guang Jan Yang-Han Lee Tamkang University (TKU) Whai-En Chen National Ilan University (NIU) | Deferred as part of HO Process discussion |
| C80216m-MAC-08_047 | Keiichi Nakatsugawa Yuji Kojima Yanling Lu Wei-Peng Chen Fujitsu | Incorporated except for 16e/16m HO text. |
| C80216m-MAC-08_048r1 |  |  |
| C80216m-MAC-08_050 | Kaibin Zhang Gang Shen Jimin Liu Shan Jin Alcatel Shanghai Bell | Have the following questions on this contributions: - Not sure how easy a BS can determine MSs are in the same group (hence, moving together) without a RS? Seems to be more suitable for relay - Based on the contribution, what is the optimization proposed? BS triggered HO? Need some clarification on the proposal. - If the proposal is to have optimize steps between serving BS and target BS, then, that is out of scope of 16m? |
| C80216m-MAC-08_051 | Stavros Tzavidas Motorola Inc. | Incorporated text on section 10.3.1 Rest of text is deferred as part of HO procedure discussion |
| C80216m-MAC-08_058 | Vivek Gupta Intel Corporation Jianmin Lu Huawei Les Eastwood Motorola Juan Carlos Zunniga InterDigital Junghoon Jee ETRI Ronny Yongho Kim Giwon Park Jin Lee LG Electronics | Deferred till inter-RAT discussion |
| C80216m-MAC-08_059 | Chun-Yen Wang Chie-Ming Chou Fang-Ching (Frank) Ren Yung-Han Chen Chun-Yuan Chiu Richard Li ITRI Wern-Ho sheen NCTU/ITRI | Have the following questions: - HO preparation phrase is after HO initiation. In HO initiation, either a MOB_MSHO-REQ or MOB_BSHO-REQ should be received. Based on the text in this contribution, for BS initiated HO, the ranging will happen prior to HO initiation, is that correct? - For MS initiated HO, the MS will need to wait for a message from BS and perform ranging first at target BS. This does prolong HO process. And for HO interruption, would it be the same as doing it at network re-entry time? - Could this be achieved using association for both MS/BS initiated HO. For now, I will move the text to 10.3.1 as 10.3.1.3 Association (or Early UL Synchronization), but leave it in bracket. We need to revisit this text and decide |
| C80216m-MAC-08_061 | Ranga Reddy US Army Weihuang Fu Jung Hyun Jun Demin Wang Xiaoyuan Wang Dharma P. Agrawal University of Cincinnati Andrew DeCarlo Jonathan Portny Infoscitex Corporation | Accepted |
| C80216m-MAC-08_063 | Mo-Han Fong Hang Zhang Robert Novak Sophie Vrzic Jun Yuan Dongsheng Yu Hosein Nikopourdeilami Kathiravetpillai Sivanesan Nortel Networks Hongwei Yang Alcatel Shanghai Bell Pantelis Monogioudis Alcatel-Lucent Yinggang Du Lina Liu Sean McBeath Huawei Technologies Co.,Ltd. | Deferred as part of HO Process discussion |
| C80216m-MAC-08_064 | Inuk Jung Ronny Yongho Kim Kiseon Ryu Youngsoo Yuk LG Electronics Inc. | Deferred as part of HO process discussion. Although following question: - BS switch time is the HO action time, correct? Is it included in MOB_BSHO-RSO message? - In the case of BS initiated HO, how is this mismatch detected? - What does the following sentence mean: “When receiving the system information of neighbor BSs, a bitmap for each SI version set, with mappings to each SI attribute, are included.”? Included HO Ranging text as bracket text. |
| C80216m-MAC-08_065 | Xiangying Yang Susan Hartman Shahrnaz Azizi Muthaiah Venkatachalam Intel Kelvin Chou Yih-Shen Chen I-Kang Fu Paul Cheng Mediatek Inc. Haihong Zheng Shashikant Maheshwari Yousuf Saifullah NSN Zexian Li Nokia Jaehyuk Jang Jungje Son Rakesh Taori Samsung Electronics Baowei Ji Samsung Telecommunication America | Deferred as part of HO process discussion except for HO cancel and Network re-entry text. Those sections are incorporated into the draft. Also incorporated modification to TOC. |
| C80216m-MAC-08_072r1 | Seongwon Oh Kyounghwan Lee Yerang Hur Bongho Kim POSDATA | Regarding the use of MAC signaling header/subheader/ESF, since MAC header/subheader format has not been determined for 16m, it’s too early to decide what can be used. Suggest to defer this comment to later date when MAC header definition is finalized. |

# Text Proposal

[Editor’s Note: the text enclosed by [] and shown in blue are non-consensus text.]

---------------------------------------------------------Start of the text--------------------------------------------------

10.3 Handover

[Editor’s Note: overall description of HO, needs to update the current SDD text]

10.3.1 Network topology acquisition

[Editor’s Note: This section is only related to intra-RAT 16m to 16m HO]

10.3.1.1 Network topology advertisement

[Editor’s Note: This section contains text related to network topology as well as the description of neighbor BS information and the method/procedure as how the information is transmitted by the BS. ]

A BS periodically broadcasts the system information of the neighboring BSs using MOB_NBR-ADV message. The BS formats MOB_NBR-ADV message based on the cell types of neighbor cells, in order to achieve overhead reduction and facilitate scanning priority for MS. MOB_NBR-ADV message does not include information of neighbor femto cells. Special handling of neighbor information of femto cell is described in section 10.3.4

[ In addition to broadcasted MOB_NBR-ADV, a unicast message containing Neighbor BS System Configuration information can be sent to MS.]

10.3.1.2 Scanning Procedure

[Editor’s Note: This section contains text related to MS scanning procedure Contribution]

The scanning procedure provides the opportunity for the MS to perform measurement of the neighboring cells for handover decision. The MS may use current or negotiate new unavailable intervals to perform scanning. In addition, the MS may perform scanning procedure during its active cycle without interrupting its communication with the serving BS if the MS supports such capability.

MS selects the scanning candidate BSs by the information obtained from MOB_NBR-ADV or information cached in the MS, or instructed by BS. The BS or MS may prioritize the BSs to be scanned based on various metrics, such as cell type, loading, RSSI and path delay/location.

MS measures the selected scanning candidate BSs and reports the measurement result back to the serving BS [according to the following rules: the serving BS defines trigger conditions (exact mechanism for trigger definition is FFS) based on which the MS sends scanning report; the serving BS can also explicitly request a scan report at any time. ]

[ 10.3.1.2 Early UL Synchronization (Association)

[Editor’s Note: The content of this section is not consensus text]

The HO Ranging procedure can be performed before HO initiation phase. The serving BS may send the MS a dedicated ranging code pre-assigned by a target BS. When receiving the dedicated ranging code, MS performs ranging at the target BS by transmitting the dedicated ranging code. After transmission of the dedicated ranging code, the MS returns to serving BS without waiting for RNG-RSP from target BS. Upon receiving the dedicated ranging code, the target BS sends the ranging information to the serving BS through inter-BS communication. The serving BS provides ranging parameters to the MS via MOB_BSHO-REQ for BS initiated HO or MOB_BSHO-RSP for MS initiated HO during HO procedure. ]

10.3.2 Handover Process

[Editor’s Note: This section is only related to intra-RAT 16m to 16m HO for non-multi-carrier case. This section should contain text related to generic handover procedures]

10.3.2.1 HO Framework

[Editor’s Note: Partial text form contribution C80216m-MAC-08_022r2.

Issue: Request comments from members regarding defining two HO schemes namely Break-before-Make and Make-before-Break. Will this be two options and two procedures?]

Handover procedure may be initiated by either MS or BS and [controlled by the BS]. When handover is initiated by the MS, the serving BS defines the triggers and conditions based on which the MS initiates a handover. The MS may utilize its sleep cycle for operations with other base stations such as scanning, association and [network re-entry]. The network re-entry procedure with the target BS may be optimized by target BS possession of MS information obtained from serving BS over the backbone network.

10.3.2.1.1 Break-before-Make Handover

10.3.2.1.2 Make-before-Break Handover

10.3.2.2 HO Procedure

[Editor’s Note: Some sections of HO procedures have a few similar contributions/proposals. However, in general, there are many open issues without any reasonable consensus at the moment. In this draft, no SDD text will be included. Instead, more detailed ToC are included and the members are requested to propose SDD text and harmonize within the group. Also please note that the ToC subsection proposal is not a finalized version, it is meant to be a place holder for most of the proposals received in session #56.The following are a list of topics I have seen:

Topics for discussion:

- For intra-Frequency HO, is UL synchronization at target BS necessary during network re-entry? We have a few different proposals from session #56 which indicates it might not be necessary: a. proposal for MS to monitor both target and serving BS during HO and allows MS to communicate with serving BS while performing network re-entry; a proposal allows the multiple BSs (in active set) monitor the same Ranging Region and performs ranging adjustment to MS prior the MS entering the target BS (new serving BS)

- MS monitoring DL control channel from multiple BS?

- Should we consider active set management? Active set management is not limited to FBSS/MDHO. It is also can be used for any HO procedure. We need to decide if we should capture this as part of HO procedures

- Ranging procedure at target BS: there are several proposals on Ranging procedure, namely, dedicated ranging code and dedicated ranging region during network entry time at target BS; dedicated ranging code at neighbor BS during scanning (??); multiple BSs monitoring ranging region to eliminate the need for ranging during network re-entry. We need to converge into one solution and also should make sure we are not using too many ranging codes ( ).

- HO and sleep mode: a few contributions proposing using sleep period for HO operation. We need to consider the following: a. is this an implementation issue or a standard issue?; b. impact on power saving, any?; c. dependency to sleep mode design

- MDHO support? Is it transparent to the MS, hence doesn’t need to be specified?]

10.3.2.2.1 HO initiation

[ 10.3.2.2.1.1 MS initiated HO

10.3.2.2.1.2 BS initiated HO

]

10.3.2.2.2 HO preparation

During HO preparation phase, the target BS may obtain MS information from the serving BS via backbone network for HO optimization. Dedicated ranging resource (e.g. code, channel, etc.) at target BS may be reserved for the MS to facilitate non-contention-based HO ranging. Information regarding MS identity (e.g. TEK, MS logical IDs, etc.), may be pre-updated in HO preparation. [MS may request for bandwidth based on its buffer status during HO preparation.] [Editor note: need clarification on this sentence. MS requests BW from serving BS or target BS, more detail needed]

10.3.2.2.3 HO execution

10.3.2.2.4 HO cancellation

After the HO is initiated, the MS may cancel HO at any time by sending a HO cancellation message to the serving BS. After the HO cancellation message is processed, the MS and serving BS resume their normal operation if MS information is retained at serving BS when HO cancellation message is received. [This handover cancel procedure may include allocating the dedicated resource for the MS to send the HO cancel message.] [Editor note:  please clarify this sentence]

10.3.2.3 Network Re-entry

10.3.2.3.1 HO Ranging procedure

[Editor’s Note: Includes all ranging proposals, may need additional subclauses]

If a dedicated ranging code is assigned to the MS by target BS, the MS shall transmit the dedicated ranging code to the target BS during network re-entry. If a ranging channel is scheduled by the target BS for handover purpose only, the MS should use that ranging channel in order to avoid excessive multiple access interference. Upon reception of the dedicated ranging code, the target BS should allocate uplink resources for RNG-REQ message and UL data if needed.

[Ranging with target BS can be performed prior to the handover. Detailed mechanism is TBD. When the MS handovers to the target BS, ranging may be omitted if the ranging parameters are valid.]

[The MS is provided with a dedicated code to perform HO ranging. Through negotiation between the MS and the target BS via the serving BS, the dedicated HO ranging code is reserved and its index is encrypted with the MS’s AK for authentication purposes. The encrypted dedicated code index is indicated by the target BS. The function to encrypt a dedicated HO ranging code is FFS.]

10. 3.2.3.2 Network Re-entry Procedure

Network re-entry procedure at the target BS, i.e. capability negotiation, authentication, registration, connection establishment, can be omitted if the target BS contains the necessary context information of the MS. When HO ranging is performed with the target BS, MS sends RNG-REQ upon receiving UL allocation. When RNG-REQ message is received from the MS, the target BS responds with a RNG-RSP. While sending the RNG-RSP message, the target BS may send the MS downlink data simultaneously. Handover procedure completes with successful transaction of RNG-REQ/RSP message if the rest of network re-entry procedures are omitted.

10.3.4 Handover support for Femto BS

[Editor’s Note: This section is only related to intra-RAT 16m to 16m HO. The text proposed in C80216m-MAC-08_022r2 is included with some modification.]

The network provides the mapping between femto BS and corresponding overlay (macro/micro) BS as well as certain system information such as carrier frequency of the femto cell to MS for supporting handover between (macro/micro) BS and femto BS. MS may cache this information for future handover to the specific femto cell. At the time of handover preparation, the system information of a target femto cell may be unicast or multicast to the MS upon MS request/network trigger or obtained by the MS monitoring the femto cell.

Upon entering the coverage area of the overlay macro BS of the femto BS for an MS, the MS starts to scan for the femto BS. After the femto BS is detected, the MS may initiate handover request to the macro BS. The handover process between femto and macro BS is the same as defined in section 10.3.2.

10.3.5 Handover Process supporting Legacy system

[Editor’s Note: This section is only related to intra-RAT 16e/16m HO.]

10.3.5.1 Network topology acquisition

10.3.5.2 Handover from 16e to 16m

10.3.5.3 Handover from 16m to 16e

[Editor’s Note: This following section shows two different proposals on inter-RAT HO TOC]

[Editor’s Note: Option 1]

[10.3.6 Inter-RAT Handover Procedure

10.3.6.1 Generic inter-RAT HO procedure

10.3.6.1.1 Dual Transmitter/Dual Receiver Support

10.3.6.1.2 Single Transmitter/Dual Receiver Support

10.3.6.1.3 Single Transmitter/Single Receiver Support

10.3.6.2 Enhanced inter-RAT HO procedure

10.3.6.2.1 Fast handover between 802.11 and 802.16m]

[Editor’s Note: Option 2]

[13. Inter-Radio Access Technology Functions

13.1 Inter-Radio Access Technology Handover Procedures

13.1.1 Network topology acquisition

13.1.2 Generic Inter RAT HO Procedures]

----------------------------------------------------------End of the text----------------------------------------------------

# Reference

- IEEE C802.16m-08/003r4, “Project 802.16m System Description Document (SDD)”.