| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | 3rd Draft HO SDD Text (HO AHG) |  |
| Date Submitted | 2008-09-17 |  |
| Source(s) | Mary Chion HO AHG Chair | mchion@zteusa.com |
| Re: | MAC:HO |  |
| Abstract | This contribution provides 3rd draft for HO SDD Text. |  |
| Purpose | Draft HO SDD text for discussion |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

3rd Draft of HO SDD Text

Mary Chion

HO Ad-HOC Group Chair

# Introduction

This document contains the 3rd draft of SDD text related to HO section. This document is used as a starting point to further developing of consensus SDD text related to HO. The proposed draft is based on current version of Draft IEEE 802.16m System Description Document [1].

The 3rd draft of SDD text only includes intra-RAT 16m HO. There are still many open issues/sections, this contribution provided some text as well as list of issues need to be resolved. In addition, this is not the final SDD text for HO. The text shown as consensus text is determined by AHG chair as “reasonably consensus” and can be discussed and modified further.

# List of Contributions

This section contains a list of contributions which are submitted by the AHG members to comment on the 1st draft of SDD text proposed by HO AHG chair. After reviewing the contributions, the AHG chair provides modification to the 1st draft of SDD shown in section 3. It is the intention of AHG chair to include all comments/contributions into the ToC as long as there is a reasonable consensus shown among the members. Some comments were deferred to later date since HO AHG chair decided that only intra-16m HO related text will be finalized prior to Session #57.

Also please note that AHG chair defines “reasonable consensus” as there is not major conflicts shown in different proposals. Since AHG chair did not receive any comments from members on the contributions themselves, it is not possible to determine if there is any opposition on the proposals otherwise. In addition, AHG chair has also included text that is no concerns or issues raised by members or the chair.

There is NO consensus reached for most part of section 10.3.2 Handover Process. There are a few contributions providing different SDD texts to that section, but they can not be harmonized at the moment. AHG chair decided to not include any text that has not reach consensus, instead, will use to next week to reach consensus. So, several contributions are deferred.

| Name | Author(s) | Comment |
| --- | --- | --- |
| C80216m-MAC-08_024r1 | Haihong Zheng Shashikant Maheshwari Yousuf Saifullah NSN Zexian Li Nokia Yung-Han Chen Fang-Ching (Frank) Ren Richard Li ITRI Wern-Ho Sheen NCTU/ITRI | Deferred to later |
| C80216m-MAC-08_027r1 | Kanchei(Ken) Loa Yi-Hsueh Tsai Chih-Wei Su Chun-Yen Hsu Yung-Ting Lee Youn-Tai Lee Hua-Chiang Yin Institute for Information Industry Shiann-Tsong Sheu National Central University Yih-Guang Jan Yang-Han Lee Tamkang University (TKU) Whai-En Chen National Ilan University (NIU) | Deferred as part of HO Process discussion |
| C80216m-MAC-08_047 | Keiichi Nakatsugawa Yuji Kojima Yanling Lu Wei-Peng Chen Fujitsu | Incorporated except for 16e/16m HO text. |
| C80216m-MAC-08_048r1 |  |  |
| C80216m-MAC-08_050 | Kaibin Zhang Gang Shen Jimin Liu Shan Jin Alcatel Shanghai Bell | Have the following questions on this contributions: - Not sure how easy a BS can determine MSs are in the same group (hence, moving together) without a RS? Seems to be more suitable for relay - Based on the contribution, what is the optimization proposed? BS triggered HO? Need some clarification on the proposal. - If the proposal is to have optimize steps between serving BS and target BS, then, that is out of scope of 16m? |
| C80216m-MAC-08_051 | Stavros Tzavidas Motorola Inc. | Incorporated text on section 10.3.1 Rest of text is deferred as part of HO procedure discussion |
| C80216m-MAC-08_058 | Vivek Gupta Intel Corporation Jianmin Lu Huawei Les Eastwood Motorola Juan Carlos Zunniga InterDigital Junghoon Jee ETRI Ronny Yongho Kim Giwon Park Jin Lee LG Electronics | Deferred till inter-RAT discussion |
| C80216m-MAC-08_059 | Chun-Yen Wang Chie-Ming Chou Fang-Ching (Frank) Ren Yung-Han Chen Chun-Yuan Chiu Richard Li ITRI Wern-Ho sheen NCTU/ITRI | Have the following questions: - HO preparation phrase is after HO initiation. In HO initiation, either a MOB_MSHO-REQ or MOB_BSHO-REQ should be received. Based on the text in this contribution, for BS initiated HO, the ranging will happen prior to HO initiation, is that correct? - For MS initiated HO, the MS will need to wait for a message from BS and perform ranging first at target BS. This does prolong HO process. And for HO interruption, would it be the same as doing it at network re-entry time? - Could this be achieved using association for both MS/BS initiated HO. For now, I will move the text to 10.3.1 as 10.3.1.3 Association (or Early UL Synchronization), but leave it in bracket. We need to revisit this text and decide |
| C80216m-MAC-08_061 | Ranga Reddy US Army Weihuang Fu Jung Hyun Jun Demin Wang Xiaoyuan Wang Dharma P. Agrawal University of Cincinnati Andrew DeCarlo Jonathan Portny Infoscitex Corporation | Accepted |
| C80216m-MAC-08_063 | Mo-Han Fong Hang Zhang Robert Novak Sophie Vrzic Jun Yuan Dongsheng Yu Hosein Nikopourdeilami Kathiravetpillai Sivanesan Nortel Networks Hongwei Yang Alcatel Shanghai Bell Pantelis Monogioudis Alcatel-Lucent Yinggang Du Lina Liu Sean McBeath Huawei Technologies Co.,Ltd. | Deferred as part of HO Process discussion |
| C80216m-MAC-08_064 | Inuk Jung Ronny Yongho Kim Kiseon Ryu Youngsoo Yuk LG Electronics Inc. | Deferred as part of HO process discussion. Although following question: - BS switch time is the HO action time, correct? Is it included in MOB_BSHO-RSO message? - In the case of BS initiated HO, how is this mismatch detected? - What does the following sentence mean: “When receiving the system information of neighbor BSs, a bitmap for each SI version set, with mappings to each SI attribute, are included.”? Included HO Ranging text as bracket text. |
| C80216m-MAC-08_065 | Xiangying Yang Susan Hartman Shahrnaz Azizi Muthaiah Venkatachalam Intel Kelvin Chou Yih-Shen Chen I-Kang Fu Paul Cheng Mediatek Inc. Haihong Zheng Shashikant Maheshwari Yousuf Saifullah NSN Zexian Li Nokia Jaehyuk Jang Jungje Son Rakesh Taori Samsung Electronics Baowei Ji Samsung Telecommunication America | Deferred as part of HO process discussion except for HO cancel and Network re-entry text. Those sections are incorporated into the draft. Also incorporated modification to TOC. |
| C80216m-MAC-08_072r1 | Seongwon Oh Kyounghwan Lee Yerang Hur Bongho Kim POSDATA | Regarding the use of MAC signaling header/subheader/ESF, since MAC header/subheader format has not been determined for 16m, it’s too early to decide what can be used. Suggest to defer this comment to later date when MAC header definition is finalized. |

# Text Proposal

[Editor’s Note: the text enclosed by [] and shown in blue and red are non-consensus text.]

---------------------------------------------------------Start of the text--------------------------------------------------

10.3 Handover

10.3.1 Network topology acquisition

10.3.1.1 Network topology advertisement

A BS periodically broadcasts the system information of the neighboring BSs using MOB_NBR-ADV message. The BS formats MOB_NBR-ADV message based on the cell types of neighbor cells, in order to achieve overhead reduction and facilitate scanning priority for MS. MOB_NBR-ADV message does not include information of neighbor femto cells. Special handling of neighbor information of femto cell is described in section 10.3.4

A serving BS may unicast the MOB_NBR-ADV message to a MS.

10.3.1.2 Scanning Procedure

The scanning procedure provides the opportunity for the MS to perform measurement of the neighboring cells for handover decision. The MS may use current or negotiate new unavailable intervals to perform scanning. In addition, the MS may perform scanning procedure during its active cycle without interrupting its communication with the serving BS if the MS supports such capability.

MS selects the scanning candidate BSs by information obtained from MOB_NBR-ADV or information cached in the MS, or instructed by BS. The BS or MS may prioritize the neighbor BSs to be scanned based on various metrics, such as cell type, loading, RSSI and location.

MS measures the selected scanning candidate BSs and reports the measurement result back to the serving BS. The serving BS defines triggering conditions and rules for MS sending scanning report.

10.3.2 Handover Process

10.3.2.1 HO Framework

The handover algorithm is a network-controlled, MS-assisted handover. Although handover procedure may be initiated by either MS or BS, the final HO decision and target BS(s) selection are performed by the serving BS or the network. MS executes the HO as directed by the BS or cancels the HO procedure through HO cancellation message. The network re-entry procedure with the target BS may be optimized by target BS possession of MS information obtained from serving BS over the backbone network. MS may also maintain communication with serving BS while performing network re-entry at target BS as directed by serving BS. Figure 1x shows a general call flow for handover.

Figure 1x. A general call flow for HO

The handover procedures are divided into three phrases, namely, HO initialization, HO preparation and HO execution. When HO execution is complete, the MS is ready to perform Network re-entry procedures at target BS. In addition, HO cancellation procedure is defined to allow MS cancel a HO procedure.

10.3.2.2 HO Procedure

10.3.2.2.1 HO initiation

Handover procedure may be initiated by either MS or BS and controlled by the BS. When handover is initiated by the MS, the serving BS defines the triggers and conditions based on which the MS initiates a handover. When multiple triggers and conditions are defined, the serving BS may use combination of multiple conditions to trigger HO. When HO is initiated by MS, a HO Initiation control signaling is sent by the MS to start the HO procedure.

In case of BS initiated HO, HO initiation and HO preparation phases are carried out together.

10.3.2.2.2 HO preparation

During HO preparation phase, the serving BS selects communicate with target BS(s) for HO. The target BS may obtain MS information from the serving BS via backbone network for HO optimization. Ranging with target BS prior or during HO preparation is FFS. If ranging with target BS not performed prior to or during HO preparation, dedicated ranging resource (e.g. code, channel, etc.) at target BS may be reserved for the MS to facilitate non-contention-based HO ranging. Information regarding MS identity (e.g. TEK, STID, FIDs, etc.), may be pre-updated during HO preparation. Any mismatched system information between MS and the target BS, if detected, may be provided to the MS by the Serving BS during HO preparation.

HO preparation phase completes when serving BS informs the MS of its handover decision via a HO Command control signaling. The control signaling may include dedicate ranging resource allocation and resource pre-allocations for MS at target BS for optimized network re-entry. The control signaling includes an action time for the MS to start network re-entry at the target BS and an indication whether MS should maintain communication with serving BS during network re-entry.

10.3.2.2.3 HO execution

At the action time specified in the HO command control signaling, the MS performs network re-entry at the target BS. If communication is not maintained between MS and serving BS during network re-entry at the target BS, serving BS stops allocating resources to MS for transmission at action time.

If directed by serving BS via HO Command control signaling, the MS performs network re-entry with the target BS at action time while continuously communicates with serving BS. However, the MS stops communication with serving BS after network re-entry at target BS is completed. In addition, MS cannot exchange data with target BS prior to completion of network re-entry. Multiplexing of network re-entry signaling with target BS and communications with serving BS is FFS.

10.3.2.2.4 HO cancellation

After HO is initiated, the MS may cancel HO at any phase during HO procedure by sending a HO cancellation message to the serving BS. After the HO cancellation message is processed, the MS and serving BS resume their normal operation. The dedicated ranging code assignment for transmitting HO cancellation message is FFS.

10.3.2.3 Network Re-entry

10.3.2.3.1 CDMA-based HO Ranging procedure

If a dedicated ranging code is assigned to the MS by target BS, the MS transmits the dedicated ranging code to the target BS during network re-entry. If a ranging channel is scheduled by the target BS for handover purpose only, the MS should use that ranging channel in order to avoid excessive multiple access interference. MS authentication through CDMA-based HO ranging is FFS. For CDMA-based HO ranging without MS authentication, upon reception of the dedicated ranging code, the target BS should allocate uplinks resources for RNG-REQ message and UL data if needed.

CDMA-based HO ranging with target BS prior to network re-entry is FFS. When the MS handovers to the target BS, CDMA-based HO ranging may be omitted if the ranging parameters are valid.

10. 3.2.3.2 Network Re-entry Procedure

The network re-entry procedure is carried out as specified in IEEE P802.16Rev2 procedure unless otherwise specified in this section. Enhancement to the authentication mechanism at the target BS is FFS..

10.3.4 Handover support for Femto BS

[Editor’s Note: This section is only related to intra-RAT 16m to 16m HO. The text proposed in C80216m-MAC-08_022r2 is included with some modification.]

The network provides the mapping between femto BS and corresponding overlay (macro/micro) BS as well as certain system information such as carrier frequency of the femto cell to MS for supporting handover between (macro/micro) BS and femto BS. MS may cache this information for future handover to the specific femto cell. At the time of handover preparation, the system information of a target femto cell may be unicast or multicast to the MS upon MS request/network trigger or obtained by the MS monitoring the femto cell.

The HO procedure between femto BS and macro BS is FFS.

10.3.5 Handover Process supporting Legacy system

[Editor’s Note: This section is only related to intra-RAT 16e/16m HO.]

10.3.5.1 Network topology acquisition

10.3.5.2 Handover from 16e to 16m

10.3.5.3 Handover from 16m to 16e

----------------------------------------------------------End of the text----------------------------------------------------

# Reference

- IEEE C802.16m-08/003r4, “Project 802.16m System Description Document (SDD)”.