| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed SDD Text for Synchronization Channel in Section 11.7.2.1 |  |
| Date Submitted | 2008-09-18 |  |
| Source(s) | Zexian Li Nokia Shashikant Maheshwari, Adrian Boariu, Jianfeng Kang, Yousuf Saifullah Nokia Siemens Networks Yang-Seok Choi Intel Sungho Moon, Jin Sam Kwak LG Electronics Sung-Eun Park, Hokyu Choi Samsung Electronics Pei-Kai Liao, Paul Cheng MediaTek Inc. Chris Williams, Luciano Sarperi, Kevin Power, Sunil Vadgama Fujitsu Yong Sun, Toshiba Research Europe Limited Robert Novak, Mo-Han Fong, Sophie Vrzic, Jun Yuan, Dongsheng Yu, Hosein Nikopourdeilami, Kathiravetpillai Sivanesan Nortel Zheng Yan-Xiu, Richard Li, ITRI Sun Changyin, Fang Huiying ZTE | Voice: +358504869272 E-mail: zexian.li@nokia.com yang-seok.choi@intel.com msungho@lge.com se.park@samsung.com pk.liao @mediatek.com Chris.Williams@uk.fujitsu.com Luciano.sarperi@uk.fujitsu.com Sun@toshiba-trel.com rnovak@nortel.com richard929@itri.org.tw fang.huiying@zte.com.cn sun.changyinxa@zte.com.cn |
| Re: | PHY: Text. in response to the TGm Call for Contributions and Comments 802.16m-08/033 for Session 57 |  |
| Abstract | This contribution proposes SDD text for SCH for the 802.16m based on ToC in IEEE 802.16m-08/003r4. |  |
| Purpose | To be discussed and adopted by TGm for use in the 802.16m SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed SDD Text for Synchronization Channel in Section 11.7.2.1

Zexian Li,

Nokia

Shashikant Maheshwari, Adrian Boariu, Jianfeng Kang, Yousuf Saifullah

Nokia Siemens Networks

Yang-Seok Choi

Intel

Sungho Moon, Jin Sam Kwak

LG Electronics

Sung-Eun Park, Hokyu Choi

Samsung Electronics

Yong Sun

Toshiba Research Europe Limited

Pei-Kai Liao, Paul Cheng

MediaTek Inc.

Chris Williams, Luciano Sarperi, Kevin Power, Sunil Vadgama

Fujitsu

Robert Novak, Mo-Han Fong, Sophie Vrzic, Jun Yuan,  Dongsheng Yu, Hosein Nikopourdeilami, Kathiravetpillai Sivanesan
Nortel

Zheng Yan-Xiu, Richard Li,

ITRI

Sun Changyin, Fang Huiying

ZTE

1. Introduction

The following is the proposed text change for Section 11.7.2.1 on synchronization channel in IEEE 802.16m System Description Document (SDD). The proposed modifications are based on IEEE 802.16m-08/003r4.

------------------------------------------------------- Start of text proposal --------------------------------------------------

[Modify each section as follows]

11.7.2.1.2.1.1 Hierarchy

Two structures are possible:

• Non-hierarchical, where only a single type of 16m synchronization symbol exists, which may be in addition to the legacy 16e preamble. Where a new 16m SCH is defined, the 16e preamble is not required as part of the SCH functionality (as defined in the SCH Requirements).

• Hierarchical, where more than one type of SCH symbol exists within the superframe. They may or may not use the legacy 16e preamble as one level of hierarchy.

The decision of hierarchy or non-hierarchy structure of the synchronization channel is FFS.

[Option 1. No hierarchy of synchronization channel (SCH) exists. The SCH is transmitted with a span of one OFDM symbol, in order to enable time- and frequency-synchronization and cell-ID detection.]

[Option 2. Two levels of synchronization hierarchy exist. These are called the primary synchronization channel (P-SCH) and secondary synchronization channel (S-SCH). The P-SCH is used for initial acquisition, super frame synchronization and sending additional information. The use of P-SCH for LBS is FFS. The S-SCH is used for fine synchronization, and cell/sector identification (ID). The use of P-SCH to carry ID information is also FFS.]

11.7.2.1.2.1.2 Multiplexing

Synchronization symbols  P-SCH and S-SCH are TDM.

11.7.2.1.2.1.3 Number of symbols in SCH

A complete instance of the SCH exists within a superframe. Multiple symbols within the superframe may comprise the SCH.

In mixed deployments, the presence of the 16e legacy preamble is implicit.

The number of synchronization channel symbols in each superframe is FFS.

- 11.7.2.1.2.1.4 Location of synchronization symbols

In mixed deployments, the presence of the 16e legacy preamble in the first symbol of the 16e legacy frame is implicit. The location of the SCH symbol(s) is fixed within the superframe.

The location of SCH is FFS. [Where a SCH hierarchy exists, the time separation between the P-SCH and S-SCH is fixed.]

11.7.2.1.2.1.5 Properties of P-SCH & S-SCH (if they exist) <Editors’ note: This section only applies if the P-SCH/S-SCH hierarchy is agreed. Square bracketed items are for further discussion.>

The P-SCH has these properties:

• Common to all a group of sector(s)/cell(s)

• [Optionally s Supports limited signalling (e.g. mode, bandwidth, repeat part of cell ID, etc)]

• Fixed bandwidth (5MHz or less)

• Support LBS(FFS)

• [Carries ID information (e.g. group ID, femto ID) - FFS]

The S-SCH has these properties:

• [Same bandwidth as P-SCH | Full bandwidth]

• Carries cell ID information

###### 11.7.2.1.2.3 Cell ID support

The number of IDs is at least [TBD] 512.

Sectors are distinguished by the synchronization channel; the number of sectors is [FFS] .

##### 11.7.2.1.3 Synchronization channel Sequence Design Properties

The SCH enables timing synchronization by autocorrelation.

The power of synchronization channel can be boosted

The SCH (non-hierarchical) or P-SCH (hierarchical) is mapping with every other subcarrier on the frequency domain. Frequency reuse of 1 is applied to P-SCH.

The S-SCH (hierarchical) is mapping with every [FFS] subcarriers on the frequency domain

Frequency reuse of [TBD] is applied to S-SCH.

------------------------------------------------------- End of text proposal --------------------------------------------------