| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Dynamic RRM Support for Enhanced Multicast Broadcast Services |  |
| Date Submitted | 2008-09-05 |  |
| Source(s) | Ray-Guang Cheng National Taiwan University of Science and Technology | Voice: +886-2-27376371 E-mail: crg@ieee.org *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for Contributions on Project 802.16m System Description Document (SDD) |  |
| Abstract | This contribution proposes functions required to support dynamic RRM for E-MBS |  |
| Purpose | Discussion and Approval |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Dynamic RRM Support for Enhanced Multicast Broadcast Services

Ray-Guang Cheng

National Taiwan University of Science and Technology

# 1 Introduction

This contribution proposes address the issues and requirements regarding to the MBS-related feedback mechanism for supporting dynamic radio resource management.

# 2 Description of the Proposal

In 802.16Rev2, MBS service is offered in the downlink only and the MBS service flows should be transmitted and maintained for SS either in awake/sleep mode or in the idle mode. The lack of feedback mechanism may lead to a low spectral efficiency in delivering MBS services because dynamic radio resource management (RRM) functions cannot be adopted. It has been shown in the literature that the number of synchronized transmitting cells in an MBS zone, the cell size, and the users’ location distribution are factors that may affect the selection of MCS state. Unfortunately, due to the lack of feedback mechanism, the network cannot dynamically allocate the radio resource (e.g., transmission power, MCS state) for BSs belonging to the same MBS zone.

The selection of proper feedback information and the type of feedback channel are two main issues that should be considered while designing the feedback mechanism. The feedback information should be detail enough for SS to indicate the dissatisfaction of its QoS. It also should be simple enough to minimize the uplink signaling overhead. The feedback channel should be capable for accommodating SSs in either in awake/sleep mode or in the idle mode.

# 3 Proposed Text for the System Description Document (SDD)

---------------------------------------------Start of the text------------------------------------------------

### 15 Support for Enhanced Multicast Broadcast Service

[Add the following text to section 15.]

### 15.x E-MBS Feedback Mechanism

E-MBS may support feedback mechanism to enable dynamic radio resource management functionalities for enhancing the bandwidth efficiency. The feedback mechanism is enabled based on individual E-MBS service flow. In case that the feedback mechanism is enabled, both awake and idle SSs that receiving the same MBS services should use feedback mechanism to report information related to their received quality.

---------------------------------------------End of the text------------------------------------------------