| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAP Protection & Enhanced Authentication for IEEE 802.16m |  |
| Date Submitted | 2008-09-05 |  |
| Source(s) | Sheng Sun Nortel DJ Shyy MITRE Ranga Reddy US Army | E-mail: shengs@nortel.com E-mail: djshyy@mitre.org E-mail: ranga.reddy@us.army.mil *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | MAC/Security: in response to the TGm Call for Contributions and Comments 802.16m-08/033 for Session 57 |  |
| Abstract | Proposal to provide protection for MAPs and enhance authentication procedures |  |
| Purpose | Review, discussion by working group and acceptance into P802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# MAP Protection & Enhanced Authentication for IEEE 802.16m

Sheng Sun

Nortel

DJ Shyy

MITRE

# 1. Introduction

In this contribution a method to enable enhances security services is provided.  MAP protection is required to protect SS/MSs from malicious users trying to masquerade as a legitimate BS.  The approach in this contribution also provides some authentication service much earlier than is traditionally avaliable in the in legacy system [1].  This approach consists of:

- BS uses installed certificate or randomly generated key-pair to calculate a signature over MAPs, and adds signature as an IE to MAP

- BS transmits installed certifcate or randomly generated public key in a control channel

- SS/MS uses this certificate information in control channel to verify the signature in the MAP

There are some issues to consider when evaluating this approach.  The type of certificate used affects the overhead impact when using this apprach.  In [3] Elliptic Curve Cryptography (ECC)-based certificates are suggested to be used in 802.16m networks.  ECC based certificates may mitigate overhead issues, as they could be made to be smaller than RSA-based certificates currently used in the legacy system.

Another issue is the control channel the BS uses to transmit the information.  If the BCH is used, then the certificate would be applied on a cell-wide basis.  Every SS/MS then verifies the same signature that is installed in the MAPs.  If the USCCH is used, then the certificate would be applied to single SS/MS or a group of SS/MSs.  Each certificate then corresponds to a unique signature in MAP, so mutliple certificates would be transmitted in the control channel and multiple signatures sent with MAPs.

Determination of the appropriate certificate and the size of the signature is FFS.

# 2. Text Proposal

[---------------------------------------------------Start of Text Proposal--------------------------------------------------]

# 12 Security

[Insert the following subsection into Section 12]

## 12.x Authorization, Authentication Procedures

[Insert the following subsection into Section 12.x Authorization, Authentication Procedures]

### 12.x.x Authentication Procedure During Initial and Re-acquisition of System Parameters

[Insert the following text into subsection 12.x.x Authentication Procedure During Initial and Re-acquisition of System Parameters]

Optionally, service providers may elect to utilize a pre-ranging authentication procedure.  The purpose of the procedure is to protect SS/MSs from malicious users trying to masquerade as a legitimate BS.  This approach consists of:

- BS uses installed certificate to calculate a signature over MAPs, and adds signature as an IE to MAP

- BS transmits installed certifcate in a control channel

- SS/MS uses this certificate information in control channel to verify the signature in the MAP

A pre-installed certificate or randomly generated key pair can be used to calculate signature.  RSA PKCS #1 certificate or an ECC based certificates may be be used.  Broadcast Channel (BCH) or Unicast Services Control Channel (USCCH) may be used to transmit the information certificate/key information.  If the BCH is used, then the certificate would be applied on a cell-wide basis.  Every SS/MS then verifies the same signature that is installed in the MAPs.  If the USCCH is used, then the certificate would be applied to single SS/MS or a group of SS/MSs.  Each certificate then corresponds to a unique signature in MAP, so mutliple certificates would be transmitted in the control channel multiple signatures sent with MAPs.

Determination of the appropriate certificate and the size of the signature is FFS.

[---------------------------------------------------End of Text Proposal--------------------------------------------------]

# 3. References

[1] "Draft Standard for Local and Metropolitan Area Networks, Part16: Air Interface for Broadband Wireless Access Systems", IEEE P802.16 Rev2/D6, July 2008.

[2] Hamiti, Shkumbin, "The Draft IEEE 802.16m System Description Document", IEEE 802.16m-08/003r4, July 2008.

[3] Reddy, Ranga, et al., “Elliptic Curve Cryptography Authorization and Key Agreement of IEEE 802.16m”, IEEE C80216m-08/881r1, August 2008.

[4] Reddy, Ranga, et al., “MS ID Privacy for IEEE 802.16m Networks”, IEEE C80216m-08/880, August 2008.

[5] Shyy, D.J., “Security and Robustness Enhancement for 802.16e – A SDD proposal for 802.16m”, IEEE C80216m-08/046, January 2008.