| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MBS outer coding |  |
| Date | 2008-09-05 |  |
| Source(s) | Wataru Matsumoto, Toshiyuki Kuze, Shigeru Uchida Mitsubishi Electric Corporation 5-1-1 Ofuna Kamakura, Kanagawa 2478501, JAPAN Philip V. Orlik, Andreas F. Molisch, Zhifeng (Jeff) Tao, Jinyun Zhang Mitsubishi Electric Research Laboratories 201 Broadway Cambridge, MA 02139, USA | Matsumoto.Wataru@aj.MitsubishiElectric.co.jp kuze.Toshiyuki@ah.MitsubishiElectric.co.jp Uchida.Shigeru@dh.MitsubishiElectric.co.jp {porlik, molisch, tao, jzhang}@merl.com |
| Re: | IEEE 802.16m-08/033: Call for Comments and Contributions on Project 802.16m System Description Document (SDD). Target topic: “PHY & MAC aspects of enhanced MBS”. |  |
| Abstract | This contribution proposes Multicast/Broadcast Services in 802.16m. |  |
| Purpose | For discussion and adoption by IEEE 802.16m group |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy and Procedures | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MBS outer coding

Wataru Matsumoto ,Toshiyuuki Kuze, Shigeru Uchida

Mitsubishi Electric Corp

Philip V. Orlik, Andreas F. Molisch, Zhifeng (Jeff) Tao, Jinyun Zhang

MERL

# 1. Introduction

RC-LDPC code is suitable for MBS outer coding. Binary erasure code using RC-LDPC code can be achieved high through-put and high PER performance. Furthermore the RC-LDPC codes can be kind-of rate-less like codes as Fountain codes by a specific repetition. We introduce it for robust MBS services.

# 2. Summary of Proposal

We proposed the construction of an extended parity-check matrix of the RC (Rate-Compatible)-LDPC codes to achieve good performance for wide range of code word length and code rate. The parity-check matrix construction method can support code rates ranging from 1/3 to 4/5 and codeword lengths ranging from 288 bits to 2304bits. The RC-LDPC codes are constructed using a code-rate 1/2 parity-check matrix for code-rate greater than or equal to 1/2. For code rate smaller than ½ an additional parity check matrix is being extended to the ½ parity-check matrix.

The proposed RC-LDPC uses the current 802.16 LDPC as the baseline matrix.  It also uses the same rate-less sub-packets protocol for its rate-less operation.  The proposal provides some small additions and requires no change to the current 802.16e LDPC.

# 3. Proposed Text

##### x.1 Code description

Insert new text at end of the subsection.

We propose the parity check matrix for rate-compatible LDPC(RC-LDPC) codes using the parity-check matrix for rate-1/2 specified in the 802.16e as the following Figure 1.

Extend for Code Rate=1/3

Figure 1 Parity check matrix for rate compatible LDPC codes

Let u and r be information bits as u=u1u2⋯uK and parity bits as r=r1r2⋯rM, respectively.  And Let v be a systematic codeword such that:

v:=u1u2⋯uKr1r2⋯rM. satisfying HM⋅vT=0.

The RC-LDPC encoder consists of a common LDPC encoder and a puncturing device that punctures the parity bits. The systematic bits are not punctured. The decoder for RC-LDPC codes is the same as an ordinary LDPC decoding algorithm with received LLR=0 for puncturing bits.

A set of code rates and un-puncturing parity bits set r for RC-LDPC codes can be represented by:

For the number of un-puncturing bits i(≤M2), r=rl,

For p=0 to 11 { c(p)=0 }

For p=0 to 11 {

For  q=0 to z−1 {

wj,c(j)=rk+1  where j=(p⋅z+q)mod8, k=p⋅z+((p+q)mod8),

c(j)=c(j)+1

}

}

l=0,4,2,6,1,5,3,7,k=1

For p=0 to 7 {

For q=0 to c(p)−1 {

rk=wl(p),q,

If k=i then exit

k=k+1

}

}

For the number of un-puncturing bits i(>M2),

r=rl,  for M2≤l≤i,

where, l is a vector such that l=l(0),l(1),...,l(i),...,l(7),0≤i≤7.

##### x.2 Code block size adjustment

The LDPC code flexibly supports different block sizes for each code rate through the use of an expansion factor.

Each base model matrix has nb=24 columns, and the expansion factor (z factor) is equal to n/24 for code length n. In each case, the number of information bits is equal to the code rate times the coded length n.

Insert into  RC-LDPC block Sizes and the number of subchannels.

The code length N shall cover from 576 bits to 14400 bits for the RC-LDPC code. The supported code-rate shall be equal to and more than 1/3.

The number of slots shall be calculated as follows,

If N (bits) is a multiple of 96,

QPSK can be used as Nsubchannels = N/96,

else if N (bits) is a multiple of 96x2,

16QAM can be used as Nsubchannels = N/(96x2),

else if N (bits) is a multiple of 96x3,

64QAM can be used as Nsubchannels = N/(96x3),

end.

###### x.x.1 Packet encoding for Rate-less code

Insert.

The reordered codeword for RC-LDPC codes are defined such as

v:=v1v2⋯vK+M=u1u2⋯uKr1r2⋯rM.

Insert this subclause

# 5. Conclusions

RC-LDPC code is suitable for MBS outer coding. Binary erasure code using RC-LDPC code can be achieve high through-put and high PER performance. Furthermore the RC-LDPC codes can be kind-of rate-less like codes as Fountain codes by a specific repetition. We introduce it for robust MBS services.