| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | DL SU-MIMO codebooks |  |
| Date Submitted | 2008-09-05 |  |
| Source(s) | Yang Tang, Young Hoon Kwon, Tao Wu, Yajun Kou, Shahab Sanayei, Zhigang Rong, Jianmin Lu, Yinggang Du Huawei | E-mail: ytang@huawei.com |
| Re: | PHY:MIMO; In response to the TGm call for contributions and comments on 802.16m-08/033 for Session 57 |  |
| Abstract | Proposed for DL CL SU-MIMO codebooks |  |
| Purpose | To discuss and adopt in MIMO RG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# DL CL SU-MIMO Codebooks

Yang Tang, Young Hoon Kwon, Tao Wu, Yajun Kou, Shahab Sanayei,

Zhigang Rong, Jianmin Lu, Yinggang Du

Huawei

# Introduction

The codebook(s) for 2, 4 and 8 transmit antennas should be designed and selected based on performance, complexity and some other desirable properties.

- Performance: With a fixed amount of feedback (in bits), the system throughput as well as each individual average throughput are desirable to be superb for a wide range of channel conditions: uncorrelated, correlated and dual-polarized.

- Complexity:

  - With fixed amount of feedback, the codebook design is desirable to minimize the number of codewords searching.

  - The codebook with constant modulus property is desirable to avoid the matrix multiplications.

- Power amplifier balance: Codebook design with constant modulus property is desirable to avoid the power imbalance.

- Feedback overhead: The codebook design is desirable to support differential feedback to lower the feedback overhead.

- Flexibility to extend to other MIMO scenarios: The codebook design should cover both SU-MIMO and MU-MIMO, which in general require different sizes of codebook.

# SU-MIMO codebook design

The proposed SU-MIMO codebook design consists of the base codebook, the localized codebook(s) and corresponding scaling and rotation of the localized codebook(s).

- Base codebook

The base codebook selection can be various depending on various design criteria. For example, in uncorrelated MIMO channel, DFT matrix is one of good options in the sense that the minimum chordal distance is maximized. The base codebook can be defined asCbaseM=[cbase1,cbase2,⋯,cbaseM], where Mdenotes the number of codeword in the base codebook.

- Localized codebook(s)

With the processing of scaling and rotation of the localized codebook, the elements in the localized codebook can be interpreted as a satellite codeword of the the base codebook. Figure 1 depicts the relation between base and localized codebooks.

Figure 1: Relation between base and localized codebooks

The localized codebook is defined as anNt×N matrix ClocalN=[clocal1,clocal2,⋯,clocalN], whereNtis the number of the transmit antenna,Ndenotes the number of the codewords in the localized codebook. Without loss of generality, the centroid of the localized codebook is given asccnt=[1,0,⋯,0]1×NH. In this proposal, the localized codebook is proposed with a constraint that the centroidccnthas equal chordal distance from each codeword in ClocalN. As a result, the localized codebook can be rewritten as

ClocalN=rejθ11rejθ12⋯rejθ1Nr21ejθ21r22ejθ22⋯r2Nejθ2N⋮⋮⋱⋮rNt1ejθNt1rNt2ejθ⋯rNtNejθNtN

in which it is given that i=2Ntrij2+r2=1,j∈1,⋯,N and the distance between the centroid and the codewords in the localized codebook is 1−r2.

- Localized codebook scaling

The chordal distance between the centroid and the codewords in localized codebook can be scaled to accommodate various operation scenarios. In the case when the chordal distance is to be scaled to a value ofα1−r2, the scaled version of the localized codebook is obtained as

ClocalN(α)=1−α2(1−r2)ejθ111−α2(1−r2)ejθ12⋯1−α2(1−r2)ejθ1Nαr21ejθ21αr22ejθ22⋯αr2Nejθ2N⋮⋮⋱⋮αrNt1ejθNt1αrNt2ejθ⋯αrNtNejθNtN

where α is the scaling factor.

- Codeword rotation

With the proposed codeword rotation scheme, the vectorccnt can be rotated to the codeword cbasemin the base codebook, wherem∈1,⋯,M. By doing so, the localized codebook can be also rotated to surroundcbasem. As a result, the centroid of the rotated localized codebook becomes the codeword cbasem. More specifically, the details of the localized codeword rotation to cbasem is given as

Step 1: Find a rotation matrixRm, by which the vectorccnt can be rotated to vectorcbasem.

The rotation matrixRmcan be any unitary matrix with the constraint of

RmHcbasem=ccnt. It is obvious that we can also haveRmccnt=cbasem, which indicates

that the vectorccnt has been rotated to vectorcbasem.

Step 2: Rotate the localized codebook ClocalNby Rm

The rotated localized codebook is denoted by Clocal(N,m)=RmClocalN

- Codebook design

The eventual codebook, denoted by CXNt,r, where X=M×Ndenotes the size of the codebook and ris the number of rank, can be obtained based on the scaled and rotated localized codebooks. It is noted that CXNt,ris an Nt×(MN)-sized matrix and given by CXNt,r=Clocal(N,1),Clocal(N,2),⋯,Clocal(N,M).

  - Codebook refinements

In various deployment scenarios (e.g. MU-MIMO and SU-MIMO), the required quantization accuracy (or the size of the codebook) can be various. As a result, the codebook CXNt,rcan be further refined. More specifically, CXNt,rcan be taken as the base codebook. With predefined scaling factor and localized codebook, a new codebook can be easily obtained based the aforementioned procedure.

  - Hierarchical structure

The inherent hierarchical structure of the proposed codebook can be depicted by Figure xx

Figure 2: Diagram of the hierarchical structure of proposed codebook

In Figure 2, Ni represents the localized codebook size associated with the i-th layer (refinement).

It is noted that the inherent hierarchical structure of the proposed scheme can both simplify the codeword searching and facilitate the differential feedback.

    - Codeword searching

In the proposed codebook, the codeword searching can be simplified by the hierarchical structure. More specifically, in each layer, only the nodes which are associated with the selected nodes in the upper layer are considered. Consequently, for a codebook with Nlayerlayers, the number of searched codewords is M+i=1NlayerNi.

However, with the same size of the conventional codebook, Mi=1NlayerNicodewords should be entirely searched. It is obvious that the codeword searching complexity is significantly simplified by the proposed codebook design.

    - Differential feedback

In the proposed codebook, the nodes associated with the same mother node in the upper layer systematically have higher correlations and near to each other in the hyperspace. This feature can easily facilitate the differential feedback and save the feedback overhead.

- Proposed Remedy

To add to the current text in Line 36, page 71

In FDD and TDD systems, unitary codebook based precoding are supported.

The following tables define the hierarchical structured codebooks for 2 Tx and 4 Tx antennas

Table 6: 3-bit, 2x1 codebook V(2,1,3) for 2 Tx antennas

| Index | c812,1 |  | c822,1 |  | c832,1 |  | c842,1 |  | c852,1 |  | c862,1 |  | c872,1 |  | c882,1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary Index | 000 |  | 001 |  | 010 |  | 011 |  | 100 |  | 101 |  | 110 |  | 111 |
| Codeword | [1,1] | 1,eπ4j |  | [1,j] |  | 1,e3π4j |  | [1, -1] |  | 1,e−π4j |  | [1,-j] |  | 1,e−3π4j |  |

Table 7: 2-bit 2x2 codebook V(2,2,2) for 2 Tx antennas

| Index | c412,2 | c422,2 | c432,2 | c442,2 |
| --- | --- | --- | --- | --- |
| Binary Index | 00 | 01 | 10 | 11 |
| Codeword | c812,1,c852,1 | c822,1,c882,1 | c832,1,c872,1 | c842,1,c862,1 |

| Index | c1614,1 | c1624,1 | c1634,1 | c1644,1 |
| --- | --- | --- | --- | --- |
| Binary index | 0000 | 0001 | 0010 | 0011 |
| Codeword | [-0.5, -0.5, -0.4514-0.2151j, -0.4758+0.1536j] | [-0.5j, 0.1536-0.4758j -0.2151 -0.4514j -0.5j] | [-0.2151 -0.4514j, -0.5j, -0.5j, 0.1536-0.4758j] | [ 0.4514+0.2151j, 0.4758-0.1536j, 0.5, 0.5] |
| Index | c1654,1 | c1664,1 | c1674,1 | c1684,1 |
| Binary index | 0100 | 0101 | 0110 | 0111 |
| Codeword | [-0.5, 0.5j, 0.4514+0.2151j, -0.1536-0.4758j] | [-0.5j, -0.4758-0.1536j 0.2151+0.4514j, 0.5] | [-0.2151-0.4514j, -0.5, 0.5j 0.4758+0.1536j] | [ 0.4514+0.2151j, -0.1536-0.4758j, -0.5, 0.5j] |
| Index | c1694,1 | c16104,1 | c16114,1 | c16124,1 |
| Binary index | 1000 | 1001 | 1010 | 1011 |
| Codeword | [-0.5, 0.5, -0.4514-0.2151j, 0.4758-0.1536j] | [-0.5j, -0.1536+0.4758j, -0.2151-0.4514j, 0.5j] | [-0.2151-0.4514j, 0.5j, -0.5j -0.1536+0.4758j] | [ 0.4514+0.2151j, -0.4758+0.1536j, 0.5, -0.5] |
| Index | c16134,1 | c16144,1 | c16154,1 | c16164,1 |
| Binary index | 1100 | 1101 | 1110 | 1111 |
| Codeword | [-0.5, -0.5j, 0.4514+0.2151j, 0.1536+0.4758j] | [-0.5j, 0.4758+0.1536j, 0.2151+0.4514j, -0.5] | [-0.2151-0.4514j, 0.5, 0.5j -0.4758-0.1536j] | [ 0.4514+0.2151j, 0.1536+0.4758j, -0.5, -0.5j] |

Table 8: 4-bit 4x1 codebook V(4,1,4) for 4 Tx antennas

Table 9: 4-bit 4x2 codebook V(4,2,4) for 4 Tx antennas

| c1614,2 | c1624,2 | c1634,2 | c1644,2 | c1654,2 | c1664,2 | c1674,2 | c1684,2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0000 | 0100 | 1000 | 1100 | 0010 | 0110 | 1010 | 1110 |
| c1614,1,c1654,1 | c1624,1,c1664,1 | c1634,1,c16114,1 | c1644,1,c16124,1 | c1654,1,c1694,1 | c1664,1,c16104,1 | c1674,1,c16154,1 | c1684,1,c16164,1 |
| c1694,2 | c16104,2 | c16114,2 | c16124,2 | c16134,2 | c16144,2 | c16154,2 | c16164,2 |
| 0001 | 0101 | 1001 | 1101 | 0011 | 0111 | 1011 | 1111 |
| c1614,1,c16134,1 | c1624,1,c16144,1 | c1634,1,c16154,1 | c1644,1,c16164,1 | c1694,1,c16134,1 | c16104,1,c16144,1 | c16114,1,c16154,1 | c16124,1,c16164,1 |

Table 10: 4-bit 4x3 codebook V(4,3,4) for 4 Tx antennas

| c1614,3 | c1624,3 | c1634,3 | c1644,3 |
| --- | --- | --- | --- |
| 0000 | 0100 | 1000 | 1100 |
| c1614,1,c1654,1,c1694,1 | c1624,1,c1664,1,c16104,1 | c1634,1,c1674,1,c16114,1 | c1644,1,c1684,1,c16124,1 |
| c1654,3 | c1664,3 | c1674,3 | c1684,3 |
| 0001 | 0101 | 1001 | 1101 |
| c1614,1,c1654,1,c16134,1 | c1624,1,c1664,1,c16144,1 | c1634,1,c1674,1,c16154,1 | c1644,1,c1684,1,c16164,1 |
| c1694,3 | c16104,3 | c16114,3 | c16124,3 |
| 0010 | 0110 | 1010 | 1110 |
| c1614,1,c1694,1,c16134,1 | c1624,1,c16104,1,c16144,1 | c1634,c16114,c16154,1 | c1644,1,c16124,1,c16164,1 |
| c16134,3 | c16144,3 | c16154,3 | c16164,3 |
| 0011 | 0111 | 1011 | 1111 |
| c1654,1,c1694,1,c16134,1 | c1664,1,c16104,1,c16144,1 | c1674,c16114,c16154,1 | c1684,1,c16124,1,c16164,1 |

Table 11: 2-bit 4x4 codebook V(4,4,2) for 4 Tx antennas

| c414,4 | c424,4 | c434,4 | c444,4 |
| --- | --- | --- | --- |
| 00 | 01 | 10 | 11 |
| c1614,2,c1654,2 | c1624,2,c1664,2 | c1634,2,c1674,2 | c1644,2,c1684,2 |

# Conclusions

      - The proposed codebook is unitary matrices based codebook with constant modulus.

      - The inherent hierarchical structure of the proposed can both simplify the codebook searching and facilitate the differential feedback.

      - The proposed codebook demonstrates good throughput performance in both uncorrelated channel and correlated channel.

      - The proposed codebook has full flexibility to be expended to any size of the codebook.

# Appendix : codebook constructions

I-1: Codebooks contruction for 2 Tx antennas

    - Base codebook

Cbase2=22×1111eπ8je5π8je−3π8je−7π8j

    - Localized codebook

Clocal4=22×1.3604 - 0.2706i1.3604 + 0.2706i0.0538 + 0.2706i0.0538 - 0.2706i

    - Scaling factorα=22

I-2: Codebooks contruction for 4 Tx antennas

    - Base codebook

Cbase4=12×11111−j−1j1−11−11j−1−j

    - Localized codebook

Clocal4=12×−1−1j−jjj−11j−j−1−11−1−j−j

    - Scaling factorα=110

Simulations:

|  | Codebook type | IEEE802.16e (6bit) | Huawei Codebook (4bit) | DFT codebook (4bit)DDDDDasd |
| --- | --- | --- | --- | --- |
| Correlated Channel | Relative gain | 0% | 2.5% | 2 % |
| Zero correlation | Relative gain | 0% | -2.9% | -3% |