| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Compressed Paging Group MAP for IEEE 802.16m system |  |
| Date Submitted | 2008-09-06 |  |
| Source(s) | Juejun Liu, Lei Jin, Ke Zeng, Lina Liu Huawei Technologies | E-mail: liulina@huawei.com |
| Re: | MAC: Idle Mode; in response to the TGm Call for Contributions and Comments 802.16m-08/033 for Session 57 |  |
| Abstract | This contribution proposes a compressed paging Group MAP for IEEE 802.16m system. |  |
| Purpose | For discussion and adoption by IEEE 802.16m group |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Compressed Paging Group MAP for IEEE 802.16m system

Juejun Liu, Lei Jin, Ke Zeng, Lina Liu

Huawei Technologies

# Introduction

In wireless telecommunication system, when a MS has no active traffic, it can enter idle mode to lessen power consumption. Idle mode MS should update its location if any location update condition is met. Paging group update is one of such conditions.

Paging group is a logical entity that contains several BSs. When an MS stays within the coverage area of one paging group, it does not need to transmit in UL, yet the MS can be paged in the DL. MS shall monitor the change of paging group. When MS detects that the paging group ID it holds is not included the paging group IDs broadcast by BS, it should update its location.

In IEEE802.16e, BS periodically broadcasts paging group IDs it belongs to. That information is carried in DCD message. When an idle mode station does cell reselection, it must wait for the next DCD message to judge whether location update is needed or not. In the period of MS waiting for DCD, there may be a paging message in its old paging group, so MS will miss this message.

This proposal provides a paging group advertisement scheme to shorten LU period to some extent.

# 2. Paging Group MAP

In this proposal, we propose that BS broadcast a fixed length 18-bit compressed Paging Group MAP in BCH (S-BCH is preferred).This MAP comprises two parts: 2 bits Type Identifier (TI) and 16 bits MAP content. Type Identifier indicates the format of MAP content.

| TI | MAP content |
| --- | --- |
| 0b00 | One Full Paging Group ID |
| 0b01 | Two Paging Group IDs are mapped to 8 bits hashed bitmap respectively |
| 0b10 | Four Paging Group IDs are mapped to 4 bits hashed bitmap respectively |
| 0b11 | 16 Paging Group IDs are mapped to 1 hashed bit respectively |

Figure 1 show the example of Paging Group MAP.

Figure 1 Example of Extended Paging Group MAP

TI=0b00 is used when BS belongs to only one Paging Group.

# 3. Detailed description of Paging Group MAP

In this section, we describe the action of BS and MS when Type Identifier equals 0b11 (TI=0b01 and 0b10 are the same).

When BS belongs to several Paging Group, each Paging Group ID is mapped to a certain bit in this Paging Group MAP Using a hash algorithm (FFS). Figure 2 show an example.

Figure 2 Example of Paging Group ID Mapping

From figure 2, one Paging Group is mapped into a bit, and different Paging Group can be mapped to the same bit.

If a BS A belongs to Paging Group 1 and 2, it should send Paging Group MAP as “111000100000000000”.

If a BS B belongs to Paging Group 3 and 4, it should send Paging Group MAP as “110000000001000010”.

If a BS C belongs to Paging Group 2 and 3, it should send paging Group MAP as “110000100001000000”.

If a MS has updated its location in BS A, and the PG ID assigned to it is 1, when it moves from BS A to B, it can determined from Paging Group MAP that its PG ID does not belongs to BS B. MS can immediately update its location without waiting to decode the full PG ID list.

In another case that MS who has assigned PG ID 2 moves from BS A to BS C, because bit 4 of Paging Group MAP of A and C are all set, MS can not distinguish whether these two BS belongs to the same Paging Group 2 or different Paging Group are mapped to the same bit, so MS must decode full PG ID list advertised in extended system information message like 16e DCD message to know if location update is needed. But this case can be avoided by network planning, that is geographically adjacent Paging Groups are assigned IDs which maps to different bits in Paging Group MAP.

# 4. Summary

In this contribution, we propose a scheme of Paging Group MAP. 18 bits long Paging Group MAP is carried in BCH(S-BCH is preferred). Below are advantages of this scheme:

- Shorten LU delay since there is no need to wait for full Paging Group advertisement.

- BCH overhead reduction. When there are more than one Paging Groups in one BS, no need to carry full list in BCH.

- Fixed bit length is easy to design for BCH to guarantee the coverage of BCH

- Flexible format and adaptive to Paging Group number.

- Potential power savings for idle MS since MS need not decode extended system control information message

# Proposed Text for SDD

[Insert the following text into SDD Text Proposal by AHG Chair for Idle Mode]

------------------------------------------------------Start of the Text----------------------------------------------------

10.x. Power Management

10.x.2.3 Location Update

10.x.2.3.1 Normal Paging group update

The MS shall perform the Location Update process when the MS detects a change in paging group. The MS shall detect the change of paging group by monitoring the paging group identifiers, Paging Group IDs, which are transmitted by the Preferred BS in the extended system control information during the MS Paging Listening Interval. If the Paging Group IDs detected do not include the Paging Group to which the MS belongs, the MS shall determine that the paging group has changed.

10.x.2.3.2 Enhanced Paging group update

The MS shall perform the Location Update process when the MS detects a change in paging group. The MS shall detect the change of paging group by monitoring the compressed paging group MAP, which are transmitted by the Preferred BS in the S-BCH during the MS Paging Listening Interval. If the Paging Group IDs detected do not include the Paging Group to which the MS belongs, the MS shall determine that the paging group has changed.

BS broadcasts a compressed Paging Group MAP in BCH to notify the paging group it belongs to. Paging group MAP comprises two parts: 2 bits Type Identifier (TI) and 16 bits MAP content. Type Identifier indicates the format of MAP content.

| TI | MAP content |
| --- | --- |
| 0b00 | One Full Paging Group ID |
| 0b01 | Two Paging Group IDs are mapped to 8 bits hashed bitmap respectively |
| 0b10 | Four Paging Group IDs are mapped to 4 bits hashed bitmap respectively |
| 0b11 | 16 Paging Group IDs are mapped to 1 hashed bit respectively |

Figure xx show the paging group MAP

Figure xx Paging Group MAP

-------------------------------------------------------End of the Text----------------------------------------------------