| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Antenna permuted CDD plus MIMO |  |
| Date Submitted | 2008-9-5 |  |
| Source(s) | Xiaolong ZHU, Hongwei YANG, Dong LI, Liyu CAI Alcatel Shanghai Bell Co., Ltd. | Voice: +86-21-50554550 Fax: +86-21-50554554 Mail to: hongwei.yang@alcatel-sbell.com.cn |
| Re: | SDD Session 56 Cleanup, call for PHY details; response to IEEE 802.16m-08/033 Call for Contributions and Comments on Project 802.16m System Description Document (SDD) for Session 57 Detailed physical layer comments on: Open-loop MIMO in section 11.8.2.1.1 |  |
| Abstract | In the last 16m session #56, several OL SU-MIMO schemes were determined as FFS. This contribution proposes that antenna permuted CDD plus MIMO be specified into SDD due to the fact that it performs the best among those schemes based on dedicated pilot. |  |
| Purpose | Adopt concept and proposed text into SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Antenna Permuted CDD plus MIMO

Xiaolong ZHU, Hongwei YANG, Dong LI, Liyu CAI

Alcatel Shanghai Bell Co., Ltd.

# 1 Introduction

In the last 16m session #56, several OL SU-MIMO are determined as FFS, which includes among others the antenna permuted cyclic diversity delay (CDD) plus MIMO (including STBC/SFBC or SM) scheme, according to July meeting comment #64 in C80216m-08_880.

To harmonize the evaluation and comparison among various candidate OL SU-MIMO schemes [1], an OL SU-MIMO subgroup was established. Based on the schedule of this subgroup, we gave the formulation of antenna permuted CDD plus MIMO in the contribution of IEEE C80216m-MIMO-08_009 [2].

Basically, the candidate OL SU-MIMO schemes can be divided into two classes. One is based on the common pilot while the other is applying the dedicated pilot. There is no consensus to which one is better. For example, contribution [3] states that the common pilot based schemes outperform those with the dedicated pilot, while another contribution [4] concludes that schemes with dedicated pilot show better performance than those with common pilot due to pilot overhead gain.

As compared with the common pilot, OL SU-MIMO schemes based on dedicated pilot have among others the following two merits. First, they save greatly the pilot overhead. More importantly, 16m base station may configure up to 8 transmit antennas in some scenarios, but only 4Tx pilot pattern is defined. So, it is expected that dedicated pilot shall at least be supported in 16m, particularly considering the antenna setting of 8Tx. Second, dedicated pilot based MIMO schemes can be designed as transparent to terminals, which simplifies greatly the implementation complexities of mobile terminals. As a result, we suggest that dedicated pilot based OL SU-MIMO schemes should be included in the case of 4Tx or 8Tx antennas.

# 2 Antenna permuted CDD plus MIMO

The combination of CDD with STBC/SFBC or SM is now well known, which is feasible to both 4Tx and 8Tx. With a small delay setting, CDD plus MIMO can be transparent to the terminals in the sense that MIMO and CDD plus MIMO make no major difference for channel estimation and data decoding at the MS side.

The basic idea of antenna permuted CDD plus MIMO (including STBC/SFBC and SM) is that on the top of CDD plus MIMO, an antenna permutation function is applied to enhance the robustness against the spatial correlation. By antenna permutation, the mapping from streams to physical antennas can change from one Resource Unit (RU) to another or from one group of RUs to another. It applies also to both 4Tx and 8TX. For the case of 4Tx, the detailed formulation of antenna permuted CDD plus MIMO can be found in [2].

For the rate-1 transmit diversity scheme (i.e., antenna permuted CDD plus STBC/SFBC) and the rate-2 spatial multiplexing scheme (i.e., antenna permuted CDD plus SM), only the 2-stream orthogonal (dedicated) pilot pattern is required, which saves the pilot overhead greatly. Since the precoder including antenna permutation is performed on the basis of RU, antenna permuted CDD plus MIMO is also transparent to terminals.

## Simulation results

The contribution [3] gives a complete performance comparison among various candidate schemes. It can be seen that the antenna permuted CDD plus MIMO always works best or among the best as long as the dedicated pilot based schemes are concerned.

In this contribution, we only show the merit of antenna permutation on the top of CDD plus MIMO. Simulation conditions are listed in Table 1. Different transmit antenna spacing implies a different channel correlation level. Generally, the smaller the spacing, the stronger is the channel correlation. Antenna permutation is conducted only in the time domain slot by slot.

The simulation results are plotted in Figure 1 and Figure 2, respectively. From these two figures, we can see that for independent or weakly correlated channels, the AP-CDD+MIMO has almost the same performance as the CDD+MIMO scheme, but for correlated channels, the antenna permutation technique shall achieve a gain of about 0.4~0.8 dB (at the BLER of 0.01). We point out that for high correlated channels such as in suburban macro cells and/or antenna permutation conducted in both time and frequency domains, more gain will be expected by antenna permutation.

Table 1. Simulation Conditions

| Parameter | Assumption |
| --- | --- |
| OFDM parameters | Carrier frequency = 2.5 GHz FFT size = 1024; CP length = 128 samples WiMAX downlink, PUSC permutation |
| Channel model | 3GPP SCME - Urban Macro, 30kmph |
| Channel code | CTC, code rate = 1/2 |
| Modulation scheme | 16QAM |
| Antenna configuration | 4 transmitter with antenna spacing of {0.5, 10} |
| Channel estimation | Perfect channel estimation |
| Delay samples for CDD | [2] |
| Antenna Permutation | Only in the time domain: slot by slot |
| Decoder | STBC: Alamouti decoder SM: QRM-MLD |

Figure 1: Performance comparison of CDD+STC and AP-CDD+STC.

Figure 2: Performance comparison of CDD+SM and AP-CDD+SM.

3. Proposed SDD text

------------------------------------------- Start of Proposed SDD Text --------------------------------------------------------

11.8.2.1.1 Open-loop SU-MIMO

On a given frequency resource k [size is FFS], the precoding matrix P can be defined using the following equation:

P(k) = D(k)W(k), Equation 4

The precoder is composed of two matrices. The first matrix W(k) is an NT × M matrix, where NT is the number of transmit antennas and M is the numbers of streams. The matrix W(k) is selected from a predefined unitary codebook, and changes every u subcarriers and/or v OFDM symbols. A codebook is a unitary codebook if each of its matrices consists of columns of a unitary matrix. [The detailed unitary codebook, and the parameters u,v are FFS.]

11.8.2.1.1.1 Transmit Diversity

For the 4Tx rate-1, the output of the precoder is a 4  2 matrix

y=D×W×z, Equation 9

where W is a 4  2 unitary precoder and D is a 4  4 diagonal delay matrix. Note that W and D may be frequency/time dependent as described in section 11.x.2.1.1.

For the 8Tx rate-1, the output of the precoder is a 8  2 matrix

y=D×W×z, Equation 10

where W is a 8  2 unitary precoder and D is a 8  8 diagonal delay matrix. Note that W and D may be frequency/time dependent as described in section 11.x.2.1.1.

11.8.2.1.1.2 Spatial Multiplexing

For the 4Tx rate-2 mode, the output of the precoder is a 4  1 vector

y=D×W×z, Equation 14

where W is a 4  2 unitary precoder and D is a 4  4 diagonal delay matrix. Note that W and D may be frequency/time dependent as described in section 11.x.2.1.1.

For the 8Tx rate-2 mode, the output of the precoder is a 8 × 1 vector 37 38 39 40 41

y=D×W×z, Equation 15

where W is a 8 × 2 precoder and D is a 8 × 8 diagonal phase matrix. Note that W and D may be frequency/time dependent as described in section 11.8.2.1.1.

------------------------------------------- End of Proposed SDD Text --------------------------------------------------------

Reference

[1] IEEE C80216m-MIMO-08-029, “Consolidated document of the various OL SU-MIMO schemes”

[2] IEEE C80216m-MIMO-08_009, “Formulation for a proposal of DL OL SU-MIMO”

[3] IEEE C80216m-MIMO-08_058, “Performance Evaluation of DL Open Loop SU-MIMO Schemes”

[4] IEEE C80216m-MIMO-08_054, “Performance comparison of IEEE 802.16m DL OL-SU-MIMO”