| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | # Long CP frame structure design enabling time-multiplexed unicast and E-MBS |  |
| Date Submitted | 2008-9-11 |  |
| Source(s) | Dong Li, Hongwei Yang, Liyu Cai Alcatel Shanghai Bell Co., Ltd. | Voice: + 86-21-50554550; Fax:+ 86-21-50554554 Mail to: hongwei.yang@alcatel-sbell.com.cn |
| Re: | SDD Session 56 Cleanup, call for PHY details; response to IEEE 802.16m-08/033 Call for Contributions and Comments on Project 802.16m System Description Document (SDD) for Session 57 Detailed physical layer comments on: Long CP structure design in sections 11.3 and 11.4.1 |  |
| Abstract | This contribution analyzed four optional solutions for long CP frame structure design, including unfixed subframe duration with 1/4 CP, fixed subframe duration while halving subcarrier spacing and fixed subframe duration & subcarrier spacing with 5.5 OFDM symbols and 5 OFDM symbols per subframe. Based on the analysis, the fourth solution, i.e., fixed subframe duration & subcarrier spacing with 5 OFDM symbols per subframe is recommended for the long CP structure design. |  |
| Purpose | Adopt concept and proposed text into SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Long CP frame structure design enabling time-multiplexed unicast and E-MBS

Dong LI, Hongwei Yang, Liyu Cai

Alcatel Shanghai Bell Co., Ltd.

# 1 Introduction

From the functional and performance requirements for E-MBS in 16m SRD, the dedicated E-MBS carrier and mixed unicast/E-MBS carrier deployment scenarios shall be supported for flexible implementation of the enhanced multicast and broadcast services in 16m (page 14 and 20 in [1]). The mixed unicast-broadcast transmission mode is especially attractive due to unnecessary additional dedicated carrier allocation. The support for the mixed transmission mode is a common view of most operators [3].

Two basic approaches to implement the mixed unicast-broadcast transmission are TDM and FDM. In the companying contribution [4], we compare the multiplexing methods of TDM and FDM for the mixed unicast-broadcast transmission and we prefer the TDM approach considering the following advantages such as: 1) simple signaling for multiplexing; 2) MS can go into sleep mode during irrelevant time intervals and 3) efficient usage of long CP and short CP.

Multi-cell SFN transmission of E-MBS will have to be relied on to provide sufficiently high SINR to achieve the target E-MBS performance specified in [1] (e.g., 4bps/Hz for inter-site distance of 0.5km over 95% coverage area). The SFN transmission will lead to large channel delay spread (e.g., 20us or even up to about 30us depending on application scenarios such as cell radius, cell number in SFN network, radio channel power delay profile), which essentially requires a long CP configuration to avoid inter-symbol interference. However, in the current 16m SDD [2], two CP lengths have been defined as normal CP (11.429us) and short CP (5.714us). The support for long CP has been required in the SDD (stated as “A longer CP size is used in channels with long delay spread” in page 31 of [2]), but no solutions are provided yet.

This contribution aims to provide the long CP frame structure design, which can enable mixed transmission of unicast traffic with normal or short CP and MBS traffic with long CP in TDM mode. Should be noted that this frame structure can also be applied to FDM mode, in which case the same long CP is used for both unicast and broadcast.

# 2 Possible solutions for long-CP frame structure design

## 2.1 Option 1: unfixed subframe duration with 1/4-CP

In this option, the CP length is configured to 1/4 of the useful OFDM symbol. In this case, there are totally 43 OFDM symbols in each 5ms radio frame. The basic parameters for this option are listed in Table 1.

Table 1

| Parameters | Value |
| --- | --- |
| Subcarrier spacing (KHz) | 10.9375 |
| CP length (us)，TCP | 22.8571 |
| Useful symbol time (us)，Tu | 91.4286 |
| Symbol time (us)，Ts= TCP+Tu | 114.2857 |
| Num of OFDM symbols per frame | 43 |
| Num of OFDM symbols per subframe | 5 or 6 |
| Idle time (us) | 85.714 |

The problem with this solution is that the 43 OFDM symbols cannot evenly distributed to the 8 subframes within one frame. Variable subframe duration will have to be applied if the 43 symbols are accommodated into the 8 subframes. For example, five out of the 8 subframes have 5 OFDM symbols each and the rest three subframes have 6 OFDM symbols each (i.e., 5*5+6*3=43). Figure 1 shows this configuration.

Figure 1 Illustration of frame structure with long CP in solution 1 (an example)

Applications in mixed unicast-broadcast transmission:

For this option, the subframe duration is unfixed (0.6857ms or 0.5714ms) and isn’t equal to the subframe duration of other CP lengths (subframe duration is 0.617ms for 1/8 CP configuration). Therefore, different CP configurations cannot coexist within one frame. In this case, the unicast with smaller CP length and SFN MBS with long CP cannot be multiplexed at the subframe level within one 5ms radio frame.

If the multiplexing level is 5ms frame, i.e., CP length varies frame by frame, some problems will arise as follows

- Possibly there isn’t enough MBS traffic to occupy the long CP frames, thus lead to resource waste;

- The presence of long CP MBS frame will result in significant impact on the performance of unicast frames (e.g., unicast traffic and feedback cannot be transmitted timely).

Based on above consideration, the multiplexing level of frame isn’t a practical and feasible configuration.

Additionally, there are some other problems with this long CP solution as follows

- Legacy support (e.g., TDM at subframe level in DL/UL) may be disabled within the long-CP frames (actually, this problem is also with the short-CP frame defined in the current 16m SDD);

- Since the subframe durations are different from that of normal CP, the DL/UL switching points cannot be aligned exactly in time. The misaligned switching points may result in some interference.

Due to the above problems, option 1 for long-CP solution isn’t recommended.

2.2 Option 2: fixed subframe duration and halved subcarrier spacing

In this option, the CP length is enlarged by halving the intercarrier spacing for 1/8 CP configuration. In this case, there are totally 24 OFDM symbols in each 5ms radio frame. The basic parameters for this option are listed in the table below.

Table 2

| Parameters | Value |
| --- | --- |
| Subcarrier spacing (KHz) | 5.4687 |
| CP length (us)，TCP | 22.857 |
| Useful symbol time (us)，Tu | 182.857 |
| Symbol time (us)，Ts= TCP+Tu | 205.714 |
| Num of OFDM symbols per frame | 24 |
| Num of OFDM symbols per subframe | 3 |
| Idle time (us) | 62.857 |

In this solution, the 24 OFDM symbols can be evenly distributed to the 8 subframes with each containing 3 OFDM symbols. In this case, the subframe duration is about 0.617ms, which is exactly the same as that of the normal 1/8 CP configuration. Figure 2 shows this configuration.

Figure 2 Illustration of frame structure with long CP in solution 2(an example)

Applications in mixed unicast-broadcast transmission:

In this solution, since the subframe duration is exactly the same as that of normal 1/8 CP configuration, the unicast with normal CP length and SFN MBS with long CP can be multiplexed at the level of subframe within one 5ms frame, which is illustrated in figure 3. In this figure, the MBSFN subframes (i.e., blue blocks) apply the parameters listed in table 2, while the unicast subframes (i.e., green blocks) use the parameters of normal CP length shown in Table 3.

Figure 3 Illustration of mixed unicast-broadcast transmission in solution 2(an example)

Table 3

| Parameters | Value |
| --- | --- |
| Subcarrier spacing (KHz) | 10.9375 |
| CP length (us)，TCP | 11.428 |
| Useful symbol time (us)，Tu | 91.428 |
| Symbol time (us)，Ts= TCP+Tu | 102.857 |
| Num of OFDM symbols per frame | 48 |
| Num of OFDM symbols per subframe | 6 |
| Idle time (us) | 62.857 |

Potential problems with this solution:

In this solution, the reduction of subcarrier spacing will incur an additional vulnerability to Doppler-induced frequency dispersion due to inter-subcarrier interference (ICI) that accompanies the reduction in subcarrier separation from 10.9375kHz to 5.4687kHz. Figure 4 shows the SIR due to ICI for these two subcarrier separations at 2.5GHz. The data of Figure 4 is computed according to a reasonable approximation to the ICI power given by [5].

PICI=(2πfdΔf)212                                   (1)

where fd is the Doppler frequency and ∆f is the subcarrier separation. Equation (1) and figure 4 suggest a 6dB loss of SIR from every halving of subcarrier separation. The performance degradation is significant, especially at high velocities.

Figure 4 Signal to ICI power ratio at different velocities for two subcarrier spacings

2.3 Option 3: fixed subframe duration and subcarrier spacing (5.5 OFDM symbols/subframe)

In this solution, it is assumed that the subframe duration fixed to 0.617ms while the subcarrier spacing remain unchanged. The subframe structure with long CP length in this solution is shown in Figure 5. From this figure, we can see that there are totally 5.5 OFDM symbols in each subframe, that is, one half OFDM symbol is used to fit the subframe duration. Actually, this subframe with 5.5 OFDM symbols can be easily implemented using single set of FFT/DFT modules. To form half OFDM symbol, traffic data can be loaded only on even-numbered or odd-numbered subcarriers. After full-size IFFT transform, the half OFDM symbol is obtained by removing the first or second half of the time samples. The parameters of this solution are shown in table 4.

Table 4

| Parameters | Value |
| --- | --- |
| Subcarrier spacing (KHz) | 10.9375 |
| CP length (us)，TCP | 18.9286 or 19.1071 |
| Useful symbol time (us)，Tu | 91.428 (45.714 for half symbol) |
| Symbol time (us)，Ts= TCP+Tu | 110.54 or 110.36 (64.64 for half symbol) |
| Num of OFDM symbols per frame | 40 full symbols and 8 half symbols |
| Num of OFDM symbols per subframe | 5 full symbols and one half symbol |
| Idle time (us) | 62.857 |

Figure 5 Illustration of the subframe structure with long CP in solution 3 (an example)

Applications in mixed unicast-broadcast transmission:

In this solution, since the subframe duration is exactly the same as that of normal 1/8 CP configuration, the unicast with normal CP length and SFN MBS with long CP can be multiplexed at the level of subframe within one 5ms frame, which is illustrated in figure 6. The potential problem with this solution is that the presence of half symbol incurs some inconvenience in physical resource mapping and pilot allocations, but this can be solved elaborately in SDD stage 3.

Figure 6 Illustration of the frame structure with mixed unicast-broadcast transmissions

2.4 Option 4: fixed subframe duration and subcarrier spacing (5 OFDM symbols/subframe)

In the previous subsection, the fixed subframe duration and subcarrier spacing is assumed and the subframe structure is designed to contain 5.5 OFDM symbols. The half OFDM symbol is designed to sufficiently exploit the available frequency and time resource, but the cost is that it will bring some inconvenience in baseband processing (e.g., necessitate rate matching and specific resource mapping). In this subsection, we will investigate another solution (termed option 4) that distributes the samples in the half OFDM symbol into the other 5 OFDM symbols. In this solution, a longer CP length will be achieved at the cost of reduced time & frequency resource within a subframe.

Here, we consider two fixed subframe durations, one is 0.617ms (which corresponds to the subframe duration of 1/8 CP) and the other is 0.583ms (which corresponds to the 6-OFDM symbol subframe duration of 1/16 CP). The two values are considered for time-multiplexing the E-MBS with 1/8 CP subframe and 1/16 CP subframe, respectively. Note that for 1/16 CP, in each frame there are 5 subframes each containing 6 OFDM symbols and 3 subframes each containing 7 OFDM symbols. We assume that the E-MBS will be conveyed in subframes with 6 OFDM symbols.

The subframe structure parameters corresponding to the two subframe durations of 0.617ms and 0.583ms are listed in Table 5 and 6, respectively. The subframe structure with long CP is illustrated in Figure 7. An example of TDD and FDD Frame structure with mixed CP lengths is illustrated in Figure 8.

Table 5 Parameters for subframe duration of 0.617ms

| Parameters | Value |
| --- | --- |
| Subcarrier spacing (KHz) | 10.9375 |
| CP length (us)，TCP | 31.964 |
| Useful symbol time (us)，Tu | 91.428 |
| Symbol time (us)，Ts= TCP+Tu | 123.393 |
| Num of OFDM symbols per subframe | 5 symbols |
| Idle time at the end of subframe(us) | 0.179 |

Table 6 Parameters for subframe duration of 0.583ms

| Parameters | Value |
| --- | --- |
| Subcarrier spacing (KHz) | 10.9375 |
| CP length (us)，TCP | 25 |
| Useful symbol time (us)，Tu | 91.428 |
| Symbol time (us)，Ts= TCP+Tu | 116.429 |
| Num of OFDM symbols per subframe | 5 symbols |
| Idle time at the end of subframe(us) | 0.714 |

Figure 7 Illustration of the subframe structure with long CP in solution 4 (an example)

Figure 8 Illustration of TDD and FDD Frame structure with mixed CP lengths

3. Proposed SDD text

------------------------------------------- Start of Proposed SDD Text --------------------------------------------------------

[Update the table 2 in section 11.3 as follows]

| Nominal Channel Bandwidth (MHz) |  | 5 | 7 | 8.75 | 10 | 20 |
| --- | --- | --- | --- | --- | --- | --- |
| Over-sampling factor |  | 28/25 | 8/7 | 8/7 | 28/25 | 28/25 |
| Sampling Frequency (MHz) |  | 5.6 | 8 | 10 | 11.2 | 22.4 |
| FFT Size |  | 512 | 1024 | 1024 | 1024 | 2048 |
| Sub-Carrier Spacing (kHz) |  | 10.937500 | 7.812500 | 9.765625 | 10.937500 | 10.937500 |
| Useful Symbol Time Tu (us) |  | 91.429 | 128 | 102.4 | 91.429 | 91.429 |
| Cyclic Prefix (CP) Tg=1/8Tu | Symbol time Ts (us) | 102.857 | 144 | 115.2 | 102.857 | 102.857 |
|  | Number of OFDM Symbols per Frame | 48 | 34 | 43 | 48 | 48 |
|  | Idle Time (us) | 62.86 | 104 | 46.4 | 62.86 | 62.86 |
| Cyclic Prefix (CP) Tg=1/16Tu | Symbol time Ts (us) | 97.143 |  |  | 97.143 | 97.143 |
|  | Number of OFDM Symbols per Frame | 51 |  |  | 51 | 51 |
|  | Idle Time (us) | 45.71 |  |  | 45.71 | 45.71 |
| Cyclic Prefix (CP) Tg=25us | Symbol time Ts (us) | 116.429 |  |  | 116.429 | 116.429 |
|  | Number of OFDM Symbols per subframe | 5 |  |  | 5 | 5 |
|  | Idle time within subframe(us) | 0.714 |  |  | 0.714 | 0.714 |
| Cyclic Prefix (CP) Tg=31.964us | Symbol time Ts (us) | 123.393 |  |  | 123.393 | 123.393 |
|  | Number of OFDM Symbols per subframe | 5 |  |  | 5 | 5 |
|  | Idle time within subframe(us) | 0.179 |  |  | 0.179 | 0.179 |

Note:

- The CP configuration of Tg=25us can be time-multiplexed with 1/16 CP subframes within one frame.

- The CP configuration of Tg=31.964us can be time-multiplexed with 1/8 CP subframes or be applied to all the subframes within one frame.

[Insert a new subsection 11.4.1.3 and change the old subsections of 11.4.1.3, 11.4.1.4 and 11.4.1.5, accordingly]

11.4.1.3 Frame Structure for Long CP

Figure 17a illustrates an example of TDD and FDD frame structure with a CP of 31.964 us. With this CP configuration, the subframe duration is 0.617 ms, containing five OFDM symbols each with duration of 123.393 us. Within one frame, this CP configuration can be applied to all the subframes or alternatively, it can be used to part of the subframes and multiplex with subframes with 1/8 CP. For the CP configuration of 25us, the subframe duration is 0.583 ms, containing five OFDM symbols each with duration of 116.429 us. This CP configuration can be applied to part of the type-1 subframes and multiplex with subframes with 1/16 CP.

Figure 17a TDD and FDD Frame structure with CP length of 31.964 us

------------------------------------------- End of Proposed SDD Text --------------------------------------------------------

Reference

- IEEE 802.16m-07/002r4

- IEEE 802.16m-08/003r4

- 3GPP, R2-063145, “Operators' view on LTE MBMS scenarios of deployment”

- IEEE 802.16m-08/1131r1, “Multiplexing of MBS and unicast in 16m”

- Y. Li, L. J. Cimini, “Bounds on the Interchannel Interference of OFDM in Time-Varying Impairments”, IEEE Trans. Com., vol. 49, no. 3, March 2001

- IEEE C802.16m-08/201, “Subframe Structure with Variable-length CP”

- IEEE C802.16m-08/228r1, “Frame Structure to Support Multiple CP Sizes with Fixed Sub-frame Size”

- IEEE C802.16m-08/436r3, “Frame structure supporting multiple CP sizes”

- IEEE C802.16m-08/644r1, “Multiple CPs for 16m downlink OFDM frame structure”