| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Optional retransmission schemes for E-MBS |  |
| Date Submitted | 2008-9-5 |  |
| Source(s) | Ji LI, Yonggang WANG, Zhongji HU, Bijun ZHANG, Hongwei Yang Alcatel Lucent Shanghai Bell Co., Ltd. | Voice: + 86-21-50554550-9692; Fax:+ 86-21-50554554 mail to: Hongwei.yang@alcatel-sbell.com.cn ji.a.li@alcatel-sbell.com.cn Yonggang.Wang@alcatel-sbell.com.cn |
| Re: | IEEE 802.16m-08/033: Call for Contributions and Comments on Project 802.16m System Description Document (SDD) Contributions on topic: “PHY & MAC aspects of enhanced MBS” |  |
| Abstract | In this proposal, three optional multicast retransmission schemes are proposed. Two of them are very easy to realize. And the network coded one can save some retransmission resource. |  |
| Purpose | To be discussed and adopted by TGm for use in the IEEE802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Optional retransmission schemes for E-MBS

Ji LI, Yonggang WANG, Zhongji HU, Bijun ZHANG, Hongwei YANG

Alcatel Lucent Shanghai Bell Co., Ltd.

# Introduction

The retransmission scheme in E-MBS has not determined now. This proposal suggests three optional retransmission schemes.

# Retransmit on request

One simplest optional retransmission scheme is similar to that of unicast. One packet is retransmitted when any MS in the multicast group requests this packet to be retransmitted, as shown in Fig.1. The retransmitted packet is also broadcasted in the whole cell and thus all MSs can receive it.

Figure 1: retransmission request collection

The conventional 6 parallel stop-and-waits retransmission scheme can be adopted. Maximum retransmission is 3 times. MS requests packet to be retransmitted when the CRC checking failed. BS collects the requests during the transmission of other packets. When there’s request(s) for any packet, BS sets the retransmission request indicator bit (RIB) for this packet to 1. In each transmission slot, BS checks the RIB of this slot. If RIB is 1, the former packet in this slot is retransmitted. If the RIB is 0, new packet is read and transmitted in this slot. Fig.2 shows the retransmission procedure.

Figure 2: multicast retransmission procedure

Expect for the retransmission requests collection, this scheme is totally the same with the one of unicast. Thus it is very easy to realize. However this scheme occupies much transmission resource when the multicast group is large.

# Retransmission threshold

One retransmission threshold is set. When the requests on one packet are larger than the threshold α, BS sets RIB of this packet into 1. Thus only the packets requested by α or more than α UEs can be retransmitted, the packets requested by less than α UEs will be given up.

This scheme saves some of retransmission resource and it is also easy to realize. However, some UEs’ receiving performance decreases.

# Random network coding retransmission

In multicast mode, the packet lost at one MS may be successfully received by the other MSs. Thus the former received packets may be made use of to save retransmission resource.

One error pattern example of a multicast group is shown in Fig.1. For conventional retransmission scheme example, 4 packets, namely P3, P4, P5 and P7, need to be retransmitted. Notice P3 is successfully received at MS3 and MS2 has known P4, if P3 and P4 are retransmitted together, for example, by the operation of XOR, the original packets can be retrieved by the operation of P3⊕P4⊕P4 at MS2 and P3⊕P4⊕P3 at MS3. By this way, only two network coded packets, namely P3⊕P4 and P5⊕P7, need to be retransmitted.

Figure 3: One error pattern example of a multicast group

The network coding principle can be deliberately designed to ensure that the coded packets can be decoded at all MSs. Random encoding way can also be used. The definite coding way can save resource at most. On the other hand, the random way makes least change on the existing interface and procedure.

For random network coding, MS requests packet to be retransmitted when the CRC checking failed. BS collects the requests and saves the requested packet numbers. In each transmission slot, BS checks the retransmission buffer, chooses two packets randomly, encodes them and transmits the encoded packet. Since there’s no need to ensure all the packets to be decodable, the format of NACK needs not to be changed and the processing in BS is of low complexity, and thus the conventional retransmission scheme with 6 parallel stop-and-waits can still be used.

One network coded retransmission scheme is depicted as Fig.4. In Fig.4, 6 RIBs are not enough. There should be one RIB for each ‘active’ packet, which means the packet has not been received successfully or not been given up. RIB=1 means there’s retransmission request(s) on this packet after last transmission, and this packet needs to be retransmitted. RIB=0 means there’s no retransmission request on this packet up to this slot, thus this packet is considered has been successfully received by all MSs and will not be retransmitted any more. ‘x’ means this packet is ‘inactive’, which means it has been received successfully or has been given up after it has been transmitted for 4 times. It can be seen even by this sketch map that, compared with Fig.2, more packets can be transmitted by the network coded retransmission scheme during the same period.

Figure 4: network coded multicast retransmission

# Simulation results

The simulation results of multicast retransmission with network coding are shown in Figure 5. The redundancy factor α shows the amount of network coded packets which will be retransmitted. When the redundancy factor is equal to 1, each erroneous packet is network coded and retransmitted just once, which can be named the least packet retransmission. When the redundancy factor is larger than 1, some of error packets are encoded and retransmitted more than once, thus α times of packets have been retransmitted compared with the least packets scheme.

When α is equal to 1, 35% ~ 48% of retransmission resource can be saved at the cost of worse receiver PER than the normal multicast retransmission scheme. To remain the same PER, more redundant network coded packets should be retransmitted. According to Figure 2(b), 1.5 times redundant network coded packets retransmission could keep the same PER level for the multicast group with more than 30 MSs; and correspondingly in Figure 2(a), with 1.5 times redundancy, multicast retransmission with network coding would save about 25% of retransmission resource.

(a) Retransmission packets ratio                     (b) receiver PER ratio

Figure 5 Simulation results

# Conclusion

In this proposal, three kinds of multicast retransmission schemes for E-MBS have been proposed. The first one is to retransmit on request. It is very easy to realize, with nearly the same complexity of that of unicast. However, the retransmission rate is very high for large multicast group. The second is retransmission threshold scheme. It is also easy to realize and the retransmission rate decreases. Nevertheless its receiving performance decreases. The third one is network coded retransmission scheme, which can save some of retransmission resource without sacrificing the receiving performance, certainly with higher complexity.

# Text proposal

# -----------------------------------------Start of Text Proposal for SDD------------------------------------------

## 15.x retransmission of E-MBS

Three retransmission schemes are optional.

### 15.x.1 retransmit on request

6 parallel stop-and-waits retransmission scheme is adopted. Maximum retransmission time is 3. MS requests packet to be retransmitted when the CRC checking failed. BS collects the requests during the transmission of other packets. When there’s request(s) for any packet, BS sets the retransmission request indicator bit (RIB) for this packet to 1. In each transmission slot, BS checks the RIB. When RIB of this slot is 1, the former packet in this slot is multicasted again. When the RIB is 0, new packet is read and transmitted in this slot.

### 15.x.2 retransmission threshold

When the requests on one packet is equal to or larger than the threshold α, set RIB of this packet into 1. Only the packets requested by α or more than α UEs can be retransmitted.

### 15.x.2 network coded retransmission

BS collects the retransmission requests. In each transmission slot, BS checks the retransmission buffer, if there are packets need to be retransmitted, chooses two packets randomly, XORs them and transmits this XORed packet. If there’s no packet need to be retransmitted, BS transmits one new packet in this slot. Whether one packet is network coded and how it is encoded is marked in the packet head. MSs can decode the network coded packets according to this information in the packet head. If the encoded packet is erroneous or it can not be decoded correctly, MS requests the corresponding packet once more.

---------------------------------------------- End of Text Proposal for SDD---------------------------------------------