| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Unicast transmission assisted HARQ for enhanced MBS |  |
| Date Submitted | 2008-9-5 |  |
| Source(s) | Xiaolong ZHU, Hongwei YANG, Yonggang WANG Alcatel Shanghai Bell Co., Ltd. | Voice: + 86-21-50554550; Fax:+ 86-21-50554554 mail to: Hongwei.Yang@alcatel-sbell.com.cn |
| Re: | IEEE 802.16m-08/033 Call for Contributions and Comments on Project 802.16m System Description Document (SDD) Contribution on topic: “PHY & MAC aspects of enhanced MBS” |  |
| Abstract | This contribution proposes to support unicast transmission assisted HARQ for enhanced MBS to extend the coverage and improve the spectrum efficiency of enhanced MBS. |  |
| Purpose | To be discussed and adopted by TGm for use in the IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Unicast transmission assisted HARQ for enhanced MBS

Xiaolong ZHU, Hongwei YANG, Yonggang Wang

Alcatel Shanghai Bell Co., Ltd.

# 1. Introduction

In the standard IEEE P802.16Rev2/D5, multicast and broadcast service (MBS) is specified as an efficient method for concurrent transport of data common to a group of users. MBS can be deployed in two scenarios: single-cell deployment and multi-cell deployment. The former contains only one BS while the latter multiple BSs in a certain MBS zone.

In the standard IEEE P802.16Rev2/D5, it is also stated that ARQ and HARQ are not applicable to MBS connections as there is no feedback from the BS at layer 1 or layer 2. MBS may be used with time-diversity similar to that used in HARQ transmissions where some HARQ parameters are used for MBS bursts to allow proper sequencing and time diversity combining when MBS bursts are repeatedly transmitted, but without any layer 1 or layer 2 acknowledgement from the MS.

Since a BS serves simultaneously multiple MSs that may experience totally different channel conditions, the modulation and coding scheme (MCS) for MBS bursts is usually determined in such a way that most of the users can be covered. As a result, some users cannot be served with MBS due to bad channel conditions. To extend the coverage of MBS, the BS can select to transmit repeatedly the MBS bursts regardless of the number of users that cannot recover the MBS bursts. Obviously, this type of retransmission shall reduce the spectrum efficiency of MBS because most users generally have already detected successfully the MBS packets.

In order to extend the coverage and improve the spectrum efficiency of enhanced MBS, this contribution proposes a unicast transmission assisted HARQ mechanism for enhanced MBS.

# 2. Unicast transmission assisted HARQ

Suppose BS1 serves a group of users including MS1 with enhanced MBS services and it also serves MS1 with unicast services, as shown in Figure 1. In the case of multi-cell deployment, this group of users also receives the MBS services from other BS(s) like BS2 in Figure 1. The MBS and unicast services can operate in different carriers or in time-division multiplexing (TDM) mode by sharing a common carrier.

Figure 1: Illustration of enhanced MBS deployment. For the single-cell MBS, the MBS connections from BS2, as shown by dotted red lines, do not exist. Besides the MBS, MS1 is also served with unicast services from its serving BS1. MBS and unicast links can operate in different carriers or share a common carrier.

As depicted in Figure 2, the key idea for unicast transmission assisted HARQ for enhanced MBS includes the following points.

- The user can receive not only unicast services from its serving BS but also enhanced MBS services from its serving BS and possible other BS(s).

- When the user, MS1, fails to recover one MBS packet, it makes a decision whether to report a NACK of this packet to its serving BS, BS1, on the unicast uplink. For example, if this MBS packet is of high priority, MS1 sends a NACK to BS1. For another example, if this MBS packet is not important or there is no free unicast uplink (fast-feedback channel) resource for NACK report, MS1 does not send the NACK message to BS1.

- After BS1 receives a NACK from its served user MS1, it makes a decision whether and how to retransmit the MBS packet. If many users report the NACK messages for the same MBS packet, BS1 can select to retransmit this MBS packet on the MBS link. Alternatively, if only a few users send the NACK messages, BS1 can reject the request for retransmission or schedule appropriate unicast downlink resource to perform the retransmission of the false MBS packets.

- If the BS1 decides to retransmit the enhanced MBS packet on the unicast downlink, it may adjust the MCS level of this packet or apply an enhanced unicast technique like beamforming and closed-loop MIMO (Note that such unicast techniques may be not available for MBS due to concurrent transport to a group of users). The serving BS1 can also request other BS(s) to perform jointly transmission or suppress interference to MS1 on the scheduled unicast downlink.

- MS1 receives the enhanced MBS packet and recovers it using e.g. the soft combining technique where both the originally received packet on the MBS link and the newly received packet on the unicast downlink are utilized.

- If MS1 still fails the recovery, it can give up this MBS packet or select to report another NACK to its serving BS and request another retransmission.

Figure 2: An example of unicast transmission assisted HARQ procedure for enhanced MBS. MS1 is capable to receive the MBS services and unicast service from BS1. In the case of multi-cell deployment, MS1 also receives the MBS packets from other BS(s). Enhanced MBS and unicast links can operate in different carriers or share a common carrier.

The proposed HARQ mechanism for enhanced MBS is expected to impact the PHY, MAC and upper layers. For example, the MS shall have the capability to report the NACK of its MBS packet on the unicast fast feedback channel. The BS shall have the capability to allocate a unicast connection for enhanced MBS packet and retransmit the MBS packet on the unicast downlink. Nevertheless, the MBS can be potentially improved greatly. If the user can report the channel quality for its enhanced MBS downlink on its unicast uplink, a proper MCS level can be determined for MBS and thereby achieving high spectrum efficiency (Note that if the enhanced MBS and unicast services share a common carrier and operate in TDM, then no additional channel quality report is necessary for enhanced MBS). Moreover, an optimized resource usage can be achieved by balanced scheduling between unicast and MBS services. On the one hand, if there is enough unicast resource, we can improve the spectrum efficiency of enhanced MBS by increasing its MCS level and extend the coverage of enhanced MBS through the unicast transmission assisted HARQ. Many advanced unicast techniques, such as beamforming, closed-loop MIMO and multi-cell MIMO, can also be used to further enhance the performance of enhanced MBS connections. On the other hand, if the unicast resource is limited, we can apply a conservative MCS level for enhanced MBS to cut down the usage of unicast channel for MBS retransmission.

3. Tex proposal

-----------------------------------------------Start of Text Proposal for SDD-------------------------------------------

Enhanced multicast and broadcast service (MBS) is an efficient method for concurrent transport of data common to a group of users. Enhanced MBS can be deployed in single-cell or multi-cell: the former contains only one BS while the latter multiple BSs in a certain MBS zone. The enhanced MBS and unicast services can operate in different carriers or in time-division multiplexing (TDM) mode by sharing a common carrier.

For a coexistence system of enhanced MBS and unicast services, the resource usage shall be optimized for two services. The MS served with both types of services shall have an optional function to report the NACK message of enhanced MBS packet on its unicast uplink. The BS providing both types of services shall have an optional function to retransmit the enhanced MBS packet on its unicast downlink after it receives a corresponding NACK from its served MS.

When performing retransmission of an enhanced MBS packet on the unicast downlink, the BS may adjust the MCS level of this MBS packet or apply an advanced unicast technique like beamforming and closed-loop MIMO. The serving BS can also request other BS(s) to perform jointly retransmission or suppress interference to this MS on the scheduled unicast downlink.

For the enhanced MBS packet with retransmission assisted by unicast downlink, the MS may apply the soft combining technique to recover it where both the originally received packet on the MBS connection and the newly received packet on the unicast downlink are utilized.

---------------------------------------------- End of Text Proposal for SDD---------------------------------------------