| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for SDD text on Synchronization Channel for IEEE 802.16m |  |
| Date Submitted | 2008-09-05 |  |
| Source(s) | Robert Novak, Mo-Han Fong, Sophie Vrzic, Jun Yuan, Dongsheng Yu, Hosein Nikopourdeilami, Kathiravetpillai Sivanesan Nortel Networks | rnovak@nortel.com mhfong@nortel.com |
| Re: | SDD Session 56 Cleanup, Call for PHY Details: In response to IEEE 802.16m-08/033 “Call for Contributions and Comments on Project 802.16m System Description Document (SDD)” for Session 57 |  |
| Abstract | This contribution proposes downlink SCH structure for P802.16m SDD. |  |
| Purpose | Propose to be discussed and adopted by TGm for the use in Project 802.16m SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposal for SDD text on Synchronization Channel for IEEE 802.16m

Robert Novak, Mo-Han Fong, Sophie Vrzic, Jun Yuan,  Dongsheng Yu, Hosein Nikopourdeilami, Kathiravetpillai Sivanesan

Nortel Networks

- Introduction

This contribution proposes SDD text for the downlink SCH structure of IEEE 802.16m.

- Text Proposal

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[Add the following into the TGm System Description Document]

#### Synchronization Channel (SCH)

The synchronization channel (SCH) is a DL physical channel which provides a reference signal for time, frequency, and frame synchronization, RSSI estimation, and BS identification. Using the SCH for channel estimation is FFS.

##### Synchronization channel architecture

###### Overview

            - Hierarchy

[Option 2. Two levels of synchronization hierarchy exist. These are called the primary synchronization channel (P-SCH) and secondary synchronization channel (S-SCH). The P-SCH is used for initial acquisition and carrying group cell/sector ID information. The S-SCH is used for fine synchronization, and cell/sector identification (ID).

            - Multiplexing

P-SCH and S-SCH are TDM.

            - Number of symbols in SCH

A complete instance of the SCH exists within a superframe. Multiple symbols within the superframe may comprise the SCH.

In mixed deployments, the presence of the legacy preamble is implicit.

The number of synchronization channel symbols in each superframe is FFS.

            - Location of synchronization symbols

In mixed deployments, the presence of the legacy preamble in the first symbol of the legacy frame is implicit. The location of the SCH symbol(s) is fixed within the superframe.

The location of SCH is FFS.

            - Properties of P-SCH & S-SCH

The P-SCH has these properties:

•	Common to a group of  sectors/cells

•	Optionally supports limited signaling (e.g. mode, bandwidth, etc)

•	Fixed bandwidth (5MHz )

•	Carries ID information (e.g. group ID, femto ID)

The S-SCH has these properties:

- Same bandwidth as P-SCH | Full bandwidth

- Carries ID information

###### Description of legacy support/reuse

16m system will exist in both greenfield and mixed (coexisting legacy and 16m equipment) deployments. In mixed deployments the legacy preamble will be always present. As discussed in the requirements, the 16m SCH is not to degrade the performance of legacy acquisition.

When the 16e preamble is not part of 802.16m SCH, the 802.16m SCH shall enable 802.16m MSs to synchronize in frequency and time without requiring the legacy preamble.

###### Cell ID support

The number of IDs is at least 512.

Sectors are distinguished by the synchronization channel.].

###### Multicarrier and multi-bandwidth support

The location of the SCH in frequency is FFS

###### MIMO support and channel estimation

SCH MIMO support and channel estimation if FFS.

##### Synchronization channel Sequence Design Properties

The SCH enables timing synchronization by autocorrelation.

The power of synchronization channel can be boosted

P-SCH  is mapping with every other subcarrier on the frequency domain.

The S-SCH  is mapping with every [FFS] subcarriers on the frequency domain.

-----------------------------------------------------------End of the Text---------------------------------------------------------