| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Flexible OFDM parameters for better performance |  |
| Date Submitted | 2008-09 -05 |  |
| Source(s) | Mariana Goldhamer Alvarion ltd. 21A, Ha Barzel Street Tel Aviv | E-mail: mariana.goldhamer@alvarion.com |
| Re: | SDD Session 56 Cleanup |  |
| Abstract |  |  |
| Purpose | Actions: 1. Modification of ToC 2. Capture of the text in the SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Flexible OFDM parameters for better performance

Mariana Goldhamer

Alvarion Ltd.

## OFDMA Parameters and sub-frames

The way in which the OFDMA parameters are defined within the 802.16m System Description Document is not optimal.

The OFDMA Parameters (11.3) are defined such to keep the numerology of the Rev. 2, which is not suitable to the 6 symbols-based sub-frames. We see “derivatives” using 7 symbols per sub-frame. These “odd” creations increase the system complexity reduce the performance, due to lack of support for Alamouti time-space coding scheme, using two consecutive symbols. With sub-frame size of  7 symbols, the result will be un-used symbols!

## Support for 7MHz channels

The existing numerology leaves 680us as “idle time”. Considering that 60us are used in other modes for Tx/Rx switching, we have here a wasted capacity of 12.4%. This obviously is against to the SRD. Introducing 7 data symbols/sub-frame is a not trivial complication of the design.

## Using different OFDM parameters

During each of the sub-frames may be used OFDM technologies with different PHY parameters. For example, the Legacy OFDMA uses a sampling rate such to produce 47 symbols per 5ms frame and 10MHz channels. Some time is leaved for two Tx/Rx switching points. 802.16m can use different sampling rates, which will allow the operation with sub-frames having always data transmitted in 6 symbols. If one symbol is used at the start of the DL frame for synchronization and one at the start of the UL frame for sounding, the optimal number of symbols per frame will be 50. We give in continuation an example of design for 50 symbols a frame and two Tx/Rx switching points. If the sounding symbol will not be used, the optimum number of symbols per frame will be 49.

# Proposed solution for the OFDMA numerology

Every 802.16m Frame or even every Zone may use a different OFDMA numerology, such that the spectrum will be optimally used.

The elements of the proposed solution are:

- Flexible numerology, which includes additional symbols per Frame; these symbols can be used for downlink synchronization and for up-link sounding.

- Define the numerology for the case of reduced Cyclic Prefix (1/16 of symbol duration)

- Keep the same occupied spectrum as in the Legacy solution

- Minimize the idle time to what is really needed

- Use implementable integers for clock synthesizer chips.

We will present in continuation the proposed numerology for 49 symbols in a 5ms Frame and for 50 symbols in a 5ms Frame, such to include two Tx/Rx switching points. Different solutions are proposed for CP=1/8 or 1/16 of the symbol duration. Additional switching points can be always obtained by dropping one symbol. More symbols can be created by increasing the sampling rate while keeping the same occupied spectrum.

We propose two different numerologies for the 7MHz band, based on the frequency where it is used. The symbol duration and idle time should be higher in 700MHz as compared with 3.5MHz. The 3.5MHz symbol duration can be shorter, as the propagation attenuation creates lower multi-path.

We will also propose the numerology for the 15MHz channel width, which is supported by LTE but it is not supported yet by 802.16.

## OFDMA numerology – 48 symbols per frame

This solution is detailed in the 802.16 Legacy standard and also in SDD.

Table 1 Existing OFDMA numerology

In case of 7MHz bandwidth, the idle time is actually 680us (the table entry of 104us is mistaken!). Four symbols remain un-used!

## OFDMA numerology – 50 symbols per frame, CP=1/8

To achieve, for channel BW of 5, 10 and 20MHz, the accommodation of 50 symbols per 5ms frame, with aprox. 60 us for switching time, the sampling frequency has to be increased by 50/48= 4%.

The proposed OFDMA numerology, providing two more symbols per 5ms frame and occupying the same spectrum as the legacy system, resolves the following:

- All the sub-frames have fixed duration: 6 symbols and in addition are provided two symbols for preambles (UL and DL)

- This scheme is suitable for bringing the MIMO operation, Reuse 1 and Reuse N operation at the start of the Frame; in this mode all the sub-frame symbols will be used in any MIMO mode and no symbols will be wasted.

Table 2 – Proposed OFDMA numerology 50 symbols, CP=1/8

| Nominal Channel Bandwidth (MHz) | 5 | 7 | 7 | 8.75 | 10 | 20 |
| --- | --- | --- | --- | --- | --- | --- |
| Band of operation |  | 700MHz | 3.5GHz |  |  |  |
| Over-sampling Factor | 1.167 | 1.111 | 1.167 | 1.175 | 1.167 | 1.167 |
|  | 7/6 | 4/3 | 7/6 | 47/40 | 7/6 | 7/6 |
| Sampling Frequency (MHz) | 5.833 | 7.778 | 8.167 | 10.281 | 11.667 | 23.333 |
| FFT Size | 512 | 1024 | 512 | 1024 | 1024 | 2048 |
| Sub-Carrier Spacing (kHz) | 11.393 | 7.595 | 15.951 | 10.040 | 11.393 | 11.393 |
| Useful symbol time Tu (μs) | 87.77 | 131.66 | 62.69 | 99.60 | 87.77 | 87.77 |
| Used carriers | 397 | 795 | 397 | 795 | 795 | 1591 |
| Number of sub-channels (Psc=18) | 22 | 44 | 22 | 44 | 44 | 88 |
| Legacy occupied spectrum (kHz) for PUSC | 4605 | 6570 | 6570 | 8213 | 9198 | 18386 |
| Occupied spectrum (kHz) | 4523 | 6038 | 6332 | 7982 | 9058 | 18127 |
| Cyclic prefix: Tg=1/8Tu | 0.125 | 0.125 | 0.125 | 0.125 | 0.125 | 0.125 |
| Symbol time Ts (us) | 98.743 | 148.114 | 70.531 | 112.049 | 98.743 | 98.743 |
| Number of symbols | 50 | 33 | 70 | 44 | 50 | 50 |
| Number of sub-frames (2 symbols are reserved in addition for preambles) | 8 | 5 | 11 | 7 | 8 | 8 |
| Idle time (μs) | 62.9 | 260.3 | 203.9 | 69.9 | 62.9 | 62.9 |

## OFDMA numerology – 50 symbols per frame, CP=1/16

From implementation considerations, in the case of 5,10 and 20MHz, we have preferred to reduce the oversampling factor. We gain capacity by increasing the number of sub-channels.

It is also possible to reduce the symbol duration such to increment the sub-frame number.

Table 3 – Proposed OFDMA numerology 50 symbols, CP=1/8

| Nominal Channel Bandwidth (MHz) | 5 | 7 | 7 | 8.75 | 10 | 20 |
| --- | --- | --- | --- | --- | --- | --- |
| Band of operation |  | 700MHz | 3.5GHz |  |  |  |
| Over-sampling Factor | 1.120 | 1.222 | 1.167 | 1.120 | 1.120 | 1.120 |
|  | 28/25 | 11/9 | 7/6 | 28/25 | 28/25 | 28/25 |
| Sampling Frequency (MHz) | 5.600 | 8.556 | 8.167 | 9.800 | 11.200 | 22.400 |
| FFT Size | 512 | 1024 | 512 | 1024 | 1024 | 2048 |
| Sub-Carrier Spacing (kHz) | 10.938 | 8.355 | 15.951 | 9.570 | 10.938 | 10.938 |
| Useful symbol time Tu (μs) | 91.43 | 119.69 | 62.69 | 104.49 | 91.43 | 91.43 |
| Used carriers | 413 | 795 | 397 | 849 | 827 | 1663 |
| Number of sub-channels (Psc=18) | 23 | 44 | 22 | 47 | 46 | 92 |
| Legacy occupied spectrum (kHz) for PUSC | 4605 | 6570 | 6570 | 8213 | 9198 | 18386 |
| Occupied spectrum (kHz) | 4517 | 6642 | 6332 | 8125 | 9045 | 18189 |
| Cyclic prefix: Tg=1/8Tu | 0.063 | 0.063 | 0.063 | 0.063 | 0.063 | 0.063 |
| Symbol time Ts (us) | 97.143 | 127.169 | 66.612 | 111.020 | 97.143 | 97.143 |
| Number of symbols | 51 | 39 | 75 | 45 | 51 | 51 |
| Number of sub-frames (2 symbols are reserved in addition for preambles) | 8 | 6 | 12 | 7 | 8 | 8 |
| Idle time (μs) | 142.9 | 167.6 | 70.7 | 115.1 | 142.9 | 142.9 |

## OFDMA numerology – 49 symbols per frame, CP=1/8

Below is the proposed numerology for 49 symbols per frame and CP=1/8 of the symbol duration.

Table 4 – Proposed OFDMA numerology 49 symbols, CP=1/8

| Nominal Channel Bandwidth (MHz) | 5 | 7 | 7 | 8.75 | 10 | 20 |
| --- | --- | --- | --- | --- | --- | --- |
| Band of operation |  | 700MHz | 3.5GHz |  |  |  |
| Over-sampling Factor | 1.143 | 1.250 | 1.125 | 1.143 | 1.143 | 1.143 |
|  | 8/7 | 5/4 | 9/8 | 8/7 | 8/7 | 8/7 |
| Sampling Frequency (MHz) | 5.714 | 8.750 | 7.875 | 10.000 | 11.429 | 22.857 |
| FFT Size | 512 | 1024 | 512 | 1024 | 1024 | 2048 |
| Sub-Carrier Spacing (kHz) | 11.161 | 8.545 | 15.381 | 9.766 | 11.161 | 11.161 |
| Useful symbol time Tu (μs) | 89.60 | 117.03 | 65.02 | 102.40 | 89.60 | 89.60 |
| Used carriers | 415 | 759 | 433 | 813 | 813 | 1609 |
| Number of sub-channels (Psc=18) | 23 | 42 | 24 | 45 | 45 | 89 |
| Legacy occupied spectrum (kHz) for PUSC | 4605 | 6570 | 6570 | 8213 | 9198 | 18386 |
| Occupied spectrum (kHz) | 4632 | 6486 | 6660 | 7939 | 9074 | 17958 |
| Cyclic prefix: Tg=1/8Tu | 0.125 | 0.125 | 0.125 | 0.125 | 0.125 | 0.125 |
| Symbol time Ts (us) | 100.800 | 131.657 | 73.143 | 115.200 | 100.800 | 100.800 |
| Number of symbols | 49 | 37 | 68 | 43 | 49 | 49 |
| Number of sub-frames (2 symbols are reserved in addition for preambles) | 8 | 6 | 11 | 7 | 8 | 8 |
| Idle time (μs) | 60.8 | 128.7 | 99.4 | 46.4 | 60.8 | 60.8 |

## OFDMA numerology – 49 symbols per frame, CP=1/16

The optimum solution is to change the sampling rate such to add one more sub-frame.

Table 5 – Proposed OFDMA numerology 49 symbols, CP=1/16

| Nominal Channel Bandwidth (MHz) | 5 | 7 | 7 | 8.75 | 10 | 20 |
| --- | --- | --- | --- | --- | --- | --- |
| Band of operation |  | 700MHz | 3.5GHz |  |  |  |
| Over-sampling Factor | 1.083 | 1.200 | 1.167 | 1.083 | 1.083 | 1.083 |
|  | 13/12 | 6/5 | 7/6 | 13/12 | 13/12 | 13/12 |
| Sampling Frequency (MHz) | 5.417 | 8.400 | 8.167 | 9.479 | 10.833 | 21.667 |
| FFT Size | 512 | 1024 | 512 | 1024 | 1024 | 2048 |
| Sub-Carrier Spacing (kHz) | 10.579 | 8.203 | 15.951 | 9.257 | 10.579 | 10.579 |
| Useful symbol time Tu (μs) | 94.52 | 121.90 | 62.69 | 108.03 | 94.52 | 94.52 |
| Used carriers | 433 | 795 | 415 | 867 | 867 | 1735 |
| Number of sub-channels (Psc=18) | 24 | 44 | 23 | 48 | 48 | 96 |
| Legacy occupied spectrum (kHz) for PUSC | 4605 | 6570 | 6570 | 8213 | 9198 | 18386 |
| Occupied spectrum (kHz) | 4581 | 6521 | 6619 | 8026 | 9172 | 18355 |
| Cyclic prefix: Tg=1/8Tu | 0.063 | 0.063 | 0.063 | 0.063 | 0.063 | 0.063 |
| Symbol time Ts (us) | 100.431 | 129.524 | 66.612 | 114.778 | 100.431 | 100.431 |
| Number of symbols | 49 | 38 | 75 | 43 | 49 | 49 |
| Number of sub-frames (2 symbols are reserved in addition for preambles) | 8 | 6 | 12 | 7 | 8 | 8 |
| Idle time (μs) | 78.9 | 207.6 | 137.3 | 64.5 | 78.9 | 78.9 |

## OFDMA numerology – 15MHz channels

One of the 802.16 disadvantages is that the 15MHz bandwidth, which equals the allocation size in many operator deployments, is not covered.

The 20MHz channels can be scaled to 15MHz by the following procedures:

- Keeping the same FFT size (2k) and sampling rate as used for the 20MHz channel width;

- Scaling the used number of carriers by reducing it, such to use only 75% of the carriers used in the case of the 20MHz channel width.

# Conclusions

We have created the condition for unique 6-symbols based sub-frames and have derived the numerology for the support of  the 15MHz channels, without defining a 1.5k FFT, as was done in LTE.

# SDD text changes