| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Procedures for Multicarrier Support |  |
| Date Submitted | 2008-09-09 |  |
| Source(s) | Youngsoo Yuk, Inuk Jung, Ronny Yongho Kim and Kiseon Ryu LG Electronic Inc. | Voice: +82-31-450-7808 E-mail: sixs@lge.com |
| Re: | MAC: Multi-Carrier Operation; in response to the TGm Call for Contributions and Comments 802.16m-08/033 for Session 57 |  |
| Abstract | This contribution covers the considerations about the control structure for supporting the multicarrier system. |  |
| Purpose | To be discussed and adopted by TGm for use in the IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Procedures for Multicarrier Support

Youngsoo Yuk, Inuk Jung, Ronny Yong-ho Kim and Kiseon Ryu

LG Electronics

# Introduction

In this contribution, we present the basic operation and procedures related to multicarrier support.

# Capability Negotiation for Multicarrier Support

The allocation of multiple carriers may be performed by BS through the capability negotiation process at network entry. During this procedure, various parameters such as BS capability, MS capability, BS load information, MS power management may be utilized to determine the allocation of multiple carriers. Table 1 shows the examples of the capability parameters

Table 1 Capability Parameters

|  | Parameters for Multicarrier support |
| --- | --- |
| BS capability | - Number of Multiple RF - Multicarrier Configuration information - Maximum bandwidth - … |
| MS capability | - MS type for Multicarrier support - Maximum number of RF transceiver - Maximum supportable bandwidth - … |

The basic operations of capability negotiation are as follows:

- The MS acquires the capability of BS for multicarrier support via SBCH of the corresponding BS.

- The carrier that MS tries to access is defined as the Temporary primary carrier.

- After ranging process, MS transmits its capability for multicarrier support to the BS via the SBC-REQ message.

- BS transmits to the MS the information about the carrier allocation (e.g. primary/secondary carrier allocation information) on the SBC-RSP message.

  - Even the temporary primary carrier can be directly assigned as the primary carrier, another carrier can be assigned as a primary carrier according to the status of BS (e.g. traffic load,…).

  - In case of that the temporary primary carrier is different from the allocated primary carrier, the temporary primary carrier can transmit the different part of the system information of the primary carrier to MS for fast initial access.

  - BS assigns a primary carrier and several available secondary carriers. The available secondary carriers are the maximum set of the secondary carriers which can be used. The active carrier is the real secondary carrier to be used for data transmission.

Figure 1 shows the above processes.

Figure 1 Network Entry procedure for Multicarrier Support

# Carrier Management Procedure

Even the primary carrier and the secondary carriers are assigned to MS at the network entry, the assigned secondary carriers are not always used. As mentioned above, those are the available set of secondary carriers, and a part of available secondary carriers is really used for data transmission. These secondary carriers are defined as active secondary carriers.

The set of active secondary carriers is determined according to maximum aggregated data rate with current services or sessions. Figure 2 shows the example of this concept. Though 5 carriers (including primary carrier) are assigned to a MS, the number of active carriers vary according to the requirement of data rate for current services.

We define the secondary carrier idle state in [1], and all the physical/logical connection of the secondary carriers in this state is disconnected. BS may transmit the control signal to MS to add/release a link of a secondary carrier.

For more efficient carrier management, the order of activation of secondary carriers can be considered.

In this case, each secondary carrier has its activation order and with request of higher data rate secondary carriers are turned on with sequential order. Even we can expect the diversity and flexible scheduling gain without the allocation order, large diversity gain is not expected with multiple RF carriers.

Figure 2 Available and active secondary carriers

# Neighbor Advertisement Procedure

In current IEEE802.16e/REV2, the information about the neighbor cells are transmitted via MOB_NBR-ADV message. MOB_NBR-ADV contains the list of neighbor cells and their system information.

Current MOB_NBR-ADV is designed for single-carrier support only. If we includes all information related to multicarrier support, it may be a big burden to a single carrier MS.

We propose that to define another message specified to multicarrier support. The message contains the system information not included in MOB_NBR-ADV. Since the information may be common to MSs having different primary carriers, it is not necessary to send it via all fully configured carriers. Thus it can be transmitted through any selected or predetermined carriers or we can apply the method of carrier hopping to send the message for reducing overhead.

Figure 3. The example of transmission of Neighbor advertisement for multicarrier support

# References

- IEEE C802.16m-08/994r1, Multi-Carrier Power Management, 2008-09-09

Text Proposal for the IEEE802.16m SDD

============================== Start of Proposed Text =================================

19.x MAC procedures for Multicarrier Support

19.x.1 Network Entry Procedure for Multicarrier Support

The MS acquires the capability of BS (e.g. multicarrier configuration information, …) for multicarrier support by decoding the SBCH of the corresponding BS.

The carrier, which the MS tries to access, is defined as the Temporary primary carrier.

After ranging process, MS transmits its capability for multicarrier support (e.g. MS type, maximum number of RF transceiver, maximum supportable BW,…) to the BS on the SBC-REQ message.

BS transmits carrier allocation information (e.g. primary/secondary carrier allocation information) to the MS through the SBC-RSP message.

Even if the temporary primary carrier can be directly assigned as the primary carrier, another carrier can be assigned as a primary carrier according to the status of BS (e.g. traffic load,…).

In case of when the temporary primary carrier differs from the allocated primary carrier, the temporary primary carrier can transmit the different part of the system information of the primary carrier to MS for fast initial access. Figure 19.x explains the operation of the initial access.

Figure 19.x. Network Entry procedure for Multicarrier Support

19.x.2 Carrier Management Procedure

A primary carrier and several available secondary carriers are assigned to a MS during network entry procedure.

The available secondary carriers are activated by BS’s request according to the status of current active services.

The activated secondary carrier is defined as active secondary carrier. Each available secondary carrier has its activation order.

# 19.x.y Multicarrier Handover Procedure

# 19.x.y.z Neighbor Advertisement Procedure

The system information on the neighbor cells are broadcast to whole cell through MOB_NBR-ADV message. To support multicarrier operation, additional MAC message is broadcast to the cell for transmitting all information related to multicarrier support not included in MOB_NBR-ADV.

Since the information may be common to MSs having different primary carriers, it is not necessary to send it via all fully configured carriers. Thus it can be transmitted through any selected or predetermined carriers or we can apply the method of carrier hopping to send the message for reducing overhead.

============================== End of Proposed Text =================================