| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Data Encryption & Authentication functionality Location for IEEE 802.16m |  |
| Date Submitted | 2008-09-05 |  |
| Source(s) | Kiran Thakare Ericsson AB SE-164 80 Stockholm, Sweden | Voice: +46 8 58532591 E-mail: kiran.thakare@ericsson.com |
| Re: | IEEE 802.16m-08/033 Call for Contributions on Project 802.16m System Description Document. For “Security: MAC” |  |
| Abstract | Specification [1] requires that the IEEE802.16m should be optimized for system performance. This contribution proposes the CS location for Data Encryption & authentication functionality to improve the overall system performance. This proposal considers the future requirements on system evolution and still optimizes the air resource use in robust manner for IEEE 802.16m. |  |
| Purpose | We propose to include the proposed text in the contributions in the security section of the SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Data Encryption & Authentication functionality Location for IEEE 802.16m

Kiran Thakare

Ericsson AB Sweden

# Introduction

Specification [1] requires that the IEEE802.16m should be optimized for system performance.  This contribution proposes the CS location for Data Encryption & authentication functionality to improve the overall system performance. This proposal considers the future requirements on system evolution and still optimizes the air resource use in robust manner for IEEE 802.16m.

# Background & Rational

In [2] the MAC CS layer specifies three service specific convergence sub-layers (CS) such as ATM CS, Packet CS and generic packet CS.   In [2] the MAC Security sub-layer is responsible for encryption of PDUs, data authenticity and privacy key management between the BS and MS.  In [3] the security management functionality (Key management etc) has been made part of RRCM layer.  However, the encryption & authentication functionalities remain at the MAC CPS layer.  This location of the DE & authentication functionality at MAC-CPS is an implausible position for the following reason.

- Following functionality division may result in a significant interlayer communication, inter-layer –dependency resulting in delays, data interruption and inconsistent states/ information when setting up a connection or when transferring (HO) a connection.

  - CS layer classifies SDUs coming from higher layer as per the QoS requirements, associates them with SAID and CID

  - RRCM has security management functionalities – Key management, connection management.

  - MAC-CPS has Data encryption and authentication at MAC-CPS layer

- HO performance may be deteriorated by the DE & authentication functionalities in MAC-CPS layer –as all the three layers (MAC-CS, RRCM, CPS) would be involved in transferring the user related context (for example PN number) to the target BS.

- MAC-CPS segments CS SDUs into ‘n’ MAC PDU and then applies the data encryption & authentication to these PDUs.  Thus ‘n’ time increase in the overhead.  This is illustrated in figure 2.

- Additionally encryption & authentication functionality at MAC-CPS layer is inadequate for the reason of future traffic growth & mobility aspect of the service flows.  .

MAC-CPS

Fragmentation

MAC-CS SDU

MAC-CPS SS

Encryption & Authentication

MAC header

Payload

CRC

CS-SDU1

SDU11

SDU12

SDU13

PDU1

CS-SDU2

SDU2

PDU12

PDU3

PDU4

PN1

PDU1

ICV

PN4

PDU4

ICV

MAC header

Payload

CRC

Append MAC header & CRC

Figure 2 showing the CS SDU processed by MAC-CPS layer & MAC CPS SS layer.  SDU1 is segmented in 3 parts thus requiring 3 PN numbers & header, thus increase in overhead is proportional to 憂�number of fragmentation. For n = 3 this is extra 12bytes overhead

Accordingly, we believe that the data encryption and data authentication function should be moved to CS layer.

# Proposed DE & Authentication functionality location for 802.16m

We propose to move the security functionalities, in particular, the data encryption and authentication to the CS layer.  We propose that the CS layer generates the PN (packet number) also.  Figure 3 shows the processing of CS SDU at CS SS layer.

Applying the data encryption & authentication to the CS-PDU at CS layer offers following various benefits as follows:

- HO Performance (delays and inconsistencies)  improvement

- Efficient use of air resources, as DE is applied to CS SDU rather than fragmented MAC PDUs

- Less processing overhead

- Considers future traffic growth

- Less inter-protocol layer communication

.

MAC-CS SS

Encryption & Authentication

MAC-CS SDU

MAC-CPS

Fragmentation

MAC header

Payload

CRC

SDU11

SDU12

SDU13

PDU1

CS-SDU2

SDU2

PDU12

PDU3

PDU4

MAC header

Payload

CRC

Append MAC header & CRC

Figure 3 showing the CS SDU encrypted and authenticated by SS layer at CS layer.

CS-SDUn

CS-SDU1

PN1

SDU1

ICV

PN2

SDU2

ICV

MAC-SDU2

MAC-SDU1

# Proposal

We propose following text to be included in the Security Section of the SDD

“Data encryption and data authentication functionality is located at the MAC-CS layer.  CS layer allocates & maintains the PN number when associating the security association (SA) to the concerned radio connection.”

# References

- IEEE 802.16m System Requirements, IEEE 802.16m-07/002r4

- IEEE Std. 802.16, IEEE Draft Standard P802.16Rev2/D6

- IEEE 802.16m System Description Document, IEEE 802.16m-08/003r4

- IEEE C802.16m-08/201r2