| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ARQ Enhancement for IEEE 802.16m |  |
| Date Submitted | 2008-09-05 |  |
| Source(s) | Bong Ho Kim Hyung Joon Jeon POSDATA | bhkim@posdata-usa.com hjjun@posdata.co.kr |
| Re: | Call for Contributions on Project 802.16m System Description Document (SDD) IEEE 802.16m UMAC Data Plane (ARQ) |  |
| Abstract |  |  |
| Purpose | To be discussed and approved for use in IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

ARQ Enhancement

Bong Ho Kim and Hyung Joon Jeon

POSDATA

# Introduction

ARQ (Automatic Repeat Request) mechanism is to enhance the system performance under low signal strength environment by retransmitting the unsuccessfully received packets. IEEE 802.16e also includes ARQ mechanism, but it may not maximize the performance improvement because of the following restrictions on MAC protocol for ARQ.

- CRC check is done per MAC PDU not per ARQ Block

- ARQ Block size (or transmission unit) cannot be changed adaptively depending on the radio resource and/or channel condition

- ARQ feedback cycle and message size is not predictable. So it may take long delay to transmit the ARQ feedback via uplink (BWR CDMA code transmission, CDMA Allocation IE, BR Header, UL Grant and ARQ feedback).

This proposal is to suggest modification to the IEEE802.16 MAC protocol specification for enhancing effectiveness of ARQ.

## Current IEEE802.16e-2005 Specification

Figure 1 shows the MAC PDU which has (α + β) ARQ blocks in it. As is shown in Figure 1, only one 32 bit CRC (Cyclic Redundancy Check) is appended at the tail of MAC PDU to check the whole (α + β) ARQ blocks.

(a) Packing Subheader

(b) Fragmentation Subheader

Figure 1 MAC PDU with (α + β) ARQ blocks and one CRC

Despite the ARQ feedback in IEEE802.16 specification is sent block by block, actually there’s no way to differentiate the successfully received block from the erroneous block in a MAC PDU when the calculated CRC value of MAC PDU mismatches with the appended CRC value.

Figure 2 shows an example what happens when there’s an erroneous block in MAC PDU which has only one CRC at its tail to verify if all the ARQ blocks in it successfully received. As shown in Figure 2, though only one ARQ block (α + 1) has error and should be discarded at the receiver, all successfully received ARQ blocks are discarded altogether because there’s no way to check block by block.

Figure 2 MAC PDU with erroneous ARQ block and one CRC

## Proposed Text for SDD

Insert the following text into Medium Access Control sub-layer sub-clause (i.e. Chapter 10 in [1]):

-------------------------------  Text Start  ---------------------------------------------------

10.x ARQ

“To enhance the ARQ performance, a mechanism reducing the number of retransmissions of the successfully received ARQ blocks shall be provided.”

-------------------------------  Text End  ---------------------------------------------------

## References

[1] Project 802.16m System Description Document (SDD)