| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Third draft: Harmonized SDD Text on Sleep mode |  |
| Date Submitted | 2008-08-29 |  |
| Source(s) | Mamadou Kone Ad Hoc Group Chair for Sleep Mode | itri404759@itri.org.tw makone@gmail.com |
| Re: | UMAC Rapporteur Group Discussions |  |
| Abstract | A third draft for Harmonized SDD text on Sleep Mode based on members’ uploaded contributions. |  |
| Purpose | For review and discussion in the Project 802.16m |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Third draft: Harmonized SDD Text on Sleep mode

Mamadou Kone

Ad Hoc Group Chair

# Introduction

This document contains the third draft of SDD text related to Sleep mode section. This document is used as a starting point to further developing of consensus SDD text related to Sleep mode. The proposed draft is based on current version of Draft IEEE 802.16m System Description Document [1].

There are still some open issues/sections, this contribution provided essentially text where we reached some consensus. In addition, this is not the final SDD text for sleep mode. The text of this draft shown as consensus text is determined by AHG chair as “reasonably consensus” and can be discussed and modified further.

Text Proposal

---------------------------------------------------------Start of the text--------------------------------------------------

10.x Power Management

IEEE 802.16m provides MS power management functions including sleep mode and idle mode to alleviate MS battery consumption.

MS may also opportunistically use knowledge of smaller time periods, such as subframe, during which the MS has no DL/UL transmission for it to perform a lesser degree of power savings referred to as micro-sleep. This does not involve any explicit signaling or negotiation with the BS.

10.x.1 Sleep Mode

10.x.1.1 Introduction

Sleep mode is a state in which an MS conducts pre-negotiated periods of absence from the serving BS air interface. Per MS, a single power saving class is managed in order to handle every active connections of MS. Sleep mode may be activated when an MS is in the connected state. When Sleep Mode is active, the MS is provided with a series of alternate listening window and sleep windows. During a sleep window, MS in the PSM is powered down. The listening window is the time in which MS is available to receive and send signaling and/or data from and to the BS.

The 802.16m provides a framework for dynamically adjusting the duration of sleep windows and listening windows based on changing traffic patterns and HARQ operations. The application of adjustable sleep windows and listening windows is not limited to data transmission. It can be applied to MAC control signaling procedures including scanning procedure as well. MS can send and receive transport data and MAC management messages without deactivating the sleep mode.

10.x.1.2 Sleep mode entry

Sleep mode activation/entry is initiated either by a MS or a BS. When MS is in Active mode, sleep parameters are negotiated between MS and BS. BS makes the final decision and instructs the MS to enter sleep mode. MAC management messages and/or MAC header/subheader can be used for sleep mode request/response signaling.

10.x.1.3 Sleep Mode Operations

10.x.1.3.1 Sleep cycle operation

The sleep cycle is comprised of the alternation between sleep and listening windows. A sleep cycle is the sum of a sleep window and a listening window. Sleep mode parameters are changed through explicit signaling.

BS keeps synchronized with MS on the sleep/listening windows’ boundary. The synchronization could be done either implicitly by following pre-determined procedure, or explicitly by using proper Acknowledgement mechanism.

10.x.1.3.2 Sleep Window Operation

During the sleep window, the MS is unavailable to receive any DL data and unicast MAC management message from the serving BS.  802.16m provides a framework for dynamically adjusting the duration of the sleep windows. If MS has data or management messages to transmit to BS during the sleep window, MS can interrupt the sleep window and request bandwidth for UL transmission with or without deactivating sleep mode based on sleep mode configuration.

Sleep mode configuration method is FFS.

10.x.1.3.3 Listening window operation

During the listening window MS can receive DL data and unicast MAC management message from BS. MS can also send data if any uplink data is scheduled for transmission. After termination (by explicit signaling or implicit method) of a listening window, the MS goes back to sleep for the remainder of the current sleep cycle. During the listening window, MS or BS may request change of sleep cycle via MAC management message.

[Traffic Indication Message]

During the MS listening window, BS may transmit the traffic indication message intended for one or multiple MSs. It indicates whether or not there is traffic addressed to one or multiple MSs. The traffic indication message transmission is carried out in two steps: an indicator followed by a traffic indication message. Traffic indication message is transmitted when the indicator is positive. The indicator and the traffic indication message are transmitted at pre-defined location. Upon receiving negative traffic indication in the traffic indication message, the MS can go back to sleep for the rest of the listening window.

[Listening Window Extension]

The listening window duration can be dynamically adjusted based on traffic availability in MS or BS.[The listening window can be also adjusted for accomplishing control signaling procedures efficiently]. The listening window can be extended through explicit signaling or implicit method. The maximum length of the extension is to the end of the current sleep cycle in which case there is no sleep window during the current sleep cycle.

10.x.1.4 Sleep Mode Exit

Sleep mode termination/deactivation is initiated either by MS or BS. BS makes the final decision and instructs the MS to de-activate sleep mode by using explicit signaling. MAC management messages and/or MAC header/subheader are used for sleep mode request/response signaling.

----------------------------------------------------------End of the text----------------------------------------------------

10.x.2 Idle Mode [Note]: Not included in our topic, only for your reference]

Reference

- IEEE C802.16m-08/003r4, “Project 802.16m System Description Document (SDD)”.