| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Performance of DL open loop single-user MIMO schemes |  |
| Date Submitted | Sep 02, 2008 |  |
| Source(s) | Senjie Zhang Guangjie Li Hongming Zheng Shanshan Zheng Feng Zhou Yang-Seok Choi Minnie Ho Intel Corp. | senjie.zhang@intel.com |
| Re: | PHY: MIMO; in response to the TGm Call for Contributions and Comments 802.16m-08/003r4 for Session 57 |  |
| Abstract | In this contribution, we will present a comparative performance evaluation on the above schemes for down selection. Based on the evaluation results, the “SFBC/SM+RBF” solution meets the above requirements best and it is proposed as the 802.16m OL-SU-MIMO solution. The proposed SDD text is attached. |  |
| Purpose | Discussion and Approved in 16m |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Performance of open loop single-user MIMO scheme candidates

Senjie Zhang, Guangjie Li, Hongming Zheng, Shanshan Zheng, Feng Zhou, Yang-Seok Choi and Minnie Ho

Intel Corporation

# Introduction and Background

Open loop single-user MIMO (OL-SU-MIMO) is an essential technique for 16m downlink transmission, especially for high speed and feedback-limited scenarios.

The OL-SU-MIMO schemes candidates include:

- STBC, SM, and D-STTD with antenna hopping are legacy OL-SU-MIMO schemes from 802.16e [1] [2].

- SFBC is another candidate for 2Tx rate-1 scheme in 802.16m.

- RBF (random beamforming, another name codeword cycling) is supported in current 802.16m SDD text, where a general OL-SU-MIMO formula with support to RBF is proposed [3].

- A hybrid STC/SM scheme for 4Tx rate-3 is proposed in [4].

To make 802.16m concise and effective, some down selection is necessary. In this contribution, we will present a comparative performance evaluation on the above schemes for down selection.

Some desirable properties of the OL-SU-MIMO are:

- Performance

- Low complexity

- Robustness

- Flexibility

Based on the evaluation results, the “SFBC/SM+RBF” solution meets the above requirements best. We propose “SFBC/SM+RBF” as the 802.16m OL-SU-MIMO solution. Our rationale and more technical details are explained in the following text.

# Description of OL-SU-MIMO schemes

1. For 2Tx rate1 scheme, STBC is used in legacy 802.16e. For 2Tx rate 2, SM is used in legacy 802.16e. For 4Tx rate 1 and 2 schemes, STC/AH and D-STTD/AH are used. The antenna hopping is implemented by using matrix A1~A3 or B1~B6 periodically.

2. In current SDD text, the OL-SU-MIMO is defined as follows:

For OL-SU-MIMO, on a given frequency resource k, the precoding matrix P can be defined using the following equation: P(k) = D(k)W(k). The precoder is composed of two matrices. The first matrix W(k) is an NT  M matrix, where NT is the number of transmit antennas and M is the numbers of streams. The matrix W(k) is selected from a predefined unitary codebook, and changes every u subcarriers. A codebook is a unitary codebook if each of its matrices consists of columns of a unitary matrix. The second matrix D(k) is an NT  NT diagonal matrix as follows,

D(k)=ejθ0k0⋯00ejθ1k⋯0⋮⋮⋱⋮00⋯ejθ(NT−1)k,

where k denotes frequency resource index and θi,i= 0, 1, 2, ..., NT-1 denotes the phase shift for the i-th transmit antenna across two adjacent frequency resources. Currently, θ0 = 0, θ1 = -2πD/Nfft, θ2 =2θ1, θ3 =3θ1, with D should be zero or small number (e.g. 2) where Nfft is FFT size.

The size of u should be one or multiple of 18 and the size of k should be 1.

In RBF scheme, the W(k) is chosen based on CL-SU-MIMO codebook. Using 16e codebook [5], the V(4,2,6) codebook can be used to precode the SFBC and SM for 4Tx rate 1 and 2. For 4Tx rate 3, the V(4,3,6) codebook can be used.

In RBF, precoded demodulation pilot (dedicated pilot) should be used.

3. The hybrid STC/SM scheme for 4Tx rate-3 can be described using the following matrix:

z=s1−s2s3s5s2s1s4s6s7s9s11−s12s8s10s12s11.

A summary of the OL-SU-MIMO scheme candidates is listed in Table-1.

| Rate | 1 |  |  | 2 |  |  | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Ant Cfg | 2Tx | 4Tx |  | 2Tx | 4Tx |  | 4Tx |
| Pilot Pattern | two streams | two streams | four streams | two streams | two streams | four streams | four streams |
| Scheme | STBC |  | 16e Matrix A |  |  | 16e Matrix B | Hybrid STC/SM |
|  | SFBC | RBF-SFBC |  | SM | RBF-SM |  | RBF-SM |

Table-1 OL-SU-MIMO scheme summary

# Performance evaluation

1. Assumptions

- 802.16m pilot pattern[6]

- Subcarrier level DRU

For subcarrier level DRU, the data subcarriers are divided into 384 pairs. Each DRU has 8 pairs, which are uniformly distributed in the whole band.

Each user has 4 RUs.

- eITU-VehA 30Km/h and 350Km/h

- Antenna spacing: 4 lambda (uncorrelated channel)

- AoD: [-90:10:90]

This AoD range is used to evaluate the coverage in different direction.

- CTC ½ coding rate，QPSK, 16QAM, 64QAM

- MIMO receiver: MMSE or MRC

- RU level CE

RU level CE is more practical concerning FFR partition.

2. Results

- Comparison of different schemes for 4Tx rate-1, 2 and 3.

Figure-1   Rate-1 schemes: RBF has better performance than STC/AH

Figure-2   Rate-2 Schemes: RBF has same performance with D-STTD/AH

Figure-3   Rate-3 Schemes: RBF has same performance with hybrid STC/SM

Figure 1, 2 and 3 show RBF has better performance than STC/AH and, and same performance with D-STTD/AH and hybrid STC/SM.

However RBF is with low complexity.

Concerning receiver complexity, RBF requires 2x2 matrix inversions for rate-2 and 3x3 matrix inversions for rate-3 in MMSE receiver; while D-STTD/AH and hybrid STC/SM require 4x4 and 6x6 matrix inversion. The MMSE MIMO receiver’s complexity is o(M3) using the well-known square-root algorithm [7]. So the MMSE receiver’s complexity of D-STTD/AH and hybrid STC/SM is 8 times higher than RBF. Another advantage of RBF is that the receiver designed for 2x2 antenna configuration can be used transparently in 4x2 antenna configuration.

Concerning pilot overhead, RBF requires 2 pilot streams for rate-1 and rate-2, while STC/AH and D-STTD/AH require 4 pilot streams, which leads to higher pilot overhead and 2 times higher complexity in channel estimator.

In summary, RBF has the best performance-complexity tradeoff.

- Comparison between STBC and SFBC

Figure-4   STBC vs. SFBC: MRC receiver’s performance in very high speed scenario

Figure-4 show the SFBC is more robust than STBC if using MRC receiver in very high vehicular speed.

Concerning mapping to regular sub-frame and irregular sub-frame, SFBC is preferred because it does not require even number of symbols in a sub-frame.

- Comparison between zero delay CDD and small delay CDD

Figure-5   D=0 vs. D=2: Comparison between zero delay CDD and small delay CDD

Figure-5 shows that identical matrix is preferred for matrix D in the general OL-SU-MIMO formula in SDD.

Using distributed sub-channelization, the channel changes fast in a DRU. The channel ripple provided by D(k) has little benefit to the performance, however it degrades the CE. So zero delay CDD (D=0) is preferred.

# Conclusion

We propose “SFBC/SM+RBF” as the 802.16m OL-SU-MIMO solution, because:

- SFBC provides full diversity order, also it is more flexible and robust

- RBF provides good performance

- RBF is with low complexity in terms of pilot overhead and receiver

The technical details are:

- For 2Tx rate 1, SFBC

- For 2Tx rate 2, SM

- For 4Tx rate 1, RBF + SFBC using dedicated pilot

- For 4Tx rate 2, RBF + SM using dedicated pilot

- For 4Tx rate 3, RBF

- For 4Tx rate 4, SM

- The second matrix D should be a identical matrix

Please refer to tgmsdd_Li_Guangjie.cmtb for the comments to current SDD (802.16m-08/003r4). The proposed remedy for SDD text is listed below.

# Proposed SDD text remedy

-------------------------------------Begin of proposed remedy-------------------------------------

In page 67 line 1~2, replace table 4 with the following table:

| MIMO Scheme | Resource Mapping |
| --- | --- |
| Open Loop SU-MIMO | Distributed |
| Closed Loop SU-MIMO | Localized |
| MU-MIMO | Localized |

Table 4 Supported resource channels in MIMO

In page 67 line 20~21, replace the sentence "Note that CQI, and rank feedback may or may not be frequency dependent." with "Note that CQI, and rank feedback may not be frequency dependent."

In page 69, replace line 1~11 with the follow text:

“equation:

P(k) = W(k).

The matrix W(k) is an NT  M unitary matrix, where NT and M are the numbers of physical and effective antennas, respectively. The matrix W(k) is selected from a predefined unitary matrix codebook, and changes every u subcarriers. [The detailed unitary matrix codebook is FFS]. The parameter u equals to Psc (18 continuous subcarriers).”

In page 69, replace line 19~21 with the follow text:

- 2Tx antennas, rate 1: SFBC

- 4Tx antennas, rate 1: SFBC with precoder

Replace line 35, page 69 ~ line 7, page 70 with the follow text:

“For the 4Tx rate-1, the output of the precoder is a 4 × 2 matrix

y=W× z,

where W is a 4 × 2 precoder. Note that W may be frequency dependent as described in section 11.8.2.1.1.

For the 8Tx rate-1, the output of the precoder is a 8 × 2 matrix

y=W× z,

where W is a 8 × 2 precoder. Note that W may be frequency dependent as described in section 11.8.2.1.1. ”

In page 70, replace line 31~40 with the follow text:

“For the 4Tx rate-2 mode, the output of the precoder is a 4  1 vector

y=W× z,

where W is a 4 × 2 precoder. Note that W may be frequency dependent as described in section 11.8.2.1.1.

For the 8Tx rate-2 mode, the output of the precoder is a 8 × 1 vector

y=W× z,

where W is a 8 × 2 precoder. Note that W may be frequency dependent as described in section 11.8.2.1.1. ”

In page 71, replace line 7~16 with the follow text:

“For the 4Tx rate-3 mode, the output of the precoder is a 4  1 vector

y=W× z,

where W is a 4 × 3 precoder. Note that W may be frequency dependent as described in section 11.8.2.1.1.

For the 8Tx rate-3 mode, the output of the precoder is a 4  1 vector

y=W× z,

where W is a 8 × 3 precoder. Note that W may be frequency dependent as described in section 11.8.2.1.1. ”

In page 71, replace line 28~32 with the follow text:

“For the 8Tx rate-4 mode, the output of the precoder is a 8 × 1 vector

y=W× z,

where W is a 8 × 4 precoder. Note that W may be frequency dependent as described in section 11.8.2.1.1. ”

-------------------------------------End of proposed remedy -------------------------------------

# Reference

[1] IEEE P802.16Rev2/D3, “DRAFT Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”, February 2008.

[2] Nortel Networks, “Performance Evaluation of DL Open Loop SU-MIMO Schemes”, IEEE C80216m-MIMO-08_058, Sep 2008.

[3] LG Electronics, Samsung Electronics and Intel Corporation, “Proposed harmonized OL-SU-MIMO formula based on SDD text”, IEEE C802.16m-MIMO-08/013, July 2008.

[4] Huawei, Matsushita Electric (Panasonic), ZTE, CATR and Alcatel-Shanghai Bell, “Rate-3 hybrid SM+STBC/SFBC scheme for up to 2 codeword with 4 Tx”, IEEE C802.16mMIMO-08/011, July 2008.

[5] Section 8.3.5.4.10.15 of IEEE P802.16Rev2/D3, “MIMO feedback for Tx beamforming”, February 2008.

[6] Intel Corp., ITRI, POSDATA Co. Ltd., MediaTek Inc. and Samsung Electronics, “DL Pilot Structure for IEEE 802.16m”, IEEE C80216m-08_648r6, July 2008

[7] Babak Hassibi, “An efficient square-root algorithm for BLAST”, 2000