| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Codebook design for IEEE 802.16m MIMO Schemes |  |
| Date Submitted | 2008-09-10 |  |
| Source(s) | Guangjie Li Qinghua Li, Hongming Zheng Shanshan Zheng Feng Zhou Senjie Zhang Minnie Ho Yang-seok Choi Intel Corporation | E-mail: Guangjie.li@intel.com Qinghua.li@intel.com Hongming.zheng@intel.com |
| Re: | PHY: MIMO; in response to the TGm Call for Contributions and Comments 802.16m-08/033 for Session 57 |  |
| Abstract | A universal codebook for both high and low correlated channels is depicted. It is built on and backward compatible to 802.16e codebook. |  |
| Purpose | Discuss and adopt in 802.16m |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Codebook Design for IEEE 802.16m MIMO Schemes

- Qinghua Li, Hongming Zheng, Shanshan Zheng

- Intel Corporation

- Introduction

Currently, two main codebooks are discussed in DL MIMO Rapp. One is 16e codebook [1], and the other is DFT codebook [2][3]. The 16e codebook has better performance for the uncorrelated channel while DFT codebook has better performance for the correlated channel. But none of them performs well in both channels. However, in reality, subscriber stations (SSs) experience both correlated channel and uncorrelated channel. Specifically, some SSs’ channels are correlated while the others’ channels are uncorrelated at the same time. For the each SS, the antenna correlation also varies slowly. It is undesirable for the system to maintain two sets of codebook and switch between them according to each correlation. There are ongoing efforts to design a single codebook for all scenarios.

Since backward compatibility to 802.16e is required, it is desirable to improve upon the 802.16e codebook for all correlation scenarios. In this contribution, we propose a simple transformation on the 16e codebook, where the transformation is computed from the antenna correlation. The transformation tunes the uniform 802.16e codebook pointing to a principal direction so that the transformed codebook adapts to any given correlation. Although the transformation complexity is negligible as compared to the receiver’s complexity, a simplified transformation is also devised, which share lots of commonalities with a differential feedback scheme. While the transformed codebook exploits the correlation in space and frequency domains, we also propose a differential codebook to exploit the time domain correlation. Simulation results demonstrate that the transformed 16e codebook and differential codebook outperforms the 16e and DFT codebooks for all scenarios in terms of throughput and feedback overhead.

2.	System Model

The assumed system model is

y=H V s+n,	(1)

where n is the complex AWGN with variance N0; s is the Ns by 1 transmitted vector with unit power; Ns is the number of spatial streams; y is the received vector; H is the channel matrix of size Nr by Nt; V is the beamforming matrix (or vector) of size Nt by Ns. The correlated channel matrix H is generated from the channel matrix Hw with independent, identically distributed (i.i.d.) entries as

H=Rr12HwRt12,	(2)

where Rr is the Nr by Nr receive covariance matrix and Rt is the Nt by Nt transmit covariance matrix. The covariance (or correlation) matrix Rt contains the averaged beamforming direction.

- 2.	Transformed Codebook

The advantage of the transformed codebook is illustrated in Figure 1. In this example, a 2×1 real (not complex) channel is assumed. The ideal beamforming vector is uniformly distributed over the semicircle for uncorrelated channels. The uniform codeword distribution of the 802.16e codebook matches to this input distribution and therefore has a good performance. In contrast, the DFT codebook suffers from the constant modulus constrain that requires all vector entries have the same magnitude. Although the more codewords are available, the DFT codebook only has two valid codewords in this case as shown by the two clusters at 45 and -45 degrees on the semicircle. This leaves large holes in the quantization space and causes large quantization errors for input vectors around 10 and 01. On the other hand, for highly correlated channels, the entry magnitudes of the input beamforming vector are close due to the high correlation of the channel responses. The DFT codewords with constant modulus entries matches to the input distribution and thus has a good performance. Note that there are two clusters of the DFT codewords and only one of them is used for each correlated scenario. The other cluster is too far away from the input vector is not used. This is the drawback of DFT codebook because it only exploits the rough information about the magnitude similarity. In contrast, the transformed codebook makes use of both the magnitude and phase information about the antenna correlation. The uniform codewords of 16e codebook are dynamically transformed to one cluster pointing to only one direction as shown on the right of Figure 1. As the correlation decreases, the codeword concentration of the transformed codebook decreases and the codebook degrades to the 802.16e codebook for uncorrelated channels, which has the best performance.

Figure 1. Codebook distribution and concept illustration.

Denote the transmit antenna correlation matrix (or covariance matrix) as Rt, which is measured as the SS, and denote the eigenvalue decomposition of Rt as Rt=QΣ2QH, where Q is an Nt by Nt unitary matrix i.e. QHQ=I; Σ is the diagonal matrix with the square roots of the eigenvalues σi for i=1,⋯,Nt in decreasing order σi≥σi+1. We would like to transform the uniform 802.16e codebook for the correlated channels. The transformation takes the form of

Vi=orthF Vi	(3)

where Vi and Vi is the i-th codeword of the original 802.16e codebook and the transformed codebook, respectively; orthX converts the input matrix (or vector) X to an orthogonal matrix with orthonormal column(s) that span the same subspace as X’s columns. orthX is essentially the orthogonalization of X’s columns and can be simply implemented by Grant-Schmidt. The transformation matrix F can have various choices e.g. 4th and square roots of Rt. For practicality, we take F=Rt12, which can be computed by Cholesky decomposition. Note that F becomes the identity matrix for uncorrelated channels and the transformed codebook automatically degrades to the 802.16e codebook for the optimal performance.

The advantages of the transformed codebook are:

- One codebook for all correlation scenarios.

- No additional codebook and codebook switch are needed.

- Backward compatible. Share the same codebook and indexing with the 802.16e codebook.

- Correlation matrix feedback incurs little overhead. It is fed back infrequently e.g. every 100 ms and shared by all subbands’ feedbacks.

- Feedback mechanism for long term CSI is already defined in the 802.16e standard.

  - Simplified Codebook Transformation

The scheme above can be simplified. Note that the ideal beamforming matrix has the its distribution peak at the column(s) or the principal subspace spanned by the columns of Q, where the principal subspace corresponds to the large eigenvalues of Σ. The variation from the peak diminishes as the correlation increases. The simplified codebook has two components: codebook center and polar cap. Firstly, the codebook center is the principal subspace spanned by the columns of Q. Secondly, the polar cap is a codebook that only covers a small portion of the quantization space of the 802.16e codebook as shown in Figure 2 for the vector case.

Figure 2. Full quantization space and polar cap.

For quantization, the SS first rotates the center of the polar cap to the principal subspace of Q and then quantizes the differential matrix from the input, ideal beamforming matrix to the principal subspace using the polar cap. Equivalently, the SS first removes the principal subspace from the ideal beamforming matrix and then quantizes the remaining using the polar cap codebook. The BS receives the quantization index and adds the differential matrix to the principal subspace for the final beamforming matrix.

Let Q=Q1Q2, where Q1 is Nt×Ns and is the quantized, principal subspace for beamforming; Nt and Ns are the numbers of transmit antennas and spatial streams; Q2 is Nt×Nt−Ns with the complementary columns of Q1. The beamforming, matrix quantization, and matrix reconstruction can be written as

Differential at SS:

D=Q1Q3HV	(4)

Quantization at SS:

D=argmaxDi∈CpDHDiF	(5)

Beamforming matrix reconstruction at BS:

V=Q1Q3D	(6)

Beamforming at BS:

y=H V s+n,	(7)

In (2), the Nt×Ns V is the ideal beamforming matrix; Q3 is the complementary columns orthogonal to Q1 and can be different from Q2 by a Ns×Ns rotation. In other words, Q2HQ3 is an Ns×Ns unitary matrix. Q3 can be computed from Q1 using Householder reflection so that the SS doesn’t need to estimate Q2 hidden in the channel samples. Q1 is fed back by the SS. Based on the same Q1, both the SS and the BS generates the same Q3 using the same method. In (5), Di is the codeword matrix of the polar cap codebook denoted by Cp; the quantization criterion maximizes the received signal power and other criteria such as channel capacity or MSE can be also used. In (5), V is the Nt×Ns reconstructed beamforming matrix and s is the Ns×1 data vector; H is the channel matrix with transmit antenna correlation Rt.

The quantization of Q1 and computation of Q3 are illustrated by an example with a 4×2 Q1. Denote the two column of Q1 as Q1=pq. p can be quantized by 16e 4×1 6-bit codebook. Denote the quantized p as p. The 4×1 q is converted to a 3×1 q by a Householder operation defined in 802.16e standard as

ΦpQ1=0.9×0.00.1×0.0q0.0,	(8)

where Φx=I−2w2wwH and w=x−e1=x1−1x2x3x4. q is then quantized by 16e 3×1 6-bit codebook. Denote the quantized p and q as p and q. The quantized version of Q1Q3, which is actually used by both BS and SS for the beamforming computation, is computed as

Q1Q3=Φp100000Φq0.	(9)

2.1.1. Polar Cap Codebook

For backward compatibility, the polar cap codebooks can be generated from the 802.16e codebooks using a simplified version of (3). The transformation reduces the chordal distance between each codeword and matrix 1⋱1 as illustrated in Figure 3. Note that each 3-bit vector codebook and some other codebooks of 802.16e have the codeword 1⋱1. Since the principal subspace i.e. Q1 is fed back separately, the transformation is simplified as

Vi=orthΛρ Vi,	(10)

where Λρ=λ1ρ⋱λNtρ is the diagonal matrix for correlation scenario ρ. We predefine Nρ, say 4, scenarios and precompute Λρ of each scenario. For example, Λ0.95=1.940.400.230.17 for 0.5 wavelength antenna spacing. After the SS feeds back the index of ρ e.g. using 2 bits, both the BS and the SS can generate the same polar cap codebook.

Figure 3. Generation of polar cap by codeword concentration.

3.	Differential Feedback

There exist strong correlations between beamforming matrixes in adjacent frequencies and subframes. The correlation can be exploited to reduce feedback overhead. We take example of time domain correlation to depict the differential feedback scheme. It is very similar to the polar cap scheme above if Q1 is replaced by the beamforming matrix of the previous subframe.

Differential at SS:

D=Vt−1Vt−1HVt	(11)

Quantization at SS:

D=argmaxDi∈CdDHDiF	(12)

Beamforming matrix reconstruction at BS:

Vt=Vt−1Vt−1D	(13)

Beamforming at BS:

y=H Vt s+n,	(14)

where the Nt×Ns Vt and Vt are the ideal and quantized beamforming matrixes for subframe t, respectively; Vt−1 has the complementary columns orthogonal to Vt−1 and Vt−1 can be computed from Vt−1 by a Householder operation as in (9). In (3), Di is the codeword matrix of the differential codebook denoted by Cd; the quantization criterion maximizes the received signal power and other criteria such as channel capacity or MSE can be also used.

The polar cap codebook can be used as the differential codebook for backward compatibility. Besides transformation approach in (10), an alternative is as follows. To lower the computational complexity, we concatenate polar cap vector codebook to build polar cap matrix codebook without the orth operation. The concatenation operation is the Householder operation defined in 802.16e standard. Let Cdm,1 denote the polar cap codebook for m×1 whose codewords are vi for i=1,2,⋯,Nm. The polar cap codebook for m+1×2 matrix denoted by Cdm+1,2 is generated by

Vi,j=Φvi000000110000vj,	(15)

where Vi,j is the codeword of Cdm+1,2 for i,j=1,2,⋯,Nm; Φvi is the m×m Householder matrix of vi defined in 802.16e standard.

- Feedback Overhead Analysis

Based on the cholesky decomposition method for the transformation matrix we will have the following feedback overhead analysis.

Feedback in the uplink will contain two parts: one is the codebook mapping index over 16e codebook, for example, every frame level (5ms); another is to feedback the quantization of transform matrix infrequently, for example, every 100ms once which can also give out the better performance.

So the total feedback overhead is the sum of 16e codebook index every 5 ms and quantized transform matrix every 100ms.

Quantization over Transform Matrix:

Transformation matrix by cholesky decomposition will have the following form as

Rt12=F11000F21F2200F31F32F330F41F42F43F44                   (16)

where the number of F11, F22, F33 and F44 are real value and all other numbers are complex values. We can use the following constellation points to quantize the above matrix.

0.4

0.9

0.0

Figure 4 Constellation Points for Quantization of Transform Matrix (4 bits)

Example for Feedback Overhead Calculation:

| FB Overhead | 16e |  | DFT |  | Transform |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Full | Best M=3 | Full | Best M=3 | Full | Best M=3 |
| Tx2 (bits/5ms /subband) | 3x12/12= 3 | (3x3+12*)/12= 1.75 | 3x12/12= 3 | (3x3+12)/12= 1.75 | (3x12+6/20)/12**= 3.03 | (3x3+6/20+12)/12= 1.78 |
| Tx4 (bits/5ms /subband) | 6x12/12= 6 | (6x3+12)/12= 2.5 | 4x12/12= 4 | (4x3+12)/12= 2 | (3x12+28/20)/12= 3.12 | (3x3+28/20+12)/12= 1.87 |

In the above feedback overhead calculation table we have the following notes.

*:

There are 12 subbands in the system and so the assumed bit map for Best-M is 12 bit

**:

(1) For Tx4, transform matrix F in equation (4) is 4x4 matrix and also with the quantization method for transform matrix described in above section. There are 4 real values in diagonal position (1bit for each value because diagonal element must be larger than zero) and also 6 complex values in off-diagonal positions in lower triangular part (4bits for each complex value) so the total feedback overhead is 1*4+4*6=28 over the time period of 100ms.

(2) For Tx2 transform matrix F is 2x2 matrix. There are 2 real values and 1 complex number in transform matrix. So the total feedback number is 1*2+4=6 over 100 ms.

In addition 16e codebook will feedback 3 bits for each subband under full feedback mode every 5ms. So the feedback ratio between transformation matrix F and 16e codebook is 100ms/5ms=20.

So from the above feedback overhead table we can easily conclude that transformed codebook can have very lower feedback overhead compared with DFT and 16e codebook.

Conclusion: for TX 4 due to transformed operation are done over 3 bits 16e codebook the feedback is much lower than 16e and DFT codebook. For TX2 configuration these 3 codebooks have the similar feedback overhead number.

- Link Performance

- [ To be Added]

- Conclusion

In this contribution, we proposed a simple improvement upon the 802.16e codebook for full backward compatibility. The 802.16e codebook is transformed according the antenna correlation. The transformed codebook delivers performances better than 802.16e and DFT codebooks for all scenarios.

- Reference

- IEEE 802.16e-2005, “IEEE Standard for Local and Metropolitan Area Networks - Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems,” Feb. 28, 2006.

- Q. Li, G. Li, X. Lin, and S. Zheng, “A Low Feedback Scheme for WMAN MIMO Beamforming,” IEEE Radio and Wireless Symposium, pp.349-352, Long Beach, CA, Jan. 2007.

- Proposed Text

##### 11.8.2.1.3 Feedback for SU-MIMO

In FDD systems and TDD systems, a mobile station may feedback some of the following information in Closed loop SU-MIMO mode:

• Rank (Wideband or sub-band)

• Sub-band selection

• CQI (Wideband or sub-band, per layer)

• PMI (Wideband or sub-band for serving cell and/or neighboring cell)

• Doppler estimation

[To add one more feedback item:

- Correlation matrix information or its equivalent information, for example, principle subspace and its eigen power, or long term CSI.]

For codebook based precoding, the feedback from a mobile station shall be based on the same codebook as used by base station for transmission.

[To add the text as:

Codebook design is scenario independent.]

###### 11.8.2.2.3.2 CSI feedback

Channel state information feedback may be employed for MU-MIMO. Codebook-based feedback is supported in both FDD and TDD. Sounding-based feedback is supported in TDD.

The unified codebook for SU and MU is employed.

[To add the following text between line 29 and 30 in page 73:

Correlation matrix information or its equivalent information, for example, principle subspace and its eigen power, or long term CSI will be supported for the codebook design to improve MU-MIMO performance.]