| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Performance Comparison on the Different Codebook for SU/MU MIMO |  |
| Date Submitted | 2008-09-04 |  |
| Source(s) | Shanshan Zheng Hongming Zheng Feng Zhou Guangjie Li Qinghua Li Senjie Zhang Minnie Ho Yang-Seok Choi Intel Corporation | E-mail: Guangjie.li@intel.com shanshan.zheng@intel.com; hongming.zheng@intel.com qinghua.li@intel.com |
| Re: | PHY: MIMO; in response to the TGm Call for Contributions and Comments 802.16m-08/033 for Session 57 |  |
| Abstract | Comparison of Codebook |  |
| Purpose | Discuss and adopt in 16m |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Performance Comparison on the Different Codebook for SU/MU MIMO

Shanshan Zheng, Hongming Zheng, Feng Zhou, Guangjie Li, Qinghua Li

Intel Corporation

# Introduction

IEEE802.16m MIMO RG kicked off the discussion of codebook comparison. In this contribution we have compared the performance of 802.16e codebook [1], DFT codebook [2] and transformed 802.16e codebook [3] under the different scenarios. This contribution is also the latest update and complementary for IEEE C80216m-MIMO-08_066 [2].

# Issues Covered

- Simulation Scenarios

Three types of correlated antenna at BS are considered here, all of them assume uncorrelated antenna at MS

  - Uncorrelated channel : zero correlation

  - Lowly correlated channel : 4 lambda antenna spacing with angular spread of 3 degree

  - Highly correlated channel : 0.5lambda antenna spacing with angular spread of 3 degree

- Simulation Analysis

  - Capacity comparison

  - System level simulation results

- SU/MU Schemes

  - SU: Codebook based SU precoding

  - MU: MU zero-forcing precoding [4]

- Codebook feedback overhead (bits/sub-band/frame)

  - 802.16e: 3bits for 2Tx and 6 bits for 4Tx

  - DFT: 3bits for 2Tx and 4 bits for 4Tx

  - Transformed 802.16e : 3bits for 2Tx and 4Tx plus periodic transform information [3]

# Guidelines for Codebook Design

- Scenario independent codebook

- Backward compatibility to 802.16e codebook

- Good Performance

- Low feedback

# Highlights of Codebook Performance

- Transformed 802.16e codebook works well in any scenario

# Capacity Analysis

In this section the capacity of closed-loop SU rate 1 in 4x2 antenna configuration is calculated. We adopted extended Ped-B channel model with 3km/h velocity. One resource unit (RU) is composed of 64 data sub-carriers by 6 symbols. The precoding vector/matrix for one RU is from different codebook. The capacity formula used here is

C=log2det(IMR+EsMTN0HequHequH)

where Hequ=H×V(i) i is codebook index

Figure 1 shows 802.16e codebook is better than DFT codebook in uncorrelated channel and DFT codebook is better than 802.16e codebook in correlated channel. Transformed 16e codebook can work well in any scenario. Transformed codebook is based on 3 bits or 6 bits 802.16e codebook. Transform information is measured and averaged over the whole band in frequency domain and over 100ms in time domain [3].

Figure 1 Capacity analysis in different scenarios

# System Level Simulation

[To Be Added]

# Conclusion

[To Be Added]

# Reference

[1] P802.16Rev2/D6, Part 16: Air Interface for Broadband Wireless Access Systems

[2] C80216mMIMO-08/066: Performance Comparison on the Different Codebook for SU MIMO

[3] C80216mMIMO-08/xxx: Codebook design for IEEE 802.16m MIMO Schemes

[4] C80216mMIMO-08/xxx: Comparison on Different MU MIMO Scheme

[5] C80216mMIMO-08/033r2: Evaluation Criteria and Work Plan for Codebook Issues

# Appendix

Simulation assumption [5]

| Basic Parameter | Assumption |
| --- | --- |
| OFDM parameters | 10 MHz (1024 sub-carriers) |
| Number of OFDM symbols per sub-frame | 6 |
| Permutation | Localized |
| Number of total PRU in one sub-frame | 48 |
| Number of PRU for PMI and CQI calculation and feedback | 4 |
| Number of PRU for rank calculation and feedback | 4 |
| CQI, PMI and rank feedback period | Every 1 frame (5ms) |
| Link Adaptation | QPSK 1/2 with repetition 1/2/4/6, QPSK 3/4, 16QAM 1/2, 16QAM 3/4, 64QAM 1/2, 64QAM 2/3, 64QAM 3/4, 64QAM 5/6 |
| MIMO receiver | Linear Minimum Mean Squared Error (LMMSE) |
| Data Channel Estimation | Perfect data channel estimation. |
| Feedback Channel Measurement | Perfect feedback channel measurement |
| Link Mapping | RBIR |
| Scheduling Criterion | Proportional Fair |
| Users per sector | 10 |
| Channel Models | Extended Ped-B |
| Mobile Speed | 3 km/h |
| Channel Scenario | 1.Uncorrelated channel : zero correlation 2.Lowly Correlated Channel: 4 lambda antenna spacing with angular spread of 3 degree 3.Highly Correlated Channel: 0.5 lambda antenna spacing with angular spread of 3 degree |
| Number of Antenna | 2 transmitter, 2 receiver [2Tx, 2Rx] 4 transmitter, 2 receiver [4Tx, 2Rx] |
| HARQ | Synchronized CC with maximum retransmissions of 4 |
| Network Parameters | Assumption |
| Cellular Layout | Hexagonal grid, 19 cell sites, wrap-around, 3 sectors per site |
| Distance-dependent path loss | L=130.19 + 37.6log10(.R), R in kilometers |
| Inter site distance | 1.5km |
| Shadowing standard deviation | 8 dB |
| Antenna pattern (horizontal) (For 3-sector cell sites with fixed antenna patterns) | Aθ=−min12θθ3dB2,Am θ3dB= 70 degrees, Am = 20dB |
| Other Cell interference | 8 dominant interferers |

# Proposed Text

##### 11.8.2.1.3 Feedback for SU-MIMO

In FDD systems and TDD systems, a mobile station may feedback some of the following information in Closed loop SU-MIMO mode:

• Rank (Wideband or sub-band)

• Sub-band selection

• CQI (Wideband or sub-band, per layer)

• PMI (Wideband or sub-band for serving cell and/or neighboring cell)

• Doppler estimation

[To add one more feedback item:

- Correlation matrix information or its equivalent information, for example, principle subspace and its eigen power, or long term CSI.]

For codebook based precoding, the feedback from a mobile station shall be based on the same codebook as used by base station for transmission.

[To add the text as:

Codebook design is scenario independent.]

###### 11.8.2.2.3.2 CSI feedback

Channel state information feedback may be employed for MU-MIMO. Codebook-based feedback is supported in both FDD and TDD. Sounding-based feedback is supported in TDD.

The unified codebook for SU and MU is employed.

[To add the following text between line 29 and 30 in page 73:

Correlation matrix information or its equivalent information, for example, principle subspace and its eigen power, or long term CSI will be supported for the codebook design to improve MU-MIMO performance.]