| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comparison of Single Codeword and Multiple Codeword |  |
| Date Submitted | 2008-09-12 |  |
| Source(s) | Guangjie Li, Senjie Zhang, Yuval Lomnitz, Feng Zhou, Hongming Zheng, Shanshan Zheng, Yang-seok Choi, Minnie Ho Intel Corporation Jen-Yuan Hsu, Yu-Tao Hsieh, Pangan Ting, Chang-Lan Tsai, Ren-Jr Chen, Richard Li ITRI Chih-Yuan Lin, Pei-Kai Liao and Paul Cheng MediaTek Inc. | E-mail: Guangjie.li@intel.com E-mail: ythsieh@itri.org.tw *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | PHY: MIMO; in response to the TGm Call for Contributions and Comments 802.16m-08/033 for Session 57 |  |
| Abstract | Comparison of Single Codeword and Multiple Codeword |  |
| Purpose | Discuss and adopt in 16m |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Comparison of Single Codeword and Multiple Codeword

Guangjie Li, Senjie Zhang, Yuval Lomnitz, Feng Zhou,

Hongming Zheng, Shanshan Zheng, Yang-seok Choi, Minnie Ho

Intel Corporation

Jen-Yuan Hsu, Yu-Tao Hsieh, Pangan Ting, Chang-Lan Tsai, Ren-Jr Chen, Richard Li

ITRI

Chih-Yuan Lin, Pei-Kai Liao and Paul Cheng

MediaTek Inc.

# Introduction

According to July IEEE meeting and the straw-poll, SCW is mandatory to support and whether or not to support the MCW with maximum of 2 codewords is the focus of debates.

We think to support both SCW and MCW will definitely make the 16m standard more complicated, and will increase overhead, and increase the complexity of MS receiver. Only if MCW can show obvious benefit, it’s reasonable to support MCW in 16m.

The factors to consider are

- Performance

- System complexity

- Receiver complexity

- Control and feedback overhead.

- 2x2, 4x2 MIMO – high priority

- 4x4 MIMO – lower priority (less popular for mobiles)

# Summary of the study

- Performance

  - When channel is known (low speed, localized permutation, fine link adaptation is possible)

SCW CL MMSE is the best choice in practical SNR range (better than MCW), while SCW OL MLD is the better in high SNR range

  - When channel is unknown (high speed, distributed permutation, only coarse link adaptation based on geometry SINR is possible)

SCW open loop MLD has significant gain over other schemes.

- Overhead

MCW will introduce more CQI feedback overhead, low MAC control signaling than SCW.

- Complexity

MMSE baseline receiver has very low complexity and is important for scheme comparison and RCT.

The memory requirement from SIC buffer will make SIC receiver much costly, not as efficient as MLD receiver.

The SIC receiver introduces more detection delay. The shorter turbo coder length degrades the performance.

4x4 MLD has smaller silicon size than SIC when quasi-MLD is utilized.

- Summary

It’s a right decision to keep SCW as the only MIMO mode to make 16m efficient.

For 2x2, SCW MLD > MCW SIC, while the complexity of SCW MLD is lower than MCW SIC

As 4x4 is a less popular configuration, we may use MMSE which suffers some performance reduction compared to SIC, or use suboptimal MLD schemes.

In both cases, the RCT performance based on baseline MMSE receiver will be considerably better with SCW than in MCW

System and MS complexity are significantly smaller with SCW

# Performance comparison

The performance evaluation is classified into two cases.

- Coarse link adaptation

Channel is unknown at Tx side, and coarse link adaptation is based on geometry SINR. This case is suitable for high speed and distributed permutation. Open loop single user MIMO is applied for comparison. Only SM with rank adaptation are used for evaluation (diversity mode not considered).

- Fine link adaptation

Channel is known at Tx side by feedback, and fine link adaptation is possible to track the small scale fading of channel. This case is suitable for low speed and localized permutation.

Both CL-SU MIMO and OL-SU MIMO are used for evaluation.

Based on the RG group discussion, two kinds of receiver are listed for comparison, which are baseline MMSE receiver for both SCW and MCW and fancy receiver SCW+MLD and MCW+SIC. The receiver type and MIMO mode we considered are listed in the following table.

Table 1 Receiver and MIMO mode for evaluation

|  | Mode | Baseline receiver | Advanced receiver | Main focus |
| --- | --- | --- | --- | --- |
| Fine LA (low speed, localized permutation) | OL/CL MCW | MMSE | MLD MMSE+SIC MLD+SIC | MMSE+SIC |
|  | OL/CL SCW | MMSE | MLD Iterative MAP decoding | MLD |
| Coarse LA (Channel is unknown, high speed, distributed permutation) | OL MCW | MMSE | MLD MMSE+SIC MLD+SIC | MMSE+SIC |
|  | OL SCW | MMSE | MLD Iterative MAP decoding | MLD |

The major simulation assumption is listed in the appendix.

Coarse Link Adaptation

SM OL SU MIMO is used with coarse link adaptation for SCW/MCW comparison.

For MCW+SIC receiver, there are two cases

- Equally bit loading with optimum detection order (Same MCS level for all codewords)

- Unequally bit loading with fixed detection order (Conservative MCS are used for first one or two codewords to benefit from SIC, always detect from 1 to N codeword with fixed detection order)

Firstly, we compare the “equally bit loading” MCW+SIC with SCW MMSE. The outage capacity analysis is utilized. Outage is 0.01.

The 2x2 and 4x4 results are showed in figure 1. According to the result, MCW-SIC > SCW-MMSE >> MCW-MMSE. With baseline MMSE receiver, SCW is much better than MCW. When SIC is used, MCW-SIC is better than SCW-MMSE, however the slight gain is not worthy of the complexity.

Notes: MLD capacity is not shown here.

Figure 1. Outage capacity of SCW/MCW when channel is unknown and optimum order for SIC receiver

The fancy receivers for SCW are MLD and iterative MAP receiver.

The figure 2 shows the performance gain of MAP receiver over MLD receiver. In the simulation, the MAP receiver only uses 1 iteration in Turbo decoder part. The bold blue line is the pure MLD result with 8 Turbo decoder iterations.

For SCW, there is big potential to utilize the iterative MAP receiver to get better performance.

Figure 2. The gain of MAP receiver over MLD receiver.

Secondly, we compare the MCW SIC of “Unequally bit loading with fixed detection order” with SCW MMSE and SCW MLD in LLS with fixed MCS level (No Link adaptation). The simulation assumptions are:

Distributed permutation, 120km/h, PRU-based DRU (4 PRUs), CC HARQ with 5ms interval and maximum 3 re-transmission, perfect CE.

The throughput curves are the outline of multiple curves.

For SCW-MMSE, SCW-MLD, MCW-MMSE, 8 MCS level used for all the codewords.

For MCW-SIC, fixed detection order with 14 MCS combinations for multiple codewords are used, they are listed in table 2.

The results are shown in figure 3. The SCW-MLD shows the superior performance to any other schemes.

According to the result, with base line receiver, SCW-MMSE is better than MCW-MMSE. And with fancy receiver, SCW-MLD also is much better than MCW-SIC.

It’s correct decision to use SCW when channel is unknown.

Table 2 MCS combination for MCS+SIC

| 2x2 | 4x4 |
| --- | --- |
| (QPSK 1/2, QPSK 1/2) (QPSK 1/2, 16QAM 1/2) (16QAM 1/2, 64QAM 1/2) (16QAM 1/2, 64QAM 2/3) (16QAM 3/4, 64QAM 3/4) (64QAM 3/4, 64QAM 5/6) (QPSK 3/4, QPSK 3/4) (16QAM 3/4, 16QAM 3/4) (16QAM 1/2, 16QAM 1/2) (64QAM 1/2, 64QAM 1/2) (64QAM 2/3, 64QAM 2/3) (64QAM 3/4, 64QAM 3/4) (64QAM 4/5, 64QAM 4/5) (64QAM 5/6, 64QAM 5/6) | (QPSK 1/2, QPSK 1/2, QPSK 1/2, QPSK 1/2) (QPSK 1/2, 16QAM 1/2, 16QAM 1/2, 16QAM 1/2) (16QAM 1/2, 64QAM 1/2, 64QAM 1/2, 64QAM 1/2) (16QAM 1/2, 64QAM 2/3, 64QAM 3/4, 64QAM 3/4) (16QAM 3/4, 64QAM 3/4, 64QAM 5/6, 64QAM 5/6) (64QAM 3/4, 64QAM 5/6, 64QAM 5/6, 64QAM 5/6) (16QAM 1/2, 16QAM 1/2, 16QAM 1/2, 16QAM 1/2) (QPSK 3/4, QPSK 3/4, QPSK 3/4, QPSK 3/4) (16QAM 3/4, 16QAM 3/4, 16QAM 3/4, 16QAM 3/4) (64QAM 1/2, 64QAM 1/2, 64QAM 1/2, 64QAM 1/2) (64QAM 2/3, 64QAM 2/3, 64QAM 2/3, 64QAM 2/3) (64QAM 3/4, 64QAM 3/4, 64QAM 3/4, 64QAM 3/4) (64QAM 4/5, 64QAM 4/5, 64QAM 4/5, 64QAM 4/5) (64QAM 5/6, 64QAM 5/6, 64QAM 5/6, 64QAM 5/6) |

Figure 3. OL SU MIMO comparison with fixed MCS in LLS

Fine Link Adaptation

When mobility speed is low and using the localized permutation, it’s suitable to use fine/fast link adaptation to track the small scale fading of channel.

In the evaluation, SU-CL and SU-OL are used, and RBIR based link/rank adaptation are adopted for MMSE and MMSE-SIC receiver, while MIC based link/rank adaptation are used for MLD. The MCS level and rank are selected to optimize the throughput, and then link MIMO detector and Turbo decoder will check the good or bad throughput with the selected MCS/rank. Here perfect CE and localized permutation is assumed.

The results are shown in figure 4.

Figure 4, SCW/MCW comparison with fine LA

According to the result, when fine/fast LA is possible, the performance order is

CL SCW-MMSE = CL MCW-SIC >OL SCW-MLD ~> OL MCW-SIC > OL SCW-MMSE > OL MCW-MMSE

The overhead order is

CL MCW-SIC ~> OL MCW-SIC = OL MCW-MMSE > CL SCW-MMSE > OL SCW-MMSE = OL SCW-MLD

For close loop MIMO, the MCW-SIC can not show gain over SCW-MMSE.

For open loop MIMO, the SCW-MLD is slightly better than MCW-SIC.

The best choice for this scenario is CL SCW-MMSE, which has the best performance while the complexity and overhead are low.

# System complexity

In Table 3, the system complexity comparison is listed.

According to the analysis, the MCW will definitely introduce much complexity to the system, while SCW is simple and efficient.

Table 3 System complexity comparison

| items | SCW | MCW | Notes |
| --- | --- | --- | --- |
| Legacy support | Yes | No | 16m MS is required to access the 16e BS, SCW is mandatory |
| CQI/CSI feedback | 1 CQI per RB | Multiple CQI per RB | SCW has less CQI feedback |
| SU/MU unification | MU MIMO most like to report 1 CQI/CSI per RB which is aligned with SCW CQI/CSI feedback. (CQI adjust is required at BS for flexible SU and MU adaptation) | MU MIMO CQI/CSI feedback is not aligned with SU CQI/CSI feedback. The feedback overhead is different which require to re-allocate CQICH. | SCW is better in terms of SU/MU unification |
| HARQ | Simple HARQ | Blanking and non-blanking HARQ require special design to support, which will introduce complexity and control overhead | MCW HARQ is more complicated |
| Delay | Small delay because of pipeline detection/decoding | Large detection/decoding delay because of iterative detection. | SCW is better |
| Low MAC & Control | Simpler low MAC & control | Require to add new features in low MAC and control to support MCW, like data partition/assembly, resource allocation etc | MCW introduce complex to control and low MAC design |

# Receiver complexity

Baseline MMSE receiver has very low complexity, and it’s very important to scheme comparison.

The SIC and MLD fancy receiver structure of 2x2 are shown in figure 5, 6 ,7

Figure 5: 2x2 SCW/MMSE Rx model

Figure 6: 2x2 SCW/MLD Rx model

Figure 7: 2x2 MCW/SIC Rx model

The properties of these two fancy receivers are listed as following.

SIC

- Iterative detection and turbo decoding

- Low computation complexity

- Not suitable for pipeline processing, and introduce large decoding/detection delay

- Large memory is required for cancellation, which will occupy large silicon size.

MLD

- Pipeline detection and turbo decode, small delay

- High computation complexity when 4x4 and 64QAM

- Quasi-ML detection algorithm is extensively explored to reduce the complexity : QRM-MLD, Sphere Decoding,

- The logic size can be reduced by logic sharing with the increase of processing clock.

For 2x2 and 4x2 MIMO with maximum rank 2, the MLD has lower complexity than SIC while the performance is better (R1-061965). SCW MLD should be used.

For 4x4 MIMO with maximum rank 4, the complexity analysis is as following.

The assumptions are

RB is 48 PRU (48*16*6 = 4608 subcarriers)

1kByte RAM is assumed to be equivalent to 5.6K of NAND2 equivalent gates

64 gates for 16bits real adders and 2000 gates for 16bits real multiplier

Logic share with the increase of the processing clock

Power consumption is not considered.

Baseline MMSE receiver

For baseline MMSE receiver, square root detection algorithm and CORDIC algorithm for rotation are used, the gate number are 337.6k gate (pipeline, block per clock).

SIC receiver

For SIC receiver, additional buffer is required. There are two methods

- Buffer the channel H after CE

The buffer size are 4*4 (H) *2 (I/Q) * 12 bits * 4608 tones) + (6 bits * 5/6 * 4608 tones) = 1.79M bits = 224KB = 1.25M gates

- Only buffer the receiver single and pilot, need to re-do the CE when cancellation, which require more complexity.

(4 Rx antennas * 2 (I/Q) * 12 bits * 4608 tones) + (6 bits * 5/6 * 4608 tones) = 465k bits = 59 KB = 325 K gates

Iterative MMSE cancellation is assumed to be 2.5 x complexity of pure MMSE. 844k gate (block per clock).

MLD receiver

QRM-MLD is assumed, M = 16 for 16QAM and 64QAM, and the approximation of square is used for the simplified slicing (distance calculation).

The QR decomposition requires 171K gates.

The number of equivalent adder for most complicated candidate search part of MLD is (64*18 + 16*64*26 + 16*64*35 + 16*64*44) = 108672 (6.9M gates)

The total number is 7.07M gates

Computation logic sharing

In order to save logic size, the computation logic sharing is utilized.

The share factor is CLK * 0.625*10^(-3) /(4608 ) = 27  times for 200MHz processing clock.

The gates counted are

256K gates for MLD.

12.5K gates for MMSE

1.28M gates for SIC receiver (buffer H)

358K for SIC receiver. (buffer Rx signal, not consider re-do CE)

According to the analysis, the total gate number of MLD is less than SIC when processing clock is high

# Conclusions

According to the analysis of performance, complexity and overhead, it’s not necessary to add a new MCW mode for 16m MIMO.

It’s right decision for 16m MIMO to keep SCW only.

# Appendix

A. Simulation assumption

| Parameters | Values |
| --- | --- |
| Bandwidth | 10 MHz |
| FFT size | 1024 |
| Carrier frequency | 2.5 GHz |
| Subframe structure | According to the latest revision of IEEE 80216m-08/003 |
| Channel Model | Modified ITU PedB/VA with spatial correlation |
| Mobile speed | OL MIMO: 3 km/h |
| Antenna configuration | 2x2, 4x4, 4x2 |
| BS antenna spacing | 4 lamda, 15 degrees angle spread |
| Receiver algorithm | Baseline receiver: MMSE for both SCW and MCW Fancy receiver: SIC for MCW and MLD for SCW |
| Resource allocation and channelization | Case 1: With fine LA, Localized allocation 3km/h: Case 2: With coarse LA, distributed allocation, 120km/h |
| Channel coding & modulation | 16e 8 MCS |
| Coded block size | SCW: one coded block for assigned resources MCW: one coded block for layer |
| Channel estimation | Perfect channel estimation |
| HARQ scheme (async. vs. sync, IR vs. Chase Combining, adaptive vs. non-adaptive, retransmission delay) | Synchronous HARQ with Chase Combining, Retransmission delay = 5ms with maximum 3 retransmissions and initial target PER of 10% Blanking HARQfor MCW simulation |
| Rank and CQI feedback period and delay | Every 5ms and with 5ms delay |
| Link adaptation method | RBIR for MMSE receiver and MIC for MLD receiver |
| Throughput statistic | Based on the good-put from real Turbo decoder. |
| codebook | 16e codebook for CL MIMO 3 bits for 2 Tx and 6 bits for 4 Tx |

# Proposed text

#### 11.8.1.2 Layer to Stream Mapping

Delete “[The support of horizontal encoding (MCW) for SU-MIMO is FFS].”