| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Link Performance Abstraction for ML Receivers based on RBIR Metrics |  |
| Date Submitted | 2008-03-04 |  |
| Source(s) | Hongming Zheng, Intel Corporation May Wu, Intel Corporation Yang-seok Choi, Intel Corporation Nageen Himayat, Intel Corporation Jingbao Zhang, Intel Corporation Senjie Zhang, Intel Corporation Louay Jalloul, Beceem Communications | hongming.zheng@intel.com may.wu@intel.com yang-seok.choi@intel.com nageen.himayat@intel.com jingbao.zhang@intel.com senjie.zhang@intel.com Jalloul@beceem.com |
| Re: | IEEE 802.16m-07/48 – Call for Comments on Draft 802.16m Evaluation Methodology Document IEEE 802.16m-07/037r2 and the action item in comment #1 in the commentary database 802.16m-08/001r2 |  |
| Abstract | This contribution provides a link abstraction methodology for ML receivers based on RBIR metrics. |  |
| Purpose | For discussion and approval by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Link Performance Abstraction for ML Receivers based on RBIR Metrics

Hongming Zheng, May Wu, Yang-seok Choi,

Nageen Himayat, Jingbao Zhang, Senjie Zhang, Intel Corporation

Louay Jalloul, Beceem Communications

# 1.0 Purpose

This contribution provides a detailed description of a link evaluation methodology for MIMO Maximum likelihood (ML) receivers. With our proposed modeling technique, we obtain accurate link abstractions, which are based on the mean RBIR (Received Bit Information Rate) mapping between the transmitted symbols and their received LLR values after symbol-level ML detection.

# 2.0 Introduction

In order to reduce the complexity arising from modeling the actual link performance within system level simulations, an accurate block error rate (BLER) prediction method is required to map link performance for system capacity evaluation.

A well-known approach to link performance prediction is the Effective Exponential SINR Metric (EESM) method. The EESM has been widely applied to OFDM link layers ([1]-[3]) and Linear MMSE detection receiver algorithms, but this approach is only one of the many possible methods for predicting link performance.

The EESM relies on computing an “effective SINR” metric from a vector of post-processing SINR values obtained after linear detection in an OFDM system. One of the disadvantages of the EESM approach is that a normalization parameter (usually represented by a scalar, β) must be computed for each modulation and coding (MCS) scheme for many scenarios. In particular, considering broader link-system mapping applications, it can be inconvenient to use the EESM for adaptive modulation when HARQ is used in the system, as adaptive HARQ requires that codewords with different modulation types be combined across the different transmission/retransmissions. In addition, it is difficult to extend this method to ML detection in the SISO/MIMO case because it is not easy to directly compute the required post-processing SINR values.

In order to overcome the shortcomings of EESM as described above, in this contribution we focus on the conventional Mutual Information (RBIR) method for the phy abstraction/ link performance prediction for MLD receivers. This contribution provides a computationally-efficient method for computing the RBIR metric for ML receivers that is easily extensible for the MIMO case.  Once the RBIR metric is computed, link performance can be predicted simply by using the metric to look up AWGN BLER curves for error-rate performance.

Computing the RBIR metric for the ML receiver is difficult because it relies on the computation of mutual information per symbol, which is obtained as an “expected value” of symbol-level log-likelihood ratios (LLR).  Since no closed-form expression is available for this expected value, it must be computed through numerical integration over channel-dependent LLR expressions. Therefore, the computation of the RBIR metric can become prohibitive for predicting instantaneous link performance. In this contribution, we show that the LLR distribution can be well-approximated by a Gaussian distribution with mean and variance that are a function of channel-dependent “effective SINR” values. Therefore, the symbol-level mutual information can be pre-computed off-line for a range of mean and variance values and stored as a table. This table can then be utilized for computing the RBIR values for predicting instantaneous link performance. For MIMO, a channel-eigenvalue dependent, fit parameter is introduced, which allows for re-using the symbol information tables for the SISO case. We note that our proposed RBIR method for MIMO-ML receivers can be applied to both “vertical” and “horizontal” encoding profiles defined for WiMAX.

The organization of this contribution is as follows. Section 3 of the contribution provides an overview of the RBIR PHY abstraction metric for symbol-level ML detection.  Section 4 covers the theoretical analysis, as well as the simulation results to justify the use of the Gaussian approximation for modeling the LLR distribution for the ML receiver.  Both SISO, as well as MIMO cases are considered. Section 5, covers the detailed steps required to compute RBIR PHY mapping for an ML receiver, both for the SISO and MIMO systems. Finally this contribution includes the proposed text for 802.16m Evaluation Methodology Document on RBIR.

# 3.0 Overview of the RBIR Mapping

This section describes the RBIR definition for a SISO system, focusing on the notation and theoretical concepts.  Additionally, the computation of the actual RBIR from symbol-level level log likelihood ratio (LLR) values will be derived in detail.

The RBIR metric is computed from the per subcarrier symbol mutual information values comprising a coded block, as follows

where SIn is the mutual information over the n-th subcarrier and m(n) is the information bit per symbol over the n-th subcarrier.

The computation of the symbol mutual information (SI) is dependent on the symbol-level log-likelihood ratio (LLR).  The symbol-level LLR given symbol xi is transmitted, can be computed for the ML receiver as follows:

In the above, di, (i=1, 2, …, M), indicates the Euclidean distance of the symbol xi  from the  current received symbol.  Specifically, di=y−Hxi=y−Hxiy−HxiH, where  xi represents the ith symbol.

The mutual information per symbol as symbol, SI,  is given by the following expression:

If the probability distribution function (pdf), p, of the LLR values is known, then SI can be calculated as:

For QPSK modulation, the pdf of LLR values is independent of the specific transmitted symbol, but for the general case of QAM modulation, the LLR pdf must be computed for each symbol within the QAM constellation, separately. However, since the Euclidean distance from the first tier constellation dominates the LLR (i.e. first 3 or 4 neighboring constellation points), for QAM we can approximately calculate the LLR using the dominant 3-4 constellation points as follows

.

For example, for 16 and 64-QAM, the outer constellation point will have 3 dominant Euclidean distances while the inner constellation points will have 4 dominant Euclidean distances. Note that the inner and outer constellation may have different pdf for the LLR. For simplicity, we propose to choose one representative LLR among N possibilities to represent the signal quality. In the simulation we just choose the constellation point (1,1,..,1) as the representative value.

If the symbol-level LLR can be modeled as a Gaussian distribution, then the SI over the n-th subcarrier can be derived through numerical integration as follows.

where it is assumed that symbol LLRi under ML detection satisfies the Gaussian distribution with mean= AVEi and the variance =VARi.

In the following section we will validate the Gaussian approximation for the symbol LLR distribution through theoretical analysis as well as through simulation results.

# 4.0 The Gaussian Approximation for Symbol-Level LLR Distribution for ML Receivers

In this section we will theoretically derive the symbol LLR expressions, assuming different modulation levels such as QPSK, 16-QAM and 64-QAM schemes.  We will first show the results for the SISO case and then extend it for the 2x2 MIMO (matrix B) case as well. In the following we will need to use the parameter d, characterizing the minimum distance of the QAM constellation. For example,,  and  for the case of QPSK, 16-QAM and 64-QAM modulations, respectively.

## 4.1 Theoretical Derivation of Symbol LLR Distribution (SISO QPSK as Example)

We first consider the case of QPSK modulation, for a SISO system.

It is easy to show that the LLR value for the ith symbol is given by the following expression:

where

and

From the above formula we can see that, for QPSK, the symbol LLRi can be approximated as Gaussian distribution, where the average of LLRi is:

The variance of LLRi is

For that:

Here:

Then LLRi is distributed as:

Similar conclusions for the case of 16QAM and 64QAM modulations may also be obtained.

## 4.2 Simulation Results for Symbol LLR Distributions (SISO) – QPSK/16QAM/64QAM

In this section we compare the theoretical LLR distributions derived in the previous section, with those obtained through simulations. Assuming that the transmitted symbol is ’11 …1’, the LLR distributions under different normalized fading factor ‘h’ are simulated as in Figure 1a, 1b and 1c for the different modulation schemes. In Figure 1a-1b-1c the black curves are the standard Gaussian curves generated by Matlab, which are used to approximate the real LLR value shown by the red curves. It is verified that the mean and variance correspond to the derivation of LLR distribution in the previous section.

From the figures below, it is easy to see that the distribution of the symbol level LLR values for ML detection, can be well-approximated by the Gaussian distribution. This result is also consistent with the theoretical derivation of symbol LLR distribution, shown in the previous section. In the example shown, we assume the QPSK SISO case. Assuming h=1, the AVE and VAR1/2 can be computed can be computed as follows:  when SNR = 5dB, AVE = 4.2147 and VAR1/2 = 2.8290; when SNR = 10dB, AVE = 16.3990 and VAR1/2= 5.0956. From the figures, it is also clear that the simulation results closely match the theoretical LLR distributions for 16QAM and 64QAM as well.

Figure 1a QPSK LLR Distribution (SISO)			Figure 1b 16QAM LLR Distribution (SISO)

Figure 1c 64QAM LLR Distribution (SISO)

## 4.3 Theoretical Derivation of Symbol LLR Distribution (MIMO QPSK as Example)

We first consider the case of a 2x2 spatially multiplexed (SM) MIMO system (Matrix B).  The LLR distribution of each stream is derived separately.  The signal model for the 2x2 SM with MLD reception is

For the 1st stream:

The LLR for the first stream of 2x2 Matrix B is

Where:

From the above we can see that the symbol LLR for the first stream can still be approximated as a Gaussian distribution. The distribution is given by

where

For simplicity, the different conditional LLR1i distributions can be approximated by the same Gaussian because we used the dominant constellation points for LLR calculation.

And

For high SNR we will have

Similar expressions for the case of 16QAM and 64QAM modulations may also be obtained, and the same derivation can be used for the 2nd stream of a 2x2 MIMO system as well.

## 4.4 Simulation Results for Symbol LLR Distributions (MIMO) – QPSK/16QAM/64QAM

Assuming that the transmitted symbol is ’11 …1’ for each of the 2 transmit antennas, the LLR distributions under different fading factors ‘H’ are simulated as in Figure 2a, 2b and 2c for the different modulation.

The channel matrix used in the example is H = [-0.1753 + 0.1819i   0.1402 + 0.5974i;    0.4829 - 0.2616i   0.4019 + 0.3107i] and the figures give the LLR distribution obtained from H and SNR.

In Figure 2a-2b-2c the black curve is the standard Gaussian curve generated via Matlab, which is used to approximate the real LLR distribution shown by the red curve.  The figures shown are for a “horizontally encoded MIMO system. Therefore, the two LLR distributions corresponding to the two streams are shown separately.

From the figures below, it can be seen that the symbol level LLR for an ML receiver can be well-approximated by the Gaussian distribution, which is also consistent with the theoretical derivation of symbol LLR distribution, described in the previous section.

In the example shown, 2x2 SM QPSK is assumed. The channel matrix H=[ -0.1753 + 0.1819i   0.1402 + 0.5974i;    0.4829 - 0.2616i   0.4019 + 0.3107i]. The AVE and VAR can be computed as follows: when SNR = 5dB, AVE1 = 0.8848;  VAR11/2 = 1.6756;  AVE2 = 2.2740; VAR21/2 = 2.2347; when SNR = 10dB,  AVE1 = 5.0586; VAR11/2 = 3.0481; AVE2 = 9.7909;  VAR21/2  =  4.0439.

Figure 2a QPSK LLR Distribution (Matrix B 2x2)			Figure 2b 16QAM LLR Distribution  (Matrix B 2x2)

Figure 2c 64QAM LLR Distribution  (Matrix B 2x2)

# 5.0 Computing the RBIR PHY Abstraction for an ML Receiver

## 5.1 Summary of the Generalized Symbol LLR PDF Model

As shown in the previous section, the conditional PDF of symbol LLR can be approximated by the Gaussian distribution. For the SISO case, the LLR distribution for the ML receiver can be written as .

For a MIMO Matrix B, 2x2 system, the conditional PDF of symbol LLR output can be approximated by two different Gaussian curves for each of the two streams for the ‘horizontal’ encoded system. The per stream LLR for the ML receiver can be written as.

For the MIMO case, we need to introduce an optimization parameter ‘’ for weighting with the AVEstream to minimize the difference between effective SINR and AWGN SINR for a given set of BLER value. The parameter ‘’ is related to the channel condition number, which is obtained through the eigenvalue decomposition of the channel.  However, this fit parameter 'a' has been found to be independent of the specific power delay profiles and the MIMO channel models.   Specifically, the LLR distributions for the 2x2 MIMO case are modified as follows:

For MIMO Matrix B 2x2 and a ‘vertical’ encoded system, the distribution of LLR from ML receiver can be written as. Where p1 and p2 are additional fit parameters that may be computed by minimize the difference between effective SINR and AWGN SINR for a given set of BLER values.

The simplified Gaussian approximation on the symbol LLR is beneficial for different ‘encoding’ schemes and antenna configurations (for example, 4x4, etc). This approach can reduce the offline optimal parameter searching complexity greatly, and make the search practical.

Using a single Gaussian distribution to model the LLR pdf for different modulation schemes, significantly reduces the PHY abstraction computational complexity when compared to the MMIB method. For the case of MMIB, two Gaussian distributions are used to approximate the LLR distribution for the QPSK case. Whereas,  for the 16-QAM and the 64-QAM cases, 4 and 6 Gaussian  distributions are required to model the LLR distribution  for a ‘horizontal’ encoded system, respectively. Having many Gaussian distributions to model bit-level LLR distributions increases the complexity of the offline parameter search, and makes it difficult to extend the PHY abstraction computation to a 4x4 antenna system.

## 5.2 Procedure for RBIR PHY Mapping for SISO/MIMO System under ML Receiver

One of the key features behind the RBIR PHY abstraction for the ML receiver, is that once the channel matrix “H” and the SNR is known, the relationship between the LLR distribution and the BLER is fixed. This implies we can have the fixed, table driven method for predicting the PER/BLER per link

As shown in Section 4.0 the real symbol LLR distribution, given channel matrix ‘H’ and SNR, can be approximated as formula (1.10 – 1.13 and 1.20 – 1.23). So we can set up the fixed mapping function between the parameter-bin (H, SNR) and PER/BLER (from real LLR distribution) which is our RBIR PHY Mapping function for ML symbol-level detection.  This mapping function is typically computed and stored off-line.  We will provide this mapping in a tabular form as part of the proposed text in the standard, in Section 7.

As mentioned before, for the MIMO case an optimization parameter  is used to weight the AVEstream to minimize the difference between effective SINR and AWGN SINR for a given set of BLER values.  The parameter   depends on the channel condition number, which is obtained through the eigen-value decomposition of the channel, and reflects the interference impact between the different streams.  The parameter  is typically computed through an off-line computation.  In section 7, we provide the values of the parameter a in tabular format.

Procedure for RBIR PHY Mapping on symbol-level ML detection:

- Calculate the Symbol-Level LLR distribution (AVE, VAR) given the channel matrix ‘H’ and SNR.

Given the channel matrix ‘H’ and SNR for each subcarrier, the fixed LLR distribution parameter pair (AVE, ) can be computed from formulas in equations (1.10 – 1.13 and 1.20 – 1.23). The detailed formula/numerical derivation is also given as part of the proposed text in Section 7.

- Calculate the SI/RBIR metric based on SI/RBIR definition (formula 1.5 and 1.6) and LLR distribution as Step 1.

After calculating the mean and variance of LLR (AVE, VAR) for a given subcarrier, the SI can then be computed as:

We also can use the proposed method [6, Beceem] to simplify the numerical integration (1.24).

For the MIMO case, we use the parameter ‘a’ to modify the LLR distribution as follows

.

- Sum the SI values over the multiple subcarriers for OFDM system.

- Divide the SI sum by the sum of bits per symbol to get RBIR.

- Convert the RBIR for one resource block to one single ‘effective SINR’ from the SNR-to-RBIR Table given by later section.

- Lookup the AWGN table to get the predicted PER/BLER.

# References

[1] 3GPP TSG-RAN-1, Nortel Networks, "Effective SIR Computation for OFDM System-Level Simulations," Document R1-03-1370, Meeting #35, Lisbon, Portugal, November 2003

[2] Ericsson, “System-level Evaluation of OFDM – Further Considerations”, R1-031303

[3] Lei Wan, “A Fading-Insensitive Performance Metric Unified Link Quality Model”, VTC paper, 2006

[4] Mot, “Link Performance Abstraction for ML Receivers based on MMIB Metrics”, IEEE C802.16m-07/142

[5] Mot, “Link Performance Abstraction based on Mean Mutual Information per Bit (MMIB) of the LLR Channel”, IEEE C802.16m-07/097

[6] Beceem, Louay Jalloul, “On the Expected Value of the Received Bit Information Rate”, Sept. 2007

[7] Hongming Zheng et al., “Link Performance Abstraction for ML Receivers based on RBIR Metrics,” IEEE C802.16m-07/187r4, Atlanta, Georgia, November 2007.

[8] Thierry Lestable, Alain Mourad, Ming Jiang, Youngkwon Cho, Hyunkyu Yu, Taeyoung Kim, Jaeweon Cho, Louay Jalloul, “Enhanced Approximation for RBIR PHY Abstraction in TGm”, IEEE C802.16m-08/067r4.pdf, March 2008.

# Proposed Text

# 1) Update Section 4.3.1 as the text below.

# -----------------------------Begin Proposed Text ----------------------------------------------------------------------

# Received Bit Mutual Information Rate (RBIR) ESM

In this section the RBIR ESM PHY abstraction method is described for SIMO/SIMO as well as for MIMO under various receiver configurations.

# RBIR Mapping for a SISO/SIMO System

For a SISO/SIMO system the symbol mutual information (SI) is given by

(1.25)

where  is zero mean complex Gaussian with variance  per component,  SINRn is the post-equalizer SINR at the n-th symbol or sub-carrier and is the number of bits at the n-th symbol (or sub-carrier).

Assuming N sub-carriers are used to transmit a coded block, the normalized mutual information per received bit (RBIR) is given by

(1.26)

The symbol mutual information curves SI(SINR,m) are generated once in the system simulator for each modulation order, and the RBIR values are stored as shown in Table 24 in 0.5dB SINR increments ranging from -20dB to 27dB.

We note that even though we refer to the coded block being carried over a set of sub-carriers, in general, the coded block may be carried over multiple dimensions, including the spatial dimensions available with MIMO.  Also, note that in the above, the mutual information may be computed even with non-uniform modulation across the coded block. RBIR provides a direct relationship to the BLER that is dependent only on the AWGN link performance curves for a given code rate and is independent of the modulation scheme.  This feature is useful in computing the PHY abstraction for cases where the coded block comprises of mixed modulation symbols.

The relationship between the post-equalizer SINR and RBIR is shown in Table 2 for 0.5dB increments in SINR.

|  | QPSK | 16QAM | 64QAM |
| --- | --- | --- | --- |
| SINR Span (dB) | [-20:0.5:27] | [-20:0.5:27] | [-20:0.5:27] |
| RBIR Value | [0.0072 0.0080 0.0090 0.0101 0.0114 0.0127 0.0143 0.0159 0.0179 0.0200 0.0225 0.0251 0.0282 0.0315 0.0352 0.0394 0.0442 0.0493 0.0551 0.0616 0.0688 0.0767 0.0855 0.0953 0.1061 0.1180 0.1311 0.1456 0.1615 0.1788 0.1978 0.2184 0.2407 0.2650 0.2910 0.3190 0.3489 0.3806 0.4141 0.4493 0.4859 0.5239 0.5628 0.6024 0.6422 0.6817 0.7207 0.7584 0.7944 0.8281 0.8592 0.8872 0.9119 0.9331 0.9507 0.9649 0.9760 0.9842 0.9901 0.9942 0.9968 0.9983 0.9992 0.9997 0.9999 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000] | [0.0036 0.0040 0.0045 0.0050 0.0057 0.0063 0.0071 0.0080 0.0089 0.0100 0.0112 0.0126 0.0141 0.0158 0.0176 0.0197 0.0221 0.0247 0.0276 0.0308 0.0344 0.0384 0.0428 0.0476 0.0531 0.0590 0.0656 0.0728 0.0808 0.0895 0.0990 0.1094 0.1206 0.1329 0.1461 0.1603 0.1756 0.1920 0.2094 0.2279 0.2474 0.2680 0.2896 0.3122 0.3357 0.3600 0.3852 0.4112 0.4379 0.4653 0.4933 0.5219 0.5509 0.5804 0.6103 0.6403 0.6709 0.7014 0.7317 0.7617 0.7910 0.8193 0.8463 0.8716 0.8949 0.9158 0.9343 0.9501 0.9633 0.9739 0.9821 0.9883 0.9927 0.9957 0.9976 0.9988 0.9994 0.9997 0.9999 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000 1.0000] | [0.0024 0.0027 0.0030 0.0034 0.0038 0.0043 0.0047 0.0054 0.0060 0.0067 0.0075 0.0084 0.0094 0.0106 0.0117 0.0132 0.0147 0.0165 0.0184 0.0207 0.0229 0.0257 0.0285 0.0319 0.0354 0.0396 0.0437 0.0488 0.0539 0.0599 0.0660 0.0732 0.0805 0.0890 0.0974 0.1073 0.1172 0.1285 0.1398 0.1525 0.1653 0.1795 0.1937 0.2092 0.2247 0.2415 0.2583 0.2763 0.2942 0.3132 0.3321 0.3519 0.3718 0.3924 0.4131 0.4345 0.4558 0.4778 0.4997 0.5223 0.5448 0.5677 0.5907 0.6141 0.6374 0.6611 0.6848 0.7087 0.7325 0.7564 0.7802 0.8036 0.8269 0.8489 0.8708 0.8904 0.9100 0.9262 0.9425 0.9547 0.9668 0.9732 0.9796 0.9840 0.9883 0.9910 0.9937 0.9954 0.9971 0.9983 0.9995 0.9998 1.0000 1.0000 1.0000] |

Table 2: SINR to RBIR Mapping

# RBIR Mapping for a Linear MIMO Receiver

With linear equalizers such as zero-forcing (ZF) and minimum mean-squared error (MMSE), each one of the MIMO streams is treated as an equivalent SISO channel with SINRs given by the post combining SINRs of the linear receiver.  The same procedure is applied to the case of MIMO Matrix A.

# RBIR Mapping for the Maximum-Likelihood (ML) MIMO Receiver

The SI in Equation (1.26) can now be rewritten as

(1.27)

where  is the conditional pdf of the symbol-level log-likelihood ratio (LLR) of the i-th constellation point. The conditional pdf of symbol LLR for an ML receiver can be approximated as Gaussian. Note that RBIR PHY abstraction is based on the fixed relationship between the LLR distribution and BLER. Hence, a representative LLR distribution among M distributions is considered. Further using the numerical integration method of [6], the mutual information per symbol in Equation (1.27) can be approximated as

(1.28)

where the function f1(⋅) is given as f1(x)=loge(1+e−x). From Equation (1.28), it can be seen that only the mean and variance of the LLR are needed to calculate the RBIR metric.

An improved numerical approximation of the SI [8] is given by

SI≈log2(M)−1loge(2)⋅J					(1.29)

where

J=JA+JB2+JA−JB2⋅sign(T−JB);T≈0.65

and

In this section both Vertical and Horizontal encoding are considered. As shown in Table 3, the mean, AVE and the variance, VAR, are computed as a function of an intermediate variable, , defined as

where  d is the minimum distance in the QAM constellation

d=2, for QPSK210, for 16QAM

Hkis the k-th column vector of the channel matrix , and  is the variance of noise plus interference (assuming the interference is also spatially white). In Table 3 the  is quantized to 0.5dB increments  ranging from -20 dB  to 30 dB.

A detailed derivation of the AVE and VAR as shown in Table 3 is given in Appendix Q. A block size of 6 subchannels x 4 OFDMA symbols and the PUSC permutation are used.

| (dB) | [-20:0.5:30] |
| --- | --- |
| AVE | [-0.4016 -0.4123 -0.4233 -0.4344 -0.4457 -0.4571 -0.4687 -0.4804 -0.4922 -0.5041 -0.5160 -0.5279 -0.5397 -0.5515 -0.5631 -0.5745 -0.5856 -0.5962 -0.6065 -0.6161 -0.6249 -0.6329 -0.6399 -0.6456 -0.6499 -0.6524 -0.6530 -0.6513 -0.6470 -0.6396 -0.6287 -0.6139 -0.5944 -0.5697 -0.5391 -0.5018 -0.4567 -0.4031 -0.3396 -0.2650 -0.1780 -0.0770 0.0398 0.1743 0.3286 0.5051 0.7063 0.9352 1.1949 1.4889 1.8211 2.1959 2.6179 3.0926 3.6259 4.2245 4.8961 5.6491 6.4933 7.4396 8.5006 9.6904 11.0251 12.5229 14.2045 16.0930 18.2146 20.5989 23.2784 26.2897 29.6733 33.4750 37.7458 42.5431 47.9314 53.9830 60.7788 68.4100 76.9786 86.5992 97.4004 109.5263 123.1389 138.4197 155.5725 174.8260 196.4366 220.6922 247.9159 278.4700 312.7611 351.2455 394.4351 442.9043 497.2976 558.3381 626.8372 703.7054 789.9640 886.7593 995.3772] |
| VAR | [0.2952 0.3003 0.3055 0.3108 0.3162 0.3218 0.3276 0.3336 0.3400 0.3468 0.3541 0.3620 0.3705 0.3800 0.3904 0.4021 0.4152 0.4301 0.4471 0.4673 0.4887 0.5143 0.5438 0.5779 0.6175 0.6633 0.7164 0.7779 0.8491 0.9316 1.0270 1.1373 1.2645 1.4112 1.5801 1.7741 1.9967 2.2516 2.5430 2.8755 3.2542 3.6849 4.1737 4.7277 5.3548 6.0636 6.8644 7.7686 8.7895 9.9429 11.2474 12.7253 14.4033 16.3140 18.4964 20.9982 23.8761 27.1982 31.0450 35.5109 40.7058 46.7560 53.8056 62.0176 71.5751 82.6815 95.5627 110.4754 127.720 147.6512 170.6826 197.2945 228.0421 263.5665 304.6084 352.0229 406.7979 470.0740 543.1686 627.6030 725.1343 837.7913 967.9172 1118.2180 1291.8186 1492.3277 1723.9127 1991.3863 2300.3061 2657.0904 3069.1507 3545.0462 4094.6610 4729.4092 5462.4720 6309.0710 7286.7839 8415.9098 9719.8896 11225.7930 12964.8798]; |

Table 3: Mean and Variance for Symbol Level LLR

For MIMO systems, the average, AVE and the variance, VAR are scaled as follows for both horizontal and vertical coding:

As seen from the Table 4, the parameter   is referenced based on the channel condition number,  obtained through the eigen value decomposition of the channel as

(1.30)

The parameter  in Table 4 is optimized to minimize the difference between effective SINR and AWGN SINR for every definite BLER. The parameter 'a' has been found to be independent of power delay profiles and MIMO channel models. The search procedure used to obtain the parameter  in Table 4 is described as in Appendix Q. A block size of 6 subchannels x 4 OFDMA symbols and the PUSC permutation are used in the parameter search.

|  |  | QPSK 1/2 | QPSK 3/4 | 16QAM 1/2 | 16QAM 3/4 | 64QAM 1/2 | 64QAM 2/3 | 64QAM 3/4 | 64QAM 5/6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1st Stream | 3.7500 | 2.7000 | 5.0000 | 3.4000 | 5.0000 | 5.0000 | 0.1000 | 2.1000 |
|  | 2nd Stream | 5.0000 | 1.9000 | 5.0000 | 2.3000 | 5.0000 | 5.0000 | 0.1000 | 3.0000 |
|  | 1st Stream | 3.0000 | 4.9000 | 0.4000 | 1.8000 | 2.1000 | 2.1000 | 2.7000 | 0.9000 |
|  | 2nd Stream | 2.9000 | 4.9000 | 0.1000 | 1.5111 | 3.8556 | 2.5000 | 2.5000 | 1.3000 |
|  | 1st Stream | 1.5000 | 1.0000 | 1.3200 | 2.1000 | 0.4300 | 1.1111 | 1.5000 | 5.0000 |
|  | 2nd Stream | 1.5000 | 1.0000 | 2.0000 | 3.1000 | 0.4300 | 1.1000 | 1.6000 | 4.8000 |
|  | 1st Stream | 2.8500 | 1.7000 | 1.0000 | 0.6778 | 5.0000 | 4.1000 | 3.5000 | 3.3000 |
|  | 2nd Stream | 2.4000 | 1.7556 | 1.0000 | 0.8111 | 5.0000 | 2.7000 | 4.5000 | 3.4000 |
|  | 1st Stream | 0.1500 | 0.8000 | 0.1000 | 1.0556 | 0.1000 | 1.1000 | 1.3000 | 1.5000 |
|  | 2nd Stream | 0.4500 | 0.8000 | 0.0444 | 0.9889 | 0.1000 | 1.1889 | 1.2000 | 1.6000 |
|  | 1st Stream | 1.5000 | 1.0000 | 1.1000 | 2.7000 | 2.1000 | 1.4000 | 1.3000 | 1.9000 |
|  | 2nd Stream | 1.5000 | 1.0000 | 2.0000 | 1.4000 | 1.0889 | 1.3778 | 1.9000 | 1.4000 |
|  | 1st Stream | 0.7500 | 0.8667 | 0.3556 | 0.3889 | 0.5000 | 0.5222 | 0.6000 | 0.3000 |
|  | 2nd Stream | 0.7500 | 0.8444 | 0.2778 | 0.2667 | 0.7000 | 0.3000 | 0.4000 | 0.5000 |
|  | 1st Stream | 0.1500 | 0.1000 | 0.9000 | 1.0444 | 0.6889 | 1.0333 | 1.0000 | 1.1000 |
|  | 2nd Stream | 0.1500 | 0.1000 | 1.0000 | 1.5000 | 0.7444 | 1.1111 | 1.1000 | 1.0000 |
|  | 1st Stream | 1.5000 | 1.0000 | 1.0000 | 5.0000 | 5.0000 | 3.5667 | 1.2000 | 1.2000 |
|  | 2nd Stream | 1.5000 | 1.0000 | 1.0000 | 5.0000 | 5.0000 | 4.1222 | 0.1000 | 2.0000 |

Table 4: Values for the parameter

Horizontal Encoding:

For a 2x2 system using MIMO Matrix B and horizontal encoding, the conditional PDF of the symbol LLR output is approximated as Gaussian for each of the two streams.

(1.31)

The numerical approximation in Equation (1.29) can be used to calculate the mutual information per symbol. The RBIR metric is then computed as in equation (1.26).

Vertical Encoding:

For a 2x2 system using MIMO Matrix B and vertical encoding, the distribution of the LLR from an ML receiver can be approximated as a Gaussian mixture. Thus, the PDF of LLR can be expressed as a weighted sum given by

(1.32)

where the parameters  and are given in Table 5. Thus, SI is also a weighted sum of two SI values given by

(1.33)

Note that  and  are computed based on Equation (1.29) and the RBIR metric is computed as in equation (1.26).

|  |  | QPSK 1/2 | QPSK 3/4 | 16QAM 1/2 | 16QAM 3/4 | 64QAM 1/2 | 64QAM 2/3 | 64QAM 3/4 | 64QAM 5/6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | p1 | 0.5000 | 0.5000 | 0.5000 | 0.5000 | 0.5000 | 0.5000 | 0.5000 | 0.5000 |
|  | p2 | 0.5000 | 0.5000 | 0.5000 | 0.5000 | 0.5000 | 0.5000 | 0.5000 | 0.5000 |
|  | p1 | 0.5000 | 0.5000 | 0.5000 | 0.5000 | 0.5000 | 0.5000 | 0.5000 | 0.5000 |
|  | p2 | 0.5000 | 0.5000 | 0.5000 | 0.5000 | 0.5000 | 0.5000 | 0.5000 | 0.5000 |
|  | p1 | 0.5000 | 0.5000 | 0.5000 | 0.5000 | 0.5000 | 0.5000 | 0.5000 | 0.5000 |
|  | p2 | 0.5000 | 0.5000 | 0.5000 | 0.5000 | 0.5000 | 0.5000 | 0.5000 | 0.5000 |
|  | p1 | 0.5088 | 0.5000 | 0.5000 | 0.5000 | 0.5000 | 0.5000 | 0.5000 | 0.4966 |
|  | p2 | 0.4812 | 0.5000 | 0.5000 | 0.5000 | 0.5000 | 0.5000 | 0.5000 | 0.5034 |
|  | p1 | 0.5094 | 0.5000 | 0.4688 | 0.4900 | 0.5200 | 0.4688 | 0.5000 | 0.5000 |
|  | p2 | 0.4906 | 0.5000 | 0.5312 | 0.5100 | 0.4800 | 0.5312 | 0.5000 | 0.5000 |
|  | p1 | 0.5218 | 0.5184 | 0.5625 | 0.5000 | 0.5000 | 0.5156 | 0.5000 | 0.5000 |
|  | p2 | 0.4782 | 0.4816 | 0.4375 | 0.5000 | 0.5000 | 0.4844 | 0.5000 | 0.5000 |
|  | p1 | 0.5000 | 0.5000 | 0.5538 | 0.5200 | 0.5000 | 0.5000 | 0.5222 | 0.5128 |
|  | p2 | 0.5000 | 0.5000 | 0.4462 | 0.4800 | 0.5000 | 0.5000 | 0.4778 | 0.4872 |
|  | p1 | 0.5000 | 0.5000 | 0.5469 | 0.5000 | 0.5178 | 0.5469 | 0.5000 | 0.5000 |
|  | p2 | 0.5000 | 0.5000 | 0.4531 | 0.5000 | 0.4822 | 0.4531 | 0.5000 | 0.5000 |
|  | p1 | 0.4922 | 0.5000 | 0.5312 | 0.5000 | 0.5000 | 0.5312 | 0.5000 | 0.5178 |
|  | p2 | 0.5078 | 0.5000 | 0.4688 | 0.5000 | 0.5000 | 0.4688 | 0.5000 | 0.4822 |

Table 5: Values of  and  for SM with Vertical Encoding.

The parameters  and in Table 5 have been optimized to minimize the difference between effective SINR and AWGN SINR for every definite BLER. The exhaustive procedure used to obtain the parameters  and in Table 5 is described in Appendix Q. A block size of 6 subchannels x 4 OFDMA symbols and the PUSC permutation are used in the parameter search.

# 2) Add one appendix in EVM document for the search procedure of RBIR optimization parameters.

# -----------------------------Begin Appendix  Text ----------------------------------------------------------------------

Appendix Q: Derivations and Details for RBIR Metric

Q.1 Derivation of the AVE and VAR for RBIR

Given the channel matrix and SNR for each subcarrier, the LLR distribution parameter pair (AVE, VAR) can be obtained for MIMO SM 2x2 as specified below.

The mean of the LLR for the 1st stream is

where ‘d’ indicates the minimum distance in QAM constellation, for example, QPSK:  ;  16QAM: ;  64QAM:  and the mean EK1is defined by

where  H1 is the first column vector.

The variance of the LLR for the 1st stream can be written as

where

.

From the above formulae the numerical integral results for (AVE, VAR) are shown in Table 3.

Q.2 Search for Optimal ‘a’ value

The procedure used to obtain the parameter  in [7] can be described as follows:

Step 1: From the AWGN SINR-to-BLER curve, calculate  from the measured BLER.

Step 2: For a particular value of , calculate the RBIR  metric for a given channel matrix ‘H’  and SINR and then compute the effective SINReff value from the SINR to RBIR mapping in Table 2.

Step 3: Repeat the process over different values of  and choose the value of ‘a’ which results in the smallest gap over all values of BLER between the interpolated SNR (step 1) and effective SNR (step 2).

and ∀H which corresponding to a particular range of  and λmindB.

Q.3 Search for Optimal values of  and

Step 1: From the AWGN SINR-to-BLER curve, calculate the  from the measured BLER.

Step 2: Calculate the corresponding RBIR metric over the two streams for a given channel matrix ‘H’ , SINR and parameter ‘a’ determined from Table 4.

Step 3: Calculate the average RBIR metric as a weighted sum of ‘p1’ and ‘p2’ and then calculate the effective SINReff  value using the averaged RBIR from the SINR to RBIR mapping in Table 2.

Step 4: Find the parameters p1 and p2 which result in the smallest gap over all values of BLER between the interpolated SINR (step 1) and effective SNR (step 3).

∀BLERand ∀Hwhich belongs to a particular range of k and λmindB.

Note that the search for parameters p1 and p2 can be simplified by setting p1 + p2 =1.