| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Changes on Final, SDD Text Proposal by AHG Chair for Idle Mode |  |
| Date Submitted | 2008-9-15 |  |
| Source(s) | Mo-Han Fong, Hang Zhang, Robert Novak, Sophie Vrzic, Jun Yuan, Dongsheng Yu, Hosein Nikopourdeilami, Kathiravetpillai Sivanesan Nortel Networks | mhfong@nortel.com |
| Re: | Call for reply comments on 802.16m-08/034r3 |  |
| Abstract | This contribution proposes text changes on the final Draft proposal for SDD text by the Idle Mode AHG Chair |  |
| Purpose | For discussion and adopt into TGm SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text Changes to Final SDD Text proposal on Idle Mode for Project 802.16m

Mo-Han Fong, Hang Zhang, Robert Novak, Sophie Vrzic, Jun Yuan, Dongsheng Yu,

Hosein Nikopourdeilami, Kathiravetpillai Sivanesan

Nortel Networks

# Introduction

This contribution proposes text changes on the final Draft proposal for SDD text by the Idle Mode AHG Chair, i.e., C802.16m-08/1128r1.

# SDD Text Proposal

++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

10.x	Power Management

10.x.1	Sleep Mode operation

[Editor’s Note: Content of this section or section heading will be developed through discussion in Sleep Mode Adhoc Group]

10.x.2	Idle mode

Idle mode is intended as a mechanism to provide efficient power saving for the MS by allowing the MS to become periodically available for DL broadcast traffic messaging (e.g. Paging message) without registration at a specific BS.

MS monitors the paging message at MS’s paging listening interval. The start of the MS’s paging listening interval shall be derived based on paging cycle and paging offset. Paging offset and paging cycle are defined in terms of number of super-frames.

The MSs are divided into logical groups to offer a scalable paging load-balancing distribution. The criteria of load balancing are FFS.

10.x.2.1	Paging Procedure

BS transmits the list of PGIDs at the pre-determined location.

Paging mechanism in 802.16m may use the two-step paging procedure that includes the paging indication followed by the full paging message.

10.x.2.1.1 paging indication

Paging indications, if present, are transmitted at the pre-determined location. When paging indications are transmitted, BS transmits the list of PG IDs and associated paging indicator flag [The exact format of paging indicator is TBD] indicating the presence of full paging messages for the corresponding PGIDs.

10.x.2.1.2 BS Broadcast Paging message

Paging message is transmitted in the pre-determined [frame/sub-frame] during paging listening interval.

Location of transmission of paging message is FFS. Paging message includes identification of the MSs to be notified of DL traffic pending or location update.’

10.x.2.1.3	Operation during paging unavailable interval

BS should not transmit any DL traffic or paging advertisement to MS during MS’s paging unavailable interval. During paging unavailable interval, the MS may power down, scan neighbor BSs, reselect a preferred BS, conduct ranging, or perform other activities for which the MS will not guarantee availability to any BS for DL traffic.

10.x.2.1.4	Operation during paging listening interval

The MS derives the start of the paging listening interval based on the paging cycle and paging offset. At the beginning of paging listening interval, the MS scans and synchronizes on the SCH of its preferred BS. The MS decodes the BCH. The MS shall confirm whether it exists in the same paging group as it has most recently belonged by getting PG ID information.

During paging listening interval, MS monitors BCH. If BCH indicates change in system broadcast information (e.g. change in system configuration count) then MS should acquire the latest system broadcast information at the pre-determined time when the system information is broadcast by the BS.

Additionally, if paging indicators are present, MS also monitors the paging indicators. If the paging indicator flag associated with its own PGID is set then MS will subsequently decode the full paging message at the predetermined [frame/sub-frame] otherwise MS will return to paging unavailable interval.

If paging indicators are not present, MS decodes the full paging message at the predetermined [frame/sub-frame] if this message is present, otherwise MS returns to paging unavailable interval.

10.x.2.2	Idle Mode Entry/Exit Procedure

10.x.2.2.1 Idle mode initiation

In order to reduce signaling overheads and provide location privacy, the paging controller may assign temporary identifier to uniquely identify the MSs in the idle mode in a particular paging group.  The MS’s temporary identifier remains valid as long as MS stays in the same paging group. The temporary identifier assignment may happen during idle mode entry or during location update due to paging group change. Temporary identifier may be used in paging messages or during MS’s network re-entry procedure from idle mode as response to paging.

10.x.2.2.2 Idle mode termination

For termination of idle mode, MS shall perform network reentry with its preferred BS. The network reentry procedure can be shortened by the BS possession of MS information obtained from network management entity.

10.x.2.3	Location Update

10.x.2.3.1 Location update trigger condition

An MS in idle mode shall perform a location update process operation if any of the following location update trigger condition is met.

- Paging group location update

- Timer based location update

- Power down location update

During paging group location update or timer based location update, MS may update paging cycle and paging offset.

10.x.2.3.2 Location update procedure

If an MS determines or elects to update its location, depending on the security association the MS shares with its preferred BS, the MS shall use one of two processes: secure location update process or unsecure location update process.

Location update comprises condition evaluation and location update signaling.

10.x.2.3.2.1 Paging group location update

The MS performs the Location Update process when the MS detects a change in paging group. The MS detects the change of paging group by monitoring the Paging Group IDs, which are transmitted by the BS.

10.x.2.3.2.2 Timer based location update

MS shall periodically perform location update process prior to the expiration of idle mode timer. At every location update including paging group location update, idle mode timer is reset to 0 and restart. The action after expiration of idle mode timer is FFS.

10.x.2.3.2.3 Power down location update

The MS attempts to complete a location update once as part of its orderly power down procedure.

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++