| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Changes on 3rd Draft HO SDD Text (HO AHG) |  |
| Date Submitted | 2008-09-15 |  |
| Source(s) | Mo-Han Fong, Hang Zhang, Sophie Vrzic, Robert Novak, Jun Yuan, Dongsheng Yu, Hosein Nikopourdeilami, Kathiravetpillai Sivanesan Nortel Networks | mhfong@nortel.com |
| Re: | Call for reply comments on 802.16m-08/034r3 |  |
| Abstract | This contribution provides proposed text changes on 3rd draft for HO SDD Text (C802.16m-08/1082r2) |  |
| Purpose | For discussion and adopt into TGm SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text Changes on 3rd Draft HO SDD Text (HO AHG)

Mo-Han Fong, Hang Zhang, Sophie Vrzic, Robert Novak, Jun Yuan, Dongsheng Yu,

Hosein Nikopourdeilami, Kathiravetpillai Sivanesan

Nortel Networks

# Introduction

This contribution proposes text changes on the 3rd draft of HO SDD text from the UMAC HO AHG, i.e. C802.16m-08/1082r2. The proposed text changes intend to clarify some of the description and also remove specific naming of HO messages as details of messaging will be defined in stage 3.

# Text Proposal

[Editor’s Note: the text enclosed by [] and shown in blue are non-consensus text.]

---------------------------------------------------------Start of the text--------------------------------------------------

10.3 Handover

[Editor’s Note: overall description of HO, needs to update the current SDD text]

10.3.1 Network topology acquisition

[Editor’s Note: This section is only related to intra-RAT 16m to 16m HO]

10.3.1.1 Network topology advertisement

[Editor’s Note: This section contains text related to network topology as well as the description of neighbor BS information and the method/procedure as how the information is transmitted by the BS. ]

A BS periodically broadcasts the system information of the neighboring BSs using MOB_NBR-ADV message. The BS formats MOB_NBR-ADV message based on the cell types of neighbor cells, in order to achieve overhead reduction and facilitate scanning priority for MS. MOB_NBR-ADV message includes neighbor 16m BS identifier [as well as sector information]. MOB_NBR-ADV message does not include information of neighbor femto cells. Special handling of neighbor information of femto cell is described in section 10.3.4

. A serving BS may unicast the MOB_NBR-ADV message to a MS.

10.3.1.2 Scanning Procedure

[Editor’s Note: This section contains text related to MS scanning procedure Contribution]

The scanning procedure provides the opportunity for the MS to perform measurement of the neighboring cells for handover decision. The MS may use current or negotiate new unavailable intervals to perform scanning. In addition, the MS may perform scanning procedure during its active cycle without interrupting its communication with the serving BS if the MS supports such capability.

MS selects the scanning candidate BSs by information obtained from MOB_NBR-ADV or information cached in the MS, or instructed by BS. The BS or MS may prioritize the neighbor BSs to be scanned based on various metrics, such as cell type, loading, RSSI and location.

MS measures the selected scanning candidate BSs and reports the measurement result back to the serving BS  The serving BS defines triggering conditions for MS sending scanning report is FFS.

10.3.2 Handover Process

[Editor’s Note: This section is only related to intra-RAT 16m to 16m HO for non-multi-carrier case. This section should contain text related to generic handover procedures]

10.3.2.1 HO Framework

[Editor’s Note: ]

The handover algorithm is a network-controlled, MS-assisted handover. Although handover procedure may be initiated by either MS or BS, the final HO decision and target BS(s) selection are performed by the serving BS or the network. MS may execute the HO as directed by the BS or cancel the HO procedure through HO cancellation message. The network re-entry procedure with the target BS may be optimized by target BS possession of MS information obtained from serving BS over the backbone network.

The handover procedures are divided into three phrases, namely, HO initialization, HO preparation and HO execution. When HO execution is complete, the MS is ready to perform Network re-entry procedures at target BS. In addition, HO cancellation procedure is defined to allow MS cancel a HO procedure. Figure 1x shows a general call flow for handover.

Figure 1x General HO Call Flow

10.3.2.2 HO Procedure

[Editor’s Note: Only Break-Before-Entry HO procedure is included in text below (as proposed by C80216m-MAC-08_065r1). Entry-Before-Break HO needs more discussions before consensus can be reached]

10.3.2.2.1 HO initiation

Handover procedure may be initiated by either MS or BS and controlled by the BS. When handover is initiated by the MS, the serving BS defines the triggers and conditions based on which the MS initiates a handover. When multiple triggers and conditions are defined, the serving BS uses combination of multiple conditions to trigger HO initiation is FFS. A HO initiation control signaling is sent by the originator of the handover to start the HO procedure.

In case of BS initiated HO, HO initiation and HO preparation phases are carried out together.

10.3.2.2.2 HO preparation

During HO preparation phase, the serving BS selects and communicates with target BS(s) for HO. The target BS may obtain MS information from the serving BS via backbone network for HO optimization. Ranging with target BS prior or during HO preparation is FFS. HO ranging with the target BS may be omitted if the MS has valid ranging parameters from the target BS. If HO ranging is required at the target BS, dedicated ranging resource (e.g. code, channel, etc.) at target BS may be reserved for the MS to facilitate non-contention-based HO ranging. Information regarding MS identity (e.g. TEK, STID, FIDs, etc.), may be pre-updated during HO preparation. [Editor note: MS can always request UL BW to transmit data, do we need to mention it here?] Any mismatched system information between MS and the Target BS, if detected, may be provided to the MS by the Serving BS during HO preparation.

HO preparation phase completes when serving BS informs the MS of its handover decision via a HO command control signaling ). The control signaling may include dedicated ranging resource allocation and resource pre-allocations for MS at target BS for optimized network re-entry. The control signaling includes an action time for the MS to handover to the target BS.

10.3.2.2.3 HO execution

At the action time specified in the HO command control signaling, the MS performs network re-entry at the target BS. MS may explicitly indicate to the serving BS the start of network re-entry procedure with the target BS via a HO indicationcontrol signaling. At the action time or upon receiving HO indication message from MS, serving BS stops allocating resources to MS for data transmission.

10.3.2.2.4 HO cancellation

After HO is initiated, the MS may cancel HO at any time by sending a HO cancellation message to the serving BS. After the HO cancellation message is processed, the MS and serving BS resume their normal operation.  The control signaling for HO cancellation is FFS.

10.3.2.3 Network Re-entry

10.3.2.3.1 HO Ranging procedure

[Editor’s Note: Includes all ranging proposals, may need additional subclauses]

Ranging with target BS prior to network re-entry is FFS. When the MS handovers to the target BS, ranging may be omitted if ranging is performed prior to network re-entry and the ranging parameters are valid.

If a dedicated ranging code is assigned to the MS by target BS, the MS transmits the dedicated ranging code to the target BS during network re-entry. If a ranging channel is scheduled by the target BS for handover purpose only, the MS should use that ranging channel in order to avoid excessive multiple access interference. MS authentication through HO ranging is FFS. For HO ranging without MS authentication, upon reception of the dedicated ranging code, the target BS should allocate uplink resources for RNG-REQ message and UL data if needed.

.

10. 3.2.3.2 Network Re-entry Procedure

Network re-entry procedure at the target BS, i.e. capability negotiation, authentication, registration, connection establishment, can be omitted if the target BS contains the necessary context information of the MS. Ranging may if be omitted if ranging is performed prior to network re-entry and the ranging parameters are valid. The mechanism to authenticate the MS at the target BS is FFS. When HO ranging is performed with the target BS, MS sends RNG-REQ upon receiving UL allocation. When RNG-REQ message is received from the MS, the target BS responds with a RNG-RSP. Handover procedure completes with successful transaction of RNG-REQ/RSP message if the rest of network re-entry procedures are omitted.

10.3.4 Handover support for Femto BS

[Editor’s Note: This section is only related to intra-RAT 16m to 16m HO. The text proposed in C80216m-MAC-08_022r2 is included with some modification.]

The network provides the mapping between femto BS and corresponding overlay (macro/micro) BS as well as certain system information such as carrier frequency of the femto cell to MS for supporting handover between (macro/micro) BS and femto BS. MS may cache this information for future handover to the specific femto cell. At the time of handover preparation, the system information of a target femto cell may be unicast or multicast to the MS upon MS request/network trigger or obtained by the MS monitoring the femto cell.

The HO procedure between femto BS and macro BS is FFS.

10.3.5 Handover Process supporting Legacy system

[Editor’s Note: This section is only related to intra-RAT 16e/16m HO.]

10.3.5.1 Network topology acquisition

10.3.5.2 Handover from 16e to 16m

10.3.5.3 Handover from 16m to 16e

[Editor’s Note: This following section shows two different proposals on inter-RAT HO TOC]

[Editor’s Note: Option 1]

[10.3.6 Inter-RAT Handover Procedure

10.3.6.1 Generic inter-RAT HO procedure

10.3.6.1.1 Dual Transmitter/Dual Receiver Support

10.3.6.1.2 Single Transmitter/Dual Receiver Support

10.3.6.1.3 Single Transmitter/Single Receiver Support

10.3.6.2 Enhanced inter-RAT HO procedure

10.3.6.2.1 Fast handover between 802.11 and 802.16m]

[Editor’s Note: Option 2]

[13. Inter-Radio Access Technology Functions

13.1 Inter-Radio Access Technology Handover Procedures

13.1.1 Network topology acquisition

13.1.2 Generic Inter RAT HO Procedures]

----------------------------------------------------------End of the text----------------------------------------------------

# Reference

- IEEE C802.16m-08/003r4, “Project 802.16m System Description Document (SDD)”.