| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |
| --- | --- |
| Title | Symbol structure design for 802.16m - resource blocks and pilots |
| Date Submitted | March 10, 2008 |
| Source(s) | Yuval Lomnitz Yuval.Lomnitz@intel.com Huaning Niu Huaning.Niu@intel.com Jong-kae (JK) Fwu Jong-kae.Fwu@intel.com Sassan Ahmadi Sassan.Ahmadi@intel.com Hujun Yin Hujun.Yin@intel.com Intel Corp. |
| Re: | Call for Contributions on Project 802.16m System Description Document (SDD) |
| Abstract |  |
| Purpose | Discussion and Approval |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |

Symbol structure design for 802.16m - resource blocks and pilots

Yuval Lomnitz, Huaning Niu, Jong-Kae (JK) Fwu, Sassan Ahmadi, Hujun Yin

# Intel Corporation

# Introduction and Background

This contribution proposes a symbol structure design for downlink 802.16m with the main focuses on resource block and pilot pattern designs. Since the symbol structure is a tool to support various PHY functionalities (such as various MIMO modes, interference mitigation, etc), the 16m symbol structure should (1) support broadly potential basic PHY features, (2) based on SRD explicit and derived requirements and trade-offs, and (3) provide flexibility to add future advanced features (FFRs, MIMOs, etc). The key symbol structure design tasks include resource block design and pilot design.

In order to meet the requirements mentioned above, the main features of the proposed symbol structure and resource blocks should include

- Flexible symbol structure to support different modes (Localized+Distributed allocation, FFR, MIMO, BF, IM,  etc)

- Hierarchical symbol structure for dynamic and effective resource allocation.

- Support 16m subframe or shorted subframe

- Conditionally align with 16e AMC subchannels

Similarly, the main features of the proposed pilot design should include

- Maximize spectral efficiency by considering pilot overhead and channel estimation loss

- Support both dedicated (localized subchannel) and common pilots (diversity group) with same structure

- Support different MIMO modes, different number of antennas, and different number of streams

- Pilots have different locations in different cells (“randomization”) to reap the gain of boosting

- Pilot patterns obtained by computer searching/optimization with manual fine-tuning

# Description of the Concept

Please refer to [1] for any further details on the 16m downlink symbol structure general concept, requirements and design.

# Proposed Text for SDD

Insert the following text into SDD Section 11 – Physical Layer [5]

-------------------------------  Text Start  ---------------------------------------------------

11.x: DL Symbol Structure

A cluster is physically contiguous group of M subcarriers by N symbols, with default value of 9 subcarriers by 6 OFDM symbols, i.e. M=9, N=6. A miniBand consists of Nb number (TBD, default Nb =2) of these adjacent basic clusters. The miniBand can be used as basic units for CQI reporting, frequency reuse groups partitions and efficient pilot design. A slot “Resource block (RB)” is logical block of M subcarriers by N symbols unit, with default value of 9 subcarriers by 6 OFDM symbols, i.e. M=9, N=6. The total 54 (M*N) subcarriers in RB include both pilots and data tones. The number of data subcarriers in slot varies with different pilot densities. For slots in distributed allocation the 54 subcarriers are randomly distributed across all available distributed subcarriers with the reuse group.

11.x: DL Subcarrier to Resource Block Mapping

Add “DL RB Allocation Process” figure here (as shown below).

The DL RB allocation process is defined as follows:

(1)  A renumbering sequence (TBD), i.e. “Outer Permutation”, is applied on minibands to maximum spreading and map minibands into a given size of frequency reuse group

(2) The frequency reuse group is partition into localized and distributed groups. The size of each group is flexible and the mapping is uniquely determined by the size of each group.

(3) The localized and distributed groups are further mapped into localized (by direct mapping) and distributed RBs (by “inner permutation”).

(4) BS scheduler can semi-dynamically change the Ratio between localized and distributed RBs for scheduling and diversity gain based on CQI feedbacks

11.x: DL Localized resource block format

Localized subchannels contain subcarriers which are contiguous in frequency. Pilots used for localized subchannels are dedicated and should be precoded/boosted the same way as data.

11.x: DL Distributed resource block format

Distributed subchannels contain subcarriers which are spread across frequency within the frequency reuse group. The subcarriers within the distributed groups are permuted (“inner permutation”) across distributed group to maximize frequency diversity. Common pilots are used in distributed groups and are shared within the group. The distributed group may be precoded in which case the pilots are precoded together with data.

11.x: Pilot patterns

Total of 3 pilot patterns are optimized for efficiency and performance. Pattern A and C should be used for 1 and 2 pilot streams. Pattern B should be used for 4 pilot streams. Boosting values for different streams with different patterns should be used as in Table I.

Pattern A should be used as the common pilot in distributed resource blocks, and in localized resource block when the entire miniBand (2 clusters) is used by the same burst. Pattern C should be used when the localized burst is carried on 1 cluster.

Add “Pilot Patterns” figure here (as shown below).

(a) Pattern A: 6 pattern overlay

(b) Pattern C: 6 pattern overlay

(c) Pattern A 1+2 for 2 pilot stream

(d) Pattern C 1+2 for 2 pilot stream

(d) Pattern B for 4 Tx streams

Add “Pilot boosting” table here (as shown below).

Table I: Boosting value for different patterns and different streams

| Pattern | N streams | Boost value |
| --- | --- | --- |
| A | 1 | 5.6dB |
|  | 2 | 7.4dB |
| B | 4 | 8.2dB |
| C | 1 | 5dB |
|  | 2 | 5dB |

11.x: Combining pilot patterns

Pattern A and C each has 6 single patterns. The single antenna pattern should be used for single stream or combined to create 2 antenna patterns (orthogonal). Different base station should coordinate to choose different combinations of the pilot patterns for randomization.

-------------------------------  Text End  ---------------------------------------------------

# References

[1] IEEE S80216m-08/120.ppt, “Requirements, concept and design of downlink symbol structure for 802.16m”, March 2008.

[2] IEEE C80216m-08/120.doc: “Proposal for IEEE 802.16m Downlink Symbol Structure Concept|”, March2008

[3] IEEE 802.16m-07/002r4, “802.16m System Requirements”

[4] IEEE 802.16m-008/004, “802.16m Evaluation Methodology”

[5] IEEE 802.16m-08/003, “The Draft IEEE 802.16m System Description Document”

[6] Cosovic, I.; Auer, G., "Capacity Achieving Pilot Design for MIMO-OFDM over Time-Varying Frequency-Selective Channels," Communications, 2007. ICC '07. IEEE International Conference on , vol., no., pp.779-784, 24-28 June 2007

[7] Hassibi, B.; Hochwald, B.M., "How much training is needed in multiple-antenna wireless links?," Information Theory, IEEE Transactions on , vol.49, no.4, pp. 951-963, April 2003

[8] Lang Tong; Sadler, B.M.; Min Dong, "Pilot-assisted wireless transmissions: general model, design criteria, and signal processing," Signal Processing Magazine, IEEE , vol.21, no.6, pp. 12-25, Nov. 2004