| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text change on downlink MIMO SDD text |  |
| Date Submitted | 2008-09-17 |  |
| Source(s) | Jerry Pi, Ron Porat, Yong Sun MIMO Rapporteur Group Chairs | zpi@sta.samsung.com rporat@nextwave.com sun@toshiba-trel.com |
| Re: | PHY: MIMO; in response to the TGm Call for Contributions and Comments 802.16m-08/033 for Session 57 |  |
| Abstract | Proposed text change per MIMO RG discussion on 9/17/2008 in Kobe. |  |
| Purpose | To be discussed and adopted by TGm for use in the IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Final Draft: SDD Text on Uplink MIMO

Jerry Pi, Ron Porat, Yong Sun

Rapporteur Group Chairs

Proposed text changes

Page 69, line 2-11:

On a given frequency resource k [size is FFS], the precoding matrix P can be defined using the following equation:

P(k) = W(k), Equation 4

The precoder is composed of two matrices. The first matrix W(k) is an NT  M matrix, where NT is the number of transmit antennas and M is the numbers of streams. The matrix W(k) is selected from a predefined unitary codebook, and changes every u subcarriers. A codebook is a unitary codebook if each of its matrices consists of columns of a unitary matrix. [The detailed unitary codebook, and the parameter u are FFS.]

D(k)=ejθ0k0⋯00ejθ1k⋯0⋮⋮⋱⋮00⋯ejθ(NT−1)k

θi,i= 0, 1, 2, ..., NT-1

Page 69, line 13-14:

For OL SU-MIMO, the following schemes are FFS: 4Tx rate-1 SFBC + Antenna hopping, 4Tx rate-2 Double SFBC + Antenna hopping, 4Tx rate-2 SM + Antenna hopping, 4Tx rate-3 SM + Antenna hopping, 4Tx rate-3 hybrid SM + SFBC + Antenna hopping.

Page 69, line 35 – page 70, line 7:

For the 4Tx rate-1, the output of the precoder is a 4  2 matrix

y=W×z, Equation 9

where W is a 4  2 precoder. Note that W may be frequency dependent as described in section 11.x.2.1.1.

For the 8Tx rate-1, the output of the precoder is a 8  2 matrix

y=W×z, Equation 10

where W is a 8  2 precoder. Note that W may be frequency dependent as described in section 11.x.2.1.1.

Page 70, line 31 – 40:

For the 4Tx rate-2 mode, the output of the precoder is a 4  1 vector

y=W×z, Equation 14

where W is a 4  2 precoder. Note that W may be frequency dependent as described in section 11.x.2.1.1.

For the 8Tx rate-2 mode, the output of the precoder is a 8  1 vector

y=W×z, Equation 15

where W is a 8  2 precoder. Note that W may be frequency dependent as described in section 11.x.2.1.1.

Page 71, line 7 – 16:

For the 4Tx rate-3 mode, the output of the precoder is a 4  1 vector

y=W×z, Equation 18

where W is a 4  3 precoder. Note that W may be frequency dependent as described in section 11.x.2.1.1.

For the 8Tx rate-3 mode, the output of the precoder is a 8  1 vector

y=W×z, Equation 19

where W is a 8  3 precoder. Note that W may be frequency dependent as described in section 11.x.2.1.1.

Page 71, line 28 – 32:

For the 8Tx rate-4 mode, the output of the precoder is a 8  1 vector

y=W×z, Equation 23

where W is a 8  4 precoder. Note that W may be frequency dependent as described in section 11.x.2.1.1.