| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | BS capability of designating different power boosting for different PRUs in each frequency partition |  |
| Date Submitted | 2008-10-29 |  |
| Source(s) | Honghai Zhang, Narayan Prasad NEC Laboratories America, Inc. | Voice: 609-951-2454 E-mail: honghai@nec-labs.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m-08/040 |  |
| Abstract | A BS should be capable of designating different power boosting for different PRUs in each frequency partition. |  |
| Purpose | This is a supporting document for technical comments including figure change. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

BS capability of designating different power boosting for different PRUs in each frequency partition

Honghai Zhang, Narayan Prasad

NEC Laboratories America, Inc.

1. Introduction

This is a supporting document for technical comments with figure change on 802.16m SDD document.

In the second paragraph of Section 20.1.1.2, there are two issues. First, the text is inconsistent with the figure.  The text says the system can adaptively designate different DL power boosting over different PRUs in each frequency partition. However, in Figure 46, all PRUs in each block (B, C, or D) of the 1/3-reuse partition use the same power boosting. The figure does not show that different PRUs in the same block (B, C. or D) may have different DL power boosting.

Second, the system capability ("the system can adaptively designate ...") follows a sentence of describing an example. We think it is more reasonable to state the system capability and it is followed by the example and the figure.

In the second paragraph of Section 20.1.2.2, the system capability ("system adaptively designates different IoT targets for UL power control ...") is stated after an example. We think the system capability should be stated first and it is followed by the example and the figure.

2. Proposed text:

20.1.1.2 (second paragraph)

Each BS should be capable of adaptively designating different DL power boosting over different PRUs in each frequency partition. The power allocation over some PRUs may be higher or lower than the normal level of the frequency partition to which the PRUs belong to, and it should be well coordinated from system-wide consideration.  Figure 46 shows an example on how to integrate FFR with DL power control when dynamic power allocation is applied in frequency partition C. In order to support such dynamic power allocation the BSs should be capable of exchanging resource metrics associated with  some of the PRUs in each frequency partition. The exact definition of such metrics is FFS.

A

B

C

D

A

B

C

A

B

C

D

D

Sector 1

Sector 2

Sector 3

Power spectrum density

Reuse 1

Reuse 1/3

Frequency

This PRU receives higher power than the normal power level in partition C.

This PRU receives lower power than the normal power level in partition C.

Figure 46 Example to integrate FFR and DL power control.

20.1.2.2 (second paragraph)

Each BS should be capable of adaptively designating different IoT targets for UL power control over different PRUs in each frequency partition. An MS assigned for a partition needs to do power control properly considering the target IoT level of other cells for that partition. If the target IoT level of other cells for a partition is low, for example, an MS assigned for that partition should transmit with lower power not to interfere other cell users. If the target IoT level of other cells for a partition is high, then a user assigned for that partition may transmit with a higher power. To control system-wide interference, the BS can adjust the frequency partitions and the corresponding target IoT level in coordination with other BSs. Figure 47 (a,b) shows examples of integration of  FFR with UL power control (Section 11.x).