| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for multi-hop relay simulation test scenario in Relay Evaluation Methodology |  |
| Date Submitted | 2008-11-06 |  |
| Source(s) | Young-il Kim, Perumal Ramesh Babu, Jisoo Park, Khishigjargal ETRI 161 Gajeong-dong Yuseong-gu, Taejeon 305-700, Korea Kyu Ha Lee Samsung Thales San-14, Nongseo-Dong, Giheung-Gu, Yongin , Gyeonggi-Do, Korea 446-712 Suk Chan Kim, Dong Heon Lee Pusan National University Jangjeon-dong, Geumjeong-gu, Busan, 609-735, Korea | Voice: +82 42 860 5399 (Young-il Kim) +82 42 860 1755 (Perumal Ramesh Babu) +82 42 860 5748 (Jisoo Park) +82 42 860 1779 (Khishigjargal) E-mail: yikim@etri.re.kr (Young-il Kim) ramesh.babu@etri.re.kr(Perumal Ramesh Babu) jsp@etri.re.kr (Jisoo Park) khishig@etri.re.kr (Khishigjargal) Voice: +82-31-280-9917 Fax : +82-31-280-1620 E-mail: kyuha.lee@samsung.com Voice: +82 51 510 2380 (Suk Chan Kim) +82 51 515 5190 E-mail: sckim@pusan.ac.kr(Suk Chan Kim) leedheon@pusan.ac.kr(Dong Heon Lee) |
| Re: | Change Request for 16m EMD |  |
| Abstract | Relay evaluation methodology was accepted in #57 meeting. In this evaluation methodology, throughput enhancement scenario was included without the test scenario for cell coverage extension evaluation. In this document we define the scenario for cell coverage extension evaluation. |  |
| Purpose | For consideration and adoption into the 16m EMD document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for multi-hop relay simulation test scenario in Relay Evaluation Methodology

Young-il Kim, Perumal Ramesh Babu, Jisoo Park, Khishigjargal

ETRI

Kyu Ha Lee

Samsung Thales

Suk Chan Kim, Dong Heon Lee

Pusan National University

# Introduction

# Relay evaluation methodology was accepted within 16m EMD in #57 meeting. In this evaluation methodology, test scenario was included for ART(Above Rooftop) RS. The test scenario mainly was focused on the evaluation of throughput enhancement. The goal of Relay is for cell throughput enhancement and cell coverage extension.  Therefore the test scenario for cell coverage extension has to be included. The test scenarios had been evaluated in [2]. In this document we define the scenario for cell coverage extension evaluation.

# Text Proposal

[Insert the following text as section 14 of the 16m EMD document:]

# Test Scenarios

Two basic scenarios with Relay Stations (RS) are defined for the purpose of system level simulations:

- Above Rooftop (ART) RS scenario

  - Throughput enhancement scenario of two relays per sector

  - Coverage extension scenario

  - Artificial Shadow Region (ASR) scenario of two relays per sector

- Below Rooftop (BRT) RS scenario

  - Specific scenario if for further study

# Above Rooftop RS Scenario

# Throughput Enhancement Scenario of Two Relays per Sector

In this scenario two RSs are deployed in each for the evaluation of throughput enhancement. The positions of the relays are determined by the BS-RS distance r and the angle  between the boresight direction of the BS sector antenna and the LOS to the RS (refer to Figure 28). By default the distance r is equal to 3/8 of the site-to-site distance and the angle  is 26º. A value of 30º may optionally be used. Note that specified recommended values of r and  are currently aligned for DL spatial multiplexing of relay links assuming that the BS is equipped with 4 antenna elements and antenna spacing equals 4 wavelengths. The values of r and  for the BS equipped with 2 antenna elements are TBD. The optional parameters are aligned with the case when no beamforming and spatial multiplexing techniques are applied. The defined values for r and  are not obligatory and may be changed for other simulation scenarios, but in this case their values must be specified by the proponents. The particular choice may be justified by specific BS antenna system parameters (i.e., antenna spacing and number of antenna elements) and used signal processing techniques. For instance, the angles may be selected to reduce the amount of interference from neighboring cells or to increase the performance of spatially multiplexed relay links for a given BS antenna configuration.

# Coverage Extension Scenario

For the evaluation of cell coverage extension, RSs should be deployed to configure multi hop link. Detailed test scenario need to be specified by proponent in sufficient detail in order to allow other member to recreate the simulation in order to verify the results.

# Artificial Shadow Region (ASR) Scenario of Two Relays per Sector

This scenario is also for the evaluation of coverage extension. The RS can provide service for users in coverage holes that exist due to deep-shadow fading and in valleys between buildings. Shadow fading values for MSs, located in the ASR, are artificially raised above the threshold, which depends on the shadow fading model. Positions of ASR are determined by the same as the way described in Section 14.1.1.1 and RSs are located in that region. Figure 30 illustrates the deployment scenario with two ASRs and two ART RSs per sector. The determination of radius of ASR is for further study.

Figure 30: Two ASR and ART deployment scenario

------------------------------------------------------------End of the text----------------------------------------------------------

References

[1] R. Srinivasan, J. Zhuang, L. Jalloul, R. Novak, J. Park, “Project 802.16m Evaluation Methodology Document (EMD),” IEEE C80216m-08-004/r3, Oct. 2, 2008.

[2]  D.H. Lee, S.C. Kim, D.C. Park, S.S. Hwang, and Y.-I. Kim "Performance Evaluation of Multi-hop Relay System with Deployment Scenarios," in proc. MIL. Commun. Conf. , San Diego, USA, Nov. 2008.

[3]  IEEE 802.16j-06/013r3, “Multi-hop Relay System Evaluation Methodology [Channel Model and Performance Metric],” IEEE 802.16's Relay Task Group, Feb. 2007.