| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |
| --- | --- |
| Title | DL/UL PHY SDD Text Clarifications For IEEE 802.16m |
| Date Submitted | 2008-11-10 |
| Source(s) | Jong-Kae (JK) Fwu, Minh-Anh Vuong, Furuzan Atay, Yuval Lomnitz, Huaning Niu, Sassan Ahmadi, Hujun Yin Jong-kae.fwu@intel.com Intel Corporation |
| Re: | Call for Contributions on Project 802.16m System Description Document (SDD) RE: TGm SDD: Other (for contributions in support of a comment) |
| Abstract | This contribution introduces a new concept that is capable of significantly reducing the MAC header overhead for small payload applications and thereby significantly increasing the performance and capacity and reducing the end-to-end latency. The proposed scheme schedules, combines, and transmits downlink user data belonging to a number of users having similar channel conditions using a new multi-user MAC header structure. The proposed scheme does not impose any additional complexity on the BS (Base station) scheduler, rather reduces the scheduling delay for delay sensitive user traffic. |
| Purpose | Discussion and Approval |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |

DL/UL PHY SDD Text Clarifications for IEEE 802.16m

Jong-Kae (JK) Fwu, Minh-Anh Vuong,  Furuzan Atay, Yuval Lomnitz, Huaning Niu, Sassan Ahmadi, Hujun Yin

Intel Corporation

# Introduction and Background

The DL/UL PHY structure related terminologies have been confusing to many members.  The main issue with the current SDD text is that people are confused by the terminologies of different resource units (RUs), particularly between (1) LDRU/LLRU (logical localized/distributed resource unit) and (2) (distributed and localized) LRU. Based on various discussion with different members, it is clear that some clarification is needed while minimize the amount of changes to the current SDD texts. Some changes are suggested and summarized below related Section 11.5 and 11.6:

Suggested changes:

- Change the terminology of “logical distributed resource unit (LDRU)” to “distributed resource unit (DRU)”.  Replace “LDRU” to “DRU” across the entire SDD document. Please see SDD text proposal #1 in Section 2.

- Change the terminology of “logical localized resource unit (LLRU)” to “contiguous resource unit (CRU)”.  Replace “LLRU” to “CRU” across the entire SDD document. Please see SDD text proposal #2 in Section 2.

<<Note:>> Contiguous Resource unit has been defined/specified in Section 11.5 & 11.6 as the equivalent RU for localized resource unit (see the text below). Using CRU will avoid the confusions caused between the LLRU & LRU.

<<SDD text Section 1.5>> Each frequency partition can include contiguous (localized) and/or non-contiguous (distributed) physical resource units.

- Replace Figure 31 that illustrates the DL subchannelization process with the updated figure with the new terminologies. Please see SDD text proposal #3 in Section 2.

- Replace Figure 36 that illustrates the UL subchannelization process with the updated figure with the new terminologies. Please see SDD text proposal #4 in Section 2.

- Replace DL “inner permutation” with “subcarrier permutation”.

# Proposed SDD Text

Adopt the following text changes for [1]:

Original SDD text is shown in black. New proposed text is shown in blue. Removed SDD text is shown in strike out red text.

-------------------------------  Text Start  ---------------------------------------------------

////////////// 		  SDD Text Proposal #1  	//////////////////////

<<Change #1 for Proposal #1>>

11.5.1.1 Distributed resource unit

The logical distributed resource unit (LDRU) can be used to achieve frequency diversity gain. The LDRU contains a group of subcarriers which arewill be spread across the distributed resource allocations within a frequency partition.

<<Change #2 for Proposal #1>>

11.6.1.1 Distributed Resource unit

The distributed resource unit (LDRU) can be used to achieve frequency diversity gain. The LDRU contains a group of subcarriers which arewill be spread across the distributed resource allocations.

<<Change #3 for Proposal #1>>

Find and replace all occurrences of the following SDD texts by the proposed replacements:

| No | Current Text | New Text |
| --- | --- | --- |
|  | Logical distributed resource unit | Distributed resource unit |
|  | LDRU | DRU |

////////////// 		  SDD Text Proposal #2  	//////////////////////

<<Change #1 for Proposal #2>>

#### 11.5.1.2 Localized/Contiguous resource unit

The logical localized resource unit, a.k.a. contiguous resource unit (LLCRU) can be used to achieve frequency-selective scheduling gain.

<<Change #2 for Proposal #2>>

11.6.1.2 Localized/Contiguous Resource unit

The logical localized resource unit, a.k.a. contiguous resource unit, (LLCRU) can be used to achieve frequency-selective scheduling gain.

<<Change #3 for Proposal #2>>

Find and replace all occurrences of the following SDD texts by the proposed replacements:

| No | Current Text | New Text |
| --- | --- | --- |
|  | Logical localized resource unit | Contiguous resource unit |
|  | LLRU | CRU |

////////////// 		  SDD Text Proposal #3  	//////////////////////

<< Note: Replace Figure 31 in SDD by the following figure: >>

////////////// 		  SDD Text Proposal #4  	//////////////////////

<< Note: Replace Figure 36 by the following figure: >>

////////////// 		  SDD Text Proposal #5  	//////////////////////

<<Find and replace all occurrences of the following SDD texts in Sections 11.5 by the proposed replacements:>>

| No | Current Text | New Text |
| --- | --- | --- |
| 1 | Inner permutation | Subcarrier permutation |

-------------------------------  Text End  ---------------------------------------------------

# References

[1] IEEE C802.16m-08/003r5, “The Draft IEEE 802.16m System Description Document.”