| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | IEEE 802.16m Downlink OL-SU-MIMO clean-up |  |
| Date Submitted | 2008-10-31 |  |
| Source(s) | Wookbong Lee, Bin-Chul Ihm LG Electronics David Mazzarese, Bruno Clerckx Samsung Electronics Guangjie Li, Hongming Zheng, Senjie Zhang, Yang-Seok Choi Intel Corporation | E-mail: wbong@lge.com, bcihm@lge.com E-mail: d.mazzarese@samsung.com, bruno.clerckx@samsung.com E-mail: guangjie.li@Intel.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | In response to TGm Call for comments 802.16m-08/040 for session 58 |  |
| Abstract | This contribution provides IEEE 802.16m downlink OL-SU-MIMO clean-up. |  |
| Purpose | Discuss and adopt in IEEE 802.16m WG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

IEEE 802.16m Downlink OL-SU-MIMO clean-up

Wookbong Lee, Bin-Chul Ihm

LG Electronics

David Mazzarese, Bruno Clerckx

Samsung Electronics

Guangjie Li, Hongming zheng, Senjie Zhang, Yang-seok choi

Intel Corporation

# Introduction

In this contribution, we modified IEEE 802.16m downlink OL-SU-MIMO text.

In session #57, several decisions have been made. Some clean-up is still necessary to fully reflect these decisions:

- SFBC type of STC shall be output of MIMO encoder

- Up to 8 stream transmission shall be supported in downlink for 8 Tx antenna case

The transmit diversity and spatial multiplexing schemes described in the SDD have been extended in a straightforward manner to the 8 Tx and 8 streams case. These are not new schemes but just a straightforward extension of the matrix size to make the SDD consistent. The antenna hopping schemes are kept as FSS, pending further decision.

The proposed text also considerably simplifies and shortens the description of the transmit diversity and spatial multiplexing schemes without any technical change.

------------------------------------------------------------ Text begins ------------------------------------------------------------

Remedy : Line 11, page 78 to line 31, page 82

Modify texts as follows;

- 11.8.2.1 Single-user MIMO

Single-user MIMO schemes are used to improve per-link performance.

Both open-loop single-user MIMO and closed-loop single-user MIMO are supported for the antenna configurations specified in Section Error: Reference source not found.

For open-loop single-user MIMO, both spatial multiplexing and transmit diversity schemes are supported. Note that in the case of open-loop single-user MIMO, CQI and rank feedback may still be transmitted to assist the base station’s decision of rank adaptation, transmission mode switching, and rate adaptation. Note that CQI, and rank feedback may or may not be frequency dependent.

For closed-loop single-user MIMO, codebook based precoding is supported for both TDD and FDD systems. CQI, PMI, and rank feedback can be transmitted by the mobile station to assist the base station’s scheduling, resource allocation, and rate adaptation decisions. Note that the CQI, PMI, and rank feedback may or may not be frequency dependent.

For closed-loop single-user MIMO, sounding based precoding is supported for TDD systems.

As described in section Error: Reference source not found, the overall structure of MIMO processing has two parts. The first part is the MIMO encoder and second part is the precoder.

The MIMO encoder is a batch processor that operates on M input symbols at a time. The input to the MIMO encoder is represented by an M  1 vector

, Equation 1

where si si is the i-th i-th input symbol within a batch. The output of the MIMO encoder is an NS  NF MIMO STC matrix z = S(x), which serves as the input to the precoder. The output of the MIMO encoder is multiplied by NT  NS precoder, P. The output of the precoder is denoted by a matrix NT  NF matrix

, Equation 2

where yi,j yj,k is the output symbol to be transmitted via the i-th j-th physical antenna on the j-th k-th subcarrier/symbol. Note NF is the number of subcarriers or symbols used to transmit the MIMO signals derived from the input vector x. For open-loop SU-MIMO, the rate of a mode is defined as .

- 11.8.2.1.1 Open-loop SU-MIMO

A number of antenna configurations and transmission rates are supported in open-loop SU-MIMO. Among them, 2Tx, 4Tx, and 8Tx antennas with rate 1 transmission are defined as Transmit Diversity modes. The operation of these modes is specified in Section . The other modes, including 2Tx, 4Tx, and 8Tx antennas with rate 2 transmission, 4Tx and 8Tx antennas with rate 3 transmission, 4Tx and 8Tx antennas with rate 4 transmission, and 8Tx antennas with transmission up to rate 8, are defined as Spatial Multiplexing modes. The operation of these modes is specified in Section . The dimensions of the vectors and matrices for open-loop SU-MIMO are shown in the following table:

| NT | Rate | M | NS | NF |
| --- | --- | --- | --- | --- |
| 2 | 1 | 1 | 1 | 1 |
| 2 | 1 | 2 | 2 | 2 |
| 4 | 1 | 1 | 1 | 1 |
| 4 | 1 | 2 | 2 | 2 |
| 8 | 1 | 1 | 1 | 1 |
| 8 | 1 | 2 | 2 | 2 |
| 2 | 2 | 2 | 2 | 1 |
| 4 | 2 | 2 | 2 | 1 |
| 8 | 2 | 2 | 2 | 1 |
| 4 | 3 | 3 | 3 | 1 |
| 8 | 3 | 3 | 3 | 1 |
| 4 | 4 | 4 | 4 | 1 |
| 8 | 4 | 4 | 4 | 1 |
| 8 | 5 | 5 | 5 | 1 |
| 8 | 6 | 6 | 6 | 1 |
| 8 | 7 | 7 | 7 | 1 |
| 8 | 8 | 8 | 8 | 1 |

Table 1 Matrix dimensions for open-loop SU-MIMO modes

On a given subcarrier k, the precoding matrix P can be defined using the following equation:

P(k) = W(k), Equation 3

The precoder is composed of two matrices. The first matrix W(k) is an NT  NS matrix, where NT is the number of transmit antennas and NS is the numbers of streams. The matrix W(k) is selected from a predefined unitary codebook, and changes every u subcarriers, and/or v OFDM symbols. A codebook is a unitary codebook if each of its matrices consists of columns of a unitary matrix. [The detailed unitary codebook, and the parameter u and v are FFS.]

For OL SU-MIMO, the following schemes are FFS:  4Tx rate-1 SFBC + Antenna hopping, 4Tx rate-2 Double SFBC + Antenna hopping, 4Tx rate-2 SM + Antenna hopping, 4Tx rate-3 SM + Antenna hopping, 4Tx rate-3 hybrid SM + SFBC + Antenna hopping.

- 11.8.2.1.1.1 Transmit Diversity

The following transmit diversity modes are supported for open-loop single-user MIMO:

- 2Tx rate-1: For M = 2, STBC/SFBC, and for M = 1, a rank-1 precoder

- 4Tx rate-1: For M = 2, STBC/SFBC with precoder, and for M = 1, a rank-1 precoder

- 8Tx rate-1: For M = 2, STBC/SFBC with precoder, and for M = 1, a rank-1 precoder

For the transmit diversity modes with M=1, the input to MIMO encoder is x=s1x=s1, and the output of the MIMO encoder is a scalar, zz=xx. Then the output of MIMO encoder is multiplied by NT × 1 matrix W, where W is described in section .

For the transmit diversity modes with M=2, the input to the MIMO encoder is represented a 2 × 1 vector.

, Equation 4

The MIMO encoder generates the SFBC matrix.

, Equation 5

Then the output of the MIMO encoder is multiplied by NT × 2 matrix W, where W is described in section .

- The output of the rank-1 precoder for NT = 2, 4, and 8 Tx antennas is a NT × 1 matrix y=W × z, where W may be frequency and/or time dependent as described in section .

- For the transmit diversity modes with M=2, the input to the MIMO encoder is represented a 2 × 1 vector. The MIMO encoder generates 2Tx SFBC, and then multiplied by NT × 2 matrix as described in section  .

For the transmit diversity modes, the input to the MIMO encoder is represented a 2  1 vector

, Equation 6

The output of the MIMO encoder is a 2  2 matrix

, Equation 7

For the 2Tx rate-1 mode, the output of the precoder is a 2  2 matrix

, Equation 8

For the 4Tx rate-1, the output of the precoder is a 4  2 matrix

, Equation 9

where W is a 4  2 precoder. Note that W may be frequency and/ or time dependent as described in section   .

For the 8Tx rate-1, the output of the precoder is a 8  2 matrix

, Equation 10

where W is a 8  2 precoder. Note that W may be frequency and/ or time dependent as described in section  .

- 11.8.2.1.1.2 Spatial Multiplexing

The following spatial multiplexing modes are supported for open-loop single-user MIMO:

- Rate-2 spatial multiplexing modes:

  - 2Tx rate-2: rate 2 SM

  - 4Tx rate-2: rate 2 SM with precoding

  - 8Tx rate-2: rate 2 SM with precoding

- Rate-3 spatial multiplexing modes:

  - 4Tx rate-3: rate 3 SM with precoding

  - 8Tx rate-3: rate 3 SM with precoding

- Rate-4 spatial multiplexing modes:

  - 4Tx rate-4: rate 4 SM

  - 8Tx rate-4: rate 4 SM with precoding

- Rate-5 spatial multiplexing modes:

  - 8Tx rate-5: rate 5 SM with precoding

- Rate-6 spatial multiplexing modes:

  - 8Tx rate-6: rate 6 SM with precoding

- Rate-7 spatial multiplexing modes:

  - 8Tx rate-7: rate 7 SM with precoding

- Rate-8 spatial multiplexing modes:

  - 8Tx rate-8: rate 8 SM

For the rate-R spatial multiplexing modes, the input and the output of MIMO encoder is represented by an R  1 vector

, Equation 7

Then the output of the MIMO encoder is multiplied by NT × R matrix W, where W is described in section .

For the rate-2 spatial multiplexing modes, the input to the MIMO encoder is represented as a 2  1 vector

,  Equation 11

The output of the MIMO encoder is a 2  1 vector

, Equation 12

For the 2Tx rate-2 mode, the output of the precoder is a 2  1 vector

. Equation 13

For the 4Tx rate-2 mode, the output of the precoder is a 4  1 vector

Equation 14

where W is a 4  2 precoder. Note that W may be frequency and/ or time dependent as described in section .

For the 8Tx rate-2 mode, the output of the precoder is a 8  1 vector

, Equation 15

where W is a 8  2 precoder. Note that W may be frequency and/ or time dependent as described in section .

For the rate-3 spatial multiplexing modes, the input to the MIMO encoder is represented as a 3  1 vector

, Equation 16

The output of the MIMO encoder is a 3  1 vector

, Equation 17

For the 4Tx rate-3 mode, the output of the precoder is a 4  1 vector

, Equation 18

where W is a 4  3 precoder. Note that W may be frequency and/ or time dependent as described in section .

For the 8Tx rate-3 mode, the output of the precoder is a 8  1 vector

, Equation 19

where W is a 8  3 precoder. Note that W may be frequency and/ or time dependent as described in section .

For the rate-4 spatial multiplexing modes, the input to the MIMO encoder is represented as a 41 vector

, Equation 20

The output of the MIMO encoder is a 4  1 vector

, Equation 21

For the 4Tx rate-4 mode, the output of the precoder is a 4  1 vector

, Equation 22

For the 8Tx rate-4 mode, the output of the precoder is a 8  1 vector

, Equation 23

where W is a 8  4 precoder. Note that W may be frequency and/ or time dependent as described in section .

------------------------------------------------------------- Text ends --------------------------------------------------------------