| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Support of mobile-controlled handover |  |
| Date Submitted | 2008-11-12 |  |
| Source(s) | Jaehyuk Jang Baowei Ji Jung Je Son Rakesh Taori Samsung Electronics Inuk Jung Kyujin Park Kiseon Ryu Hangyu Cho Ronny Yongho Kim LG Electronics Xiangying Yang Muthaiah Venkatachalam Intel Corporation Zeng Ke Liu Lina Huawei Technologies Sungcheol Chang Chulsik Yoon ETRI | jack.jang@samsung.com bji@sta.samsung.com jungje.son@samsung.com rakesh.taori@samsung.com cooper@lge.com pkj137@lge.com ksryu@lge.com hgcho@lge.com ronnykim@lge.com xiangying.yang@intel.com muthaiah.venkatachalam@intel.com zengk@huawei.com liulina@huawei.com scchang@etri.re.kr csyoon@etri.re.kr |
| Re: | IEEE 802.16m-08/040 Call for Comments and Contributions on Project 802.16m SDD TGm SDD: Other (for contributions in support of a comment) - MAC |  |
| Abstract | This contribution proposes to support mobile-controlled handover in 802.16m |  |
| Purpose | For discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Support of mobile-controlled handover

Jaehyuk Jang, Baowei Ji, Jung Je Son, Rakesh Taori

Samsung Electronics

Inuk Jung, Kyujin Park, Kiseon Ryu, Hangyu Cho, Ronny Yongho Kim

LG Electronics

Xiangying Yang, Muthaiah Venkatachalam

Intel Corporation

Zeng Ke, Liu Lina

Huawei Technologies

Sungcheol Chang, Chulsik Yoon

ETRI

# Introduction

This contribution proposes modification of current text in order to support mobile-controlled handover in Project 802.16m. Regarding the HO framework in the current version of SDD, only network-controlled HO is included. However, mobile-controlled HO has advantages over network-controlled HO.

Since an MS chooses a proper target BS by itself among the candidates, more flexible and adaptive target BS selection is possible from MS perspective. MS always has the best knowledge of its channel environments, so decision could be made by MS as well. And the size of measurement report can be reduced compared with the network controlled HO because the network does not need to know the whole results of the measurement which an MS has performed.

Even in 802.16e system, both two methods, i.e., network-controlled handover and mobile-controlled handover, are supported, and mobile-controlled handover is one of the primary mode of operation in 802.16e.

Therefore, we suggest considering both network-controlled HO and mobile-controlled HO in Project 802.16m.

# Proposed Text

Modify the section 10.3.2.1 & 10.3.2.2 as follows.

-------------------------------------------------- Start of the text --------------------------------------------------

10.3.2.1 HO Framework

The handover algorithm is a network-controlled, MS-assisted handover. Both network-controlled and mobile-controlled handover are supported. Although hHandover procedure may be initiated by either MS or BS., the final HO decision and target BS(s) selection are performed by the serving BS or the network. MS executes the HO as directed by the BS or indicated by the MS, or cancels the HO procedure through HO cancellation message. The network re-entry procedure with the target BS may be optimized by target BS possession of MS information obtained from serving BS over the backbone network. MS may also maintain communication with serving BS while performing network re-entry at target BS as directed by serving BS. Figure 15 shows a general call flow for handover.

Figure 15 A general call flow for HO

The handover procedures are divided into three phrases, namely, HO initialization, HO preparation and HO execution. When HO execution is complete, the MS is ready to perform Network re-entry procedures at target BS. In addition, HO cancellation procedure is defined to allow MS cancel a HO procedure.

10.3.2.2 HO Procedure

10.3.2.2.1 HO initiation

Handover procedure may be initiated by either MS or BS and controlled by the BS. When handover is initiated by the MS, tThe serving BS defines the triggers and conditions based on which the MS initiates a handover. When multiple triggers and conditions are defined, the serving either MS or BS may use combination of multiple conditions to trigger HO. When HO is initiated by MS, a HO Initiation control signaling is sent by the MS to start the HO procedure. In case of BS initiated HO, HO initiation and HO preparation phases are carried out together.

10.3.2.2.2 HO preparation

During HO preparation phase, the serving BS or the MS selects communicate with target BS(s) for HO. The target BS may obtain MS information from the serving BS via backbone network for HO optimization. Ranging with target BS prior or during HO preparation is FFS. If ranging with target BS not performed prior to or during HO preparation, dedicated ranging resource (e.g. code, channel, etc.) at target BS may be reserved for the MS to facilitate non-contention-based HO ranging. Information regarding MS identity (e.g. TEK, STID, FIDs, etc.), may be pre-updated during HO preparation. Any mismatched system information between MS and the target BS, if detected, may be provided to the MS by the Serving BS during HO preparation.

HO preparation phase completes when serving BS informs the MS of its handover decision via a HO Command control signaling for network-controlled handover or when MS informs the BS of its handover decision via HO indication control signaling for mobile-controlled handover. The control signaling may include dedicate ranging resource allocation and resource pre-allocations for MS at target BS for optimized network re-entry. The control signaling includes an action time for the MS to start network re-entry at the target BS and an indication whether MS should maintain communication with serving BS during network re-entry.

10.3.2.2.3 HO execution

At the action time specified in the HO command control signaling, the MS sends HO indication to the serving BS and performs network re-entry at the target BS. If communication is not maintained between MS and serving BS during network re-entry at the target BS, serving BS stops allocating resources to MS for transmission at action time.

If directed by serving BS via HO Command control signaling, and if supported by MS, the MS performs network re-entry with the target BS at action time while continuously communicates with serving BS. However, the MS stops communication with serving BS after network re-entry at target BS is completed. In addition, MS cannot exchange data with target BS prior to completion of network re-entry. Multiplexing of network re-entry signaling with target BS and communications with serving BS is FFS.

-------------------------------------------------- End of the text --------------------------------------------------