| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on Sleep Mode Operation in IEEE 802.16m SDD |  |
| Date Submitted | 2008-November, 11 |  |
| Source(s) | Xin Qi, Shashikant Maheshwari Nokia Siemens Networks Andrea Bacioccola, Zexian Li Nokia Shantidev Mohanty, Maruti Gupta Intel Corporation Jin Lei Huawei Technologies Jerry Chow ZTE Mamadou Kone ITRI | E-mail: xin.qi@nsn.com shashi.maheshwari@nsn.com andrea.bacioccola@nokia.com shantidev.mohanty@intel.com jinlei60020191@huawei.com jchow@zteusa.com makone@gmail.com |
| Re: | TGm SDD: Other |  |
| Abstract | This contribution proposes modification on Sleep Mode Operation in IEEE 802.16m SDD. |  |
| Purpose | For discussion and adoption by IEEE 802.16m group |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Comments on Sleep Mode Operation in IEEE 802.16m SDD

Xin Qi, Shashikant Maheshwari

Nokia Siemens Networks

Andrea Bacioccola, Zexian Li

Nokia

Shantidev Mohanty, Maruti Gupta

Intel Corporation

Jin Lei

Huawei Technologies

Jerry Chow

ZTE

# 1. Comments on Sleep Mode Operation in IEEE 802.16m SDD [1]

# 802.16e defines three types of power saving class (PSC). Among them, PSC type I is optimized for best-effort (BE) users, where the length of sleep window is increased exponentially. The power saving benefit of 16e PSC type I has been analyzed and proved in a number of academic papers, e.g. in [2]. In 802.16m, we propose to keep this good feature, i.e. the length of sleep window (or sleep cycle) should be able to be increased in a predefined manner (e.g. exponentially, like the 16e PSC type I).

# In section 10.4.1.3 (Sleep Mode Operations) of 16m SDD, there are texts implicitly saying that the sleep cycle is fixed, which rule out 16e PSC type I. We propose to do modifications to remove the constraint.

# 2. Proposed Remedy to SDD text

----------------------------------------------------------------------------------------------------------------------------------------

#### 10.4.1.1 Introduction

Sleep mode is a state in which an MS conducts pre-negotiated periods of absence from the serving BS air interface. Per MS, a single power saving class is managed in order to handle all the active connections of the MS. Sleep mode may be activated when an MS is in the connected state. When Sleep Mode is active, the MS is provided with a series of alternate listening window and sleep windows. The listening window is the time in which MS is available to receive and send signaling and/or data from and to the BS.

The IEEE 802.16m provides a framework for dynamically adjusting the duration of sleep windows and listening windows within a sleep cycle based on changing traffic patterns and HARQ operations. The length of successive sleep cycles may remain constant or may be adaptive based on traffic conditions.

Sleep windows and listening windows can be dynamically adjusted for the purpose of transport data as well as MAC control signaling transmission. MS can send and receive transport data and MAC control signaling without deactivating the sleep mode.

#### 10.4.1.3.3.2 Listening Window Extension

The listening window duration can be dynamically adjusted based on traffic availability or control signaling in MS or BS. The listening window can be extended through explicit signaling or implicit method. The maximum length of the extension is to the end of the current sleep cycle in which case there is no sleep window during the current sleep cycle. There may be no sleep window within a sleep cycle due to extension of the listening window.

----------------------------------------------------------------------------------------------------------------------------------------

# 3. Reference

- IEEE 802.16m-08/003r5, Draft IEEE 802.16m System Description Document, 2008-Oct

- Yang X., "Energy saving mechanism in the IEEE 802.16e wireless MAN," IEEE Communications Letters, vol. 9,  Issue 7,  pp. 595 - 597, July 2005