| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Transmission of Additional Broadcast Information on Traffic Channel |  |
| Date Submitted | 2008-10-31 |  |
| Source(s) | Xin Chang, Hongjie Si, Hongliang Xu, Yunsong Yang Huawei | E-mail: xin.chang@huawei.com |
| Re: | IEEE 802.16m-08/040: Call for Comments and Contributions on Project 802.16m System Description Document (SDD) TGm SDD: Other (Comment on DL CTRL) |  |
| Abstract | This contribution proposes solution for the transmission of additional broadcast information. |  |
| Purpose | For discussion and approval by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Transmission of Additional Broadcast Information on Traffic Channel

Xin Chang, Hongjie Si, Hongliang Xu, Yunsong Yang

Huawei

# Introduction

In current SDD, the content and transmission of additional broadcast information is still FFS. For efficiency and flexibility, such information shall be carried on traffic channel in the format of MAC management message. The indication of the presence of additional broadcast information is FFS.

# Text Proposal for SDD

Modify SDD subsection 11.7.2.5 and 11.7.3 as following.

------------------------------------------------------Start of the Text Proposal---------------------------------------------------

11.7.2.5 Transmission of Additional Broadcast Information on Traffic Channel

Examples of additional broadcast information include system descriptors, neighbor BS information and paging information. The indication of the presence of additional broadcast information is FFS.

MAC management messages shall may be used to transmit additional broadcast information on traffic channel.

11.7.3 Mapping information to DL control channels

| Information | Channel | Location |
| --- | --- | --- |
| Synchronization information | Synchronization Channel (SCH) | FFS |
| Essential system parameters and system configuration information | Primary Broadcast Channel (PBCH) and Secondary Broadcast Channel (SBCH) | Inside of SFH |
| Extended system parameters and system configuration information | FFSAdditional Broadcast Information on Traffic Channel | Outside of SFH |
| Control and signaling for DL notifications | FFSAdditional Broadcast Information on Traffic Channel | FFSOutside of SFH |
| Control and signaling for DL notifications | Unicast Service Control Channel | Outside of SFH |

Table 3 Mapping information to DL control channels

------------------------------------------------------End of the Text Proposal---------------------------------------------------