| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Considerations on Support for Femtocells in 16m |  |
| Date Submitted | 2008-10-31 |  |
| Source(s) | Eric Xia, Xin Chang, Hongjie Si, Changhoi Koo Huawei | E-mail: ckoo@huawei.com |
| Re: | IEEE 802.16m-08/040: Call for Comments and Contributions on Project 802.16m System Description Document (SDD) TGm SDD: Femtocells |  |
| Abstract | This contribution describes some differences between Femto-cell and Macro-cell. Some alternations should be introduced to efficiently support Femto-cell in 16m. |  |
| Purpose | For discussion and approval by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Considerations on Support for Femtocells in 16m

Eric Xia, Xin Chang, Hongjie Si, Changhoi Koo

Huawei

# 1. Introduction

Femtocell has been introduced to improve and categorize the indoor coverage with some visible characteristics and capability in contrast to macro cell.

- The typical coverage of Femtocell : 10cm - 30m

- The number of accommodating users of Femtocell : 6 - 8

- The users in Femtocell are almost static or moving very slowly

Based on above explanation, some changes should be incorporated into the femtocell as followings

- Specific system parameters and configuration for Femtocell should be added on BCH

- Quick paging operation should be enabled for users in Femtocell

- Quick accessing operation should be enabled for users in Femtocell

# 2. Text Proposal

--------------------------------------- Start of the text proposal --------------------------------------------------------

[add following text into IEEE 802.16m-08_003r5]

11.7.1.2 Essential system parameters and system configuration information

……

11.7.1.2.4 Femtocell-specific information

……

11.9.2.4.3 The additional ranging structure for TDD mode is FFS.Ranging Channel for

synchronized mobile stations

……

11.9.2.4.3.3 The additional ranging structure for femtocell.

……

11.9.2.5 Ranging Channel for Synchronized Mobile Stations

……

11.9.2.5.3 The additional ranging structure for femtocell.

……

11.9.2.6 Bandwidth Request Channel

……

11.9.2.6.3 The additional bandwidth request structure for femtocell.

……

----------------------------------------- End of the text proposal --------------------------------------------------------