| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Increasing ranging opportunity by spatial orthogonality |  |
| Date Submitted | 2008-11-05 |  |
| Source(s) | Jianfeng Kang, Shaohua Li, Xin Qi Nokia Siemens Networks Zexian Li Nokia | E-mail: jianfeng.kang@nsn.com |
| Re: | TGm SDD: Other |  |
| Abstract | This contribution proposes SDD text for multi-antenna transmission on uplink ranging channel |  |
| Purpose | Comments to EEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Increasing ranging opportunity by spatial orthogonality

Nokia Siemens Networks, Nokia

# 1 Introduction

# The reference system [1] specifies that initial ranging is a contention-based procedure. It means BS will allocate some time-frequency resources, i.e. ranging channels, to all the users who want to get access to the BS. Each unsynchronized user randomly selects and transmits a PN code on the ranging channel. Different users are recognized by the BS if they use different codes. However, a collision will occur if two (or more) users are transmitting the same ranging code in the same ranging channel. In this case, there would be a contention resolution procedure, during which at least one user fails. The failed users have to wait for another ranging opportunity, which delays the network entry.

# In 802.16e, the ranging code set contains 256 codes, which are the superset of 4 non-overlapped groups, each for different purpose, i.e. initial ranging, periodic ranging, bandwidth request and handover ranging. To decrease the collision probability, BS has to allocate more ranging channels. The consequence is the decreased system spectrum efficiency. Thus there will be a tradeoff between increasing ranging opportunity and keeping still high spectrum efficiency.

# 2 Motivation

# In a wireless communication system, an effective channel consists of three part: RF chain of transmitter ( e.g. including DAC/filter/IF/RF ), radio propagation channel and RF chain of receiver. It is well known that radio propagation channel is reciprocal between two antennas in a TDD system (see for instance [2] and references therein). For a linear time-invariant communication system, it implies that the two directional channels have the same frequency response.

# Although the RF chain of transmitter and receiver at BS and MS are not exactly the same, they do not change the spectrum of the signal. The emitted signal should meet the spectrum mask requirement, so the RF chains are relative flat in the signal spectrum, and they can be regarded as a flat filter. The simulation results in [3] showed that there is no need for UE to do RF calibration.

Thus, in a WiMAX system, this channel reciprocity could be used to improve the ranging performance, especially in MIMO case. Below, a 2x2 MIMO configuration is used as example, i.e. each MS is equipped with two transmit antennas while BS has two receive and transmit antennas. This proposal could be extended to other MIMO configuration directly.

# 3 Increasing ranging opportunity by spatial orthogonality

Error: Reference source not found shows the 802.16m frame structure. The location of ranging channel in uplink is just informative. The preamble is transmitted by BS in every frame. Upon the received preamble, the MS could estimate the downlink channel state information (CSI) . Due to TDD channel reciprocity as mentioned above, this DL CSI can be used as an estimated UL channel profile, i.e. . Here,  denotes the matrix transposition, and the superscript k denotes the k-th subcarrier. For simplicity reason, the superscript k will be omitted in the equations below.

Figure 1	802.16 frame structure

Assume there are two users, each equipped with 2 transmit antennas and BS has 2 (or more) receiving antennas. The received signal vector ( ) on receive antenna 1 and antenna 2 could be modelled as:

where  is the uplink channel from j-th user,  and  are the ranging code and precoding matrix of the j-th user,  is the thermal noise on the k-th receive antenna, j = 1,2, k = 1, 2.

If  is orthogonal to , i.e. , then the signals from two users can be separated by the BS receiver with two antennas. The  and  can be any orthogonal vectors, e.g.  or . Let us assume that  are the orthogonal vectors. To compute , firstly, a user randomly selects an orthogonal vector, say , such that or . Based on the selected , the user calculates  by solving the equation

Thus it can be seen that by using the estimated downlink channel information and doing precoding, the two users transmitting the same ranging code could be separated spatially. From this point of view, the ranging opportunity is doubled and the collision probability decreases by half ideally.

Since in a WiMAX system, there are also users equipped with only one transmit antenna. In this case, the BS will receive the same signal on both antennas. To differentiate the users equipped with one and two transmit antennas, we can partition the ranging codes into two sets. One set of codes are used by 1 Transmit antenna MS, while another set of codes are used by 2 Transmit antenna MS.

# 3 Simulation results

Simulation assumptions are listed in Table 1.

Table 1	Initial ranging simulation assumptions

| Ranging channel | similar to 802.16e, but with localized subcarrier allocation. 1 ranging channel per frame. |
| --- | --- |
| Ranging code | Same as 802.16e, i.e. PN code with length of 144 |
| Ranging code set | 16 codes are used |
| Number of users | 2, 4, 6 and 8 |
| Timing offset | Randomly, 0 ~ 31 samples |
| Antenna configuration | SISO, MIMO 2x2 |
| Orthogonal vector used in case of MIMO | [1 0; 0 1] |
| False alarm rate | 1% |
| Channel estimation | Real channel estimation from DL preamble |
| Channel type | Ped B, 3km/h |
| Measurement | Detection rate vs. Es/No |
| Time spacing between DL preamble and UL ranging channel | 24 OFDM symbols |
| Collision | Users choose the same ranging code in case of SISO, or choose the same ranging code and same orthogonal vector in case of MIMO. |
| Correct detection | If no collision, the ranging code (and orthogonal vector) is correctly detected in case of SISO (and MIMO). |
| Detection rate |  |
| False alarm | The event of detecting a ranging code (and orthogonal vector) when it is not transmitted. |
| False alarm rate |  |

The detection rate of 2 ~ 8 users are shown in the Figure 2 ~ Error: Reference source not found respectively. The needed Es/N0 in dB when evaluating detection rate @ 50% and 60% are summarized in Table 2. The performance gain from our proposal are also listed. Obviously, MIMO brings at least diversity gains compared with SISO case. And as the number of users increases, which resulting in more collisions, more gains are produced since the ranging opportunities are doubled in case of MIMO by spatial orthogonality.

Table 2	Es/N0 when evaluating detection rate @ 50% & 60%, and the performance gain

|  | Detection rate = 50% |  |  | Detection rate = 60% |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| user number | SIMO 1x2 | MIMO 2x2 | Gain [dB] | SIMO 1x2 | MIMO 2x2 | Gain [dB] |
| 2 | -3.2 | -4.4 | 1.2 | -2.25 | -3.5 | 1.25 |
| 4 | -1.8 | -3.8 | 2 | -0.3 | -2.8 | 2.5 |
| 6 | -0.2 | -3.3 | 3.1 | 2.6 | -2.1 | 4.7 |
| 8 | 3 | -2.5 | 5.5 | - | -0.8 | - |

| Figure 2 Detection rate of 2 users, PedB 3km/h | Figure 3 Detection rate of 4 users, PedB 3km/h |
| --- | --- |
| Figure 4 Detection rate of 6 users, PedB 3km/h | Figure 5 Detection rate of 8 users, PedB 3km/h |

# 4 Conclusions

From the above simulation results, it can be concluded that multi-antenna for uplink ranging not only brings diversity gains, but also the ranging opportunity gains. Thus we propose to support multi-antenna in uplink ranging. The detailed proposed text are described below.

# 5 Reference

[1] 80216m-08_003r4, “The Draft IEEE 802.16m System Description Document,” July 29, 2008.

[2] Glenn S. Smith, “A direct derivation of a single-antenna reciprocity relation for the time domain,” IEEE Transactions on Antennas and Propagation, vol. 52, no. 6, pp. 1568–1577, June 2004.

[3]. Jiann-Ching Guey, “Modeling and evaluation of MIMO systems exploiting channel reciprocity in TDD mode”, VTC2004-Fall, vol 6, 2004, no. 12, pp. 4265-4269.

# 6 Proposed Text for SDD

Add a new chapter under 11.9.2.4 and 11.12.3.

-------------------------------------------------------Start of the text------------------------------------------------------------

11.9.2.4.1.2 PHY structure

When multi-antenna transmission is supported by MS, it can be used to increase the ranging opportunity by spatial orthogonality.

11.9.2.4.2.2 PHY structure

When multi-antenna transmission is supported by MS, it can be used to increase the ranging opportunity by spatial orthogonality.

11.12.3 Transmission for Control Channels

11.12.3.1 Transmission for UL ranging channel

When multi-antenna transmission is supported by MS, Precoding can be used for UL ranging channel and contention-based bandwidth request channel for TDD system. The unitary codebook  (TBD) is predefined, where .  is the receive antenna number at BS and  is the number of vectors in . For MS j equipped with multiple transmit antennas, when performing UL ranging or contention-based bandwidth request, a codeword  is randomly selected. And the precoding vector  is calculated according to equation 11.12.3.1-1

|  | (Equation 11.12.3.1-1) |
| --- | --- |

, where  is the UL channel response of MS j.

-------------------------------------------------------End of the text------------------------------------------------------------