| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | IEEE 802.16m CTC Data block sizes |  |
| Date Submitted | 2008-10-31 |  |
| Source(s) | Seunghyun Kang, Sukwoo Lee LG Electronics | Voice: +82-31-450-1918 E-mail: sh_kang@lge.com, sugoo@lge.com |
| Re: | IEEE 802.16m-08/040 – TGm SDD: Other (for contributions in support of a comment) |  |
| Abstract | We propose the CTC data block sizes for IEEE 802.16m system. |  |
| Purpose | Discussion and adoption for 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

IEEE 802.16m CTC Data block sizes

Seunghyun Kang, Sukwoo Lee

LG Electronics

# Introduction

In IEEE 802.16m System Description Document [1], the FEC block sizes are FFS and they should be resolved for each coding scheme. In this contribution, we propose CTC data block sizes which are based on the current SDD text.

# Proposed IEEE 802.16m CTC Data block sizes

In Table 2, there are 141 IEEE 802.16m CTC data block sizes which are defined and regularly increased by using pre-determined block size resolution in Table 1, and they include all the legacy data block sizes. Also, for the tail-biting encoding structure of CTC, the data block sizes which are multiple of 7 have been removed after increasing them with the block size resolution. In addition, the block size resolution has been defined for the limitation on the padding overhead while increasing data block sizes as shown in [2].

Table 1―IEEE 802.16m CTC data block sizes

| Block size range | Block size resolution |
| --- | --- |
| 40 ≤ NEP ≤ 512 | 8 |
| 512 < NEP ≤ 1024 | 16 |
| 1024 < NEP ≤ 2048 | 32 |
| 2048 < NEP | 64 |

Table 2―IEEE 802.16m CTC data block sizes

| Index | NEP | Index | NEP | Index | NEP |
| --- | --- | --- | --- | --- | --- |
| 1 | 48 | 48 | 488 | 95 | 1696 |
| 2 | 64 | 49 | 496 | 96 | 1728 |
| 3 | 72 | 50 | 512 | 97 | 1760 |
| 4 | 80 | 51 | 528 | 98 | 1824 |
| 5 | 88 | 52 | 544 | 99 | 1856 |
| 6 | 96 | 53 | 576 | 100 | 1888 |
| 7 | 104 | 54 | 592 | 101 | 1920 |
| 8 | 120 | 55 | 608 | 102 | 1952 |
| 9 | 128 | 56 | 624 | 103 | 1984 |
| 10 | 136 | 57 | 640 | 104 | 2048 |
| 11 | 144 | 58 | 656 | 105 | 2112 |
| 12 | 152 | 59 | 688 | 106 | 2176 |
| 13 | 160 | 60 | 704 | 107 | 2304 |
| 14 | 176 | 61 | 720 | 108 | 2368 |
| 15 | 184 | 62 | 736 | 109 | 2432 |
| 16 | 192 | 63 | 752 | 110 | 2496 |
| 17 | 200 | 64 | 768 | 111 | 2560 |
| 18 | 208 | 65 | 800 | 112 | 2624 |
| 19 | 216 | 66 | 816 | 113 | 2752 |
| 20 | 232 | 67 | 832 | 114 | 2816 |
| 21 | 240 | 68 | 848 | 115 | 2880 |
| 22 | 248 | 69 | 864 | 116 | 2944 |
| 23 | 256 | 70 | 880 | 117 | 3008 |
| 24 | 264 | 71 | 912 | 118 | 3072 |
| 25 | 272 | 72 | 928 | 119 | 3200 |
| 26 | 288 | 73 | 944 | 120 | 3264 |
| 27 | 296 | 74 | 960 | 121 | 3328 |
| 28 | 304 | 75 | 976 | 122 | 3392 |
| 29 | 312 | 76 | 992 | 123 | 3456 |
| 30 | 320 | 77 | 1024 | 124 | 3520 |
| 31 | 328 | 78 | 1056 | 125 | 3648 |
| 32 | 344 | 79 | 1088 | 126 | 3712 |
| 33 | 352 | 80 | 1152 | 127 | 3776 |
| 34 | 360 | 81 | 1184 | 128 | 3840 |
| 35 | 368 | 82 | 1216 | 129 | 3904 |
| 36 | 376 | 83 | 1248 | 130 | 3968 |
| 37 | 384 | 84 | 1280 | 131 | 4096 |
| 38 | 400 | 85 | 1312 | 132 | 4160 |
| 39 | 408 | 86 | 1376 | 133 | 4224 |
| 40 | 416 | 87 | 1408 | 134 | 4288 |
| 41 | 424 | 88 | 1440 | 135 | 4352 |
| 42 | 432 | 89 | 1472 | 136 | 4416 |
| 43 | 440 | 90 | 1504 | 137 | 4544 |
| 44 | 456 | 91 | 1536 | 138 | 4608 |
| 45 | 464 | 92 | 1600 | 139 | 4672 |
| 46 | 472 | 93 | 1632 | 140 | 4736 |
| 47 | 480 | 94 | 1664 | 141 | 4800 |

# Reference

- IEEE 802.16m-08/003r5, “The Draft IEEE 802.16m System Description Document”

- IEEE 802.16m-08/805r2, “Considerations on CTC data block sizes”

# Text Proposal to SDD

--------------------------------------------------------------- Start of Proposed Text -------------------------------------------------------

# 11.13.1.3 FEC encoding

The FEC block sizes for IEEE 802.16m CTC are FFS listed in Table 8.

Table 8―IEEE 802.16m CTC data block sizes

| Index | NEP | Index | NEP | Index | NEP |
| --- | --- | --- | --- | --- | --- |
| 1 | 48 | 48 | 488 | 95 | 1696 |
| 2 | 64 | 49 | 496 | 96 | 1728 |
| 3 | 72 | 50 | 512 | 97 | 1760 |
| 4 | 80 | 51 | 528 | 98 | 1824 |
| 5 | 88 | 52 | 544 | 99 | 1856 |
| 6 | 96 | 53 | 576 | 100 | 1888 |
| 7 | 104 | 54 | 592 | 101 | 1920 |
| 8 | 120 | 55 | 608 | 102 | 1952 |
| 9 | 128 | 56 | 624 | 103 | 1984 |
| 10 | 136 | 57 | 640 | 104 | 2048 |
| 11 | 144 | 58 | 656 | 105 | 2112 |
| 12 | 152 | 59 | 688 | 106 | 2176 |
| 13 | 160 | 60 | 704 | 107 | 2304 |
| 14 | 176 | 61 | 720 | 108 | 2368 |
| 15 | 184 | 62 | 736 | 109 | 2432 |
| 16 | 192 | 63 | 752 | 110 | 2496 |
| 17 | 200 | 64 | 768 | 111 | 2560 |
| 18 | 208 | 65 | 800 | 112 | 2624 |
| 19 | 216 | 66 | 816 | 113 | 2752 |
| 20 | 232 | 67 | 832 | 114 | 2816 |
| 21 | 240 | 68 | 848 | 115 | 2880 |
| 22 | 248 | 69 | 864 | 116 | 2944 |
| 23 | 256 | 70 | 880 | 117 | 3008 |
| 24 | 264 | 71 | 912 | 118 | 3072 |
| 25 | 272 | 72 | 928 | 119 | 3200 |
| 26 | 288 | 73 | 944 | 120 | 3264 |
| 27 | 296 | 74 | 960 | 121 | 3328 |
| 28 | 304 | 75 | 976 | 122 | 3392 |
| 29 | 312 | 76 | 992 | 123 | 3456 |
| 30 | 320 | 77 | 1024 | 124 | 3520 |
| 31 | 328 | 78 | 1056 | 125 | 3648 |
| 32 | 344 | 79 | 1088 | 126 | 3712 |
| 33 | 352 | 80 | 1152 | 127 | 3776 |
| 34 | 360 | 81 | 1184 | 128 | 3840 |
| 35 | 368 | 82 | 1216 | 129 | 3904 |
| 36 | 376 | 83 | 1248 | 130 | 3968 |
| 37 | 384 | 84 | 1280 | 131 | 4096 |
| 38 | 400 | 85 | 1312 | 132 | 4160 |
| 39 | 408 | 86 | 1376 | 133 | 4224 |
| 40 | 416 | 87 | 1408 | 134 | 4288 |
| 41 | 424 | 88 | 1440 | 135 | 4352 |
| 42 | 432 | 89 | 1472 | 136 | 4416 |
| 43 | 440 | 90 | 1504 | 137 | 4544 |
| 44 | 456 | 91 | 1536 | 138 | 4608 |
| 45 | 464 | 92 | 1600 | 139 | 4672 |
| 46 | 472 | 93 | 1632 | 140 | 4736 |
| 47 | 480 | 94 | 1664 | 141 | 4800 |

--------------------------------------------------------------- End of Proposed Text -------------------------------------------------------