| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification for flow identifier in IEEE 802.16m SDD |  |
| Date Submitted | 2008-10-31 |  |
| Source(s) | Jeongki Kim, Kiseon Ryu, Youngsoo Yuk, and Ronny Yongho Kim LG Electronic Inc. LG R&D Complex, 533 Hogye-1dong, Dongan-gu, Anyang, 431-749, Korea | Voice: +82-31-450-2067 E-mail: chegal@lge.com, ksryu@lge.com, sixs@lge.com, and ronnykim@lge.com |
| Re: | TGm SDD: Other(UMAC) in response to the IEEE 802.16m-08/040, Call for Comments and Contributions on Project 802.16m System Description Document (SDD) |  |
| Abstract | For clarification for the definition of flow identifier in IEEE 802.16m |  |
| Purpose | To be discussed and adopted by TGm for use in the IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification for flow identifier in IEEE 802.16m SDD

Jeongki Kim, Kiseon Ryu, Youngsoo Yuk, and Ronny Yongho Kim

LG Electronics

# Introduction

The functions of MAC management CIDs (primary, basic) in legacy system (IEEE 802.16 Rev2) are as follows:

  - In BR signaling headers (e.g. BR, BR and CINR report, etc.)

    - Notify BS of QoS information for the connection

      - Primary management message or basic management message

  - In all signaling headers except for BR headers

    - There is no function

  - In GMH (Generic MAC header)

    - Indicates whether a MAC payload is a management message payload

    - MS can decode own MPDUs by using CID included in the MAC header when a DL burst is transmitted without CID in the MAP IE.

  - In MAP messages

    - Station identifier

Logical identifier in IEEE 802.16m consists of station identifier and flow identifier and their specific functions are as follows:

  - Station identifier:

    - BS assigns a “Station Identifier” to the MS during network entry.

    - Station identifier uniquely identifies the MS within the domain of the BS.

    - Some specific “Station Identifiers” are reserved, for example, for broadcast, multicast, and ranging.

  - Flow identifier:

    - Each connection is assigned a “flow identifier”.

    - Flow identifier uniquely identifies the connection within the MS.

    - “Flow Identifiers” identify management connections and active transport Service Flows.

# Proposal

In 16m system flow identifiers don’t need to be used to indentify management connections. We can use QoS identifier instead of flow id in BR header to distinguish between primary management connections and basic management connections. We can use a bit flag instead of flow id in GMH to indicate whether the MAC payload is the management message payload.

The advantages of this proposal are as follows:

  - Providing more spaces to indentify services in a MS

  - Bits occupied by flow id in MAC header can be used for other purposes.

    - E.g.) For selective encryption of MAC management messages (EC field)

    - Detailed usage for bits is FFS.

Text Proposal for the 802.16m SDD

============================== Start of Proposed Text =================================

# 10.1.2.2 “Flow Identifier”

Each MS connection is assigned a “Flow Identifier” that uniquely identifies the connection within the MS. “Flow Identifiers” identify management connections and active transport Service Flows. Some specific “Flow Identifiers” may be pre-assigned by default.

============================== End of Proposed Text =================================