| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Femto BS Network Entry Procedure |  |
| Date Submitted | 2008-11-10 |  |
| Source(s) | Kanchei (Ken) Loa, Chun-Yen Hsu, Chiu-Wen Chen, Yi-Hsueh Tsai, Youn-Tai Lee, Chih-Wei Su, Jiun-Je Jian Institute for Information Industry Shiann-Tsong Sheu, Chih-Cheng Yang National Central University (NCU) Yih-Guang Jan, Yang-Han Lee, Ming-Hsueh Chuang, Hsien-Wei Tseng Tamkang University (TKU) Whai-En Chen National Ilan University (NIU) David W. Lin, Ching-Yao Huang, Yung-Lan Tseng National Chiao Tung University (NCTU) | Voice: +886-2-66000100 Fax: +886-2-66061007 loa@iii.org.tw hcy@iii.org.tw |
| Re: | TGm SDD: Femtocells; IEEE 802.16m-08/040: Call for Comments and Contributions on Project 802.16m System Description Document (SDD) |  |
| Abstract | Comment on 802.16m System Description Document |  |
| Purpose | For discussion and approval by IEEE 802.16 TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Femto BS Network Entry Procedure

Kanchei (Ken) Loa, Chun-Yen Hsu, Yi-Hsueh Tsai, Chiu-Wen Chen, Youn-Tai Lee, Chih-Wei Su, Tsung-Yu Tsai, Jiun-Je Jian, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu, Chih-Cheng Yang, Yih-Guang Jan, Yang-Han Lee, Ming-Hsueh Chuang, Hsien-Wei Tseng, Whai-En Chen, David W. Lin, Ching-Yao Huang, Yung-Lan Tseng
III, Coiler, NCU, TKU, NIU, NCTU

Introduction

This contribution proposes network entry procedure for femto BSs.

A femto BS is a low-power base station uses fixed broadband connection for backhaul. Femto BSs are typically fixed or nomadic. Network Access Provider (NAP) is in charge of femto BS management. Femto BSs are mainly targeted for providing network access in indoor or hotspot. In these scenarios a large amount of femto BSs may be deployed in a limited area. A new turned-on femto BS has to perform certain measurement and adjustment over air interface during the network entry procedure to minimize interference.

Femto BS’s network entry consists of two phases. The first phase is the backhaul attachment phase and the second phase is the radio access configuration phase. During the backhaul attachment phase, the fomto BS performs initial scan, establishes backhaul connection, registers to backhaul network, negotiates basic capability and being configured by NAP.

- Initial scan: The femto BS establishes the list of candidate operational channels. The femto BS also collects information relevant to positioning through the air interface if applicable.

- Backhaul link establishment: The femto BS establishes physical link with backhaul network.

- Backhaul network registration: The femto BS identifies itself to register to backhaul network. AAA for the femto BS is performed in this stage.

- Basic capability negotiation and initial configuration: The femto BS negotiates basic capabilities, such as applicable frequency band, maximum transmit power, multicarrier support, and MIMO, with NAP. The femto BS also reports the outcomes of initial scan to NAP. The NAP then assigns initial configurations such as potential operational channels and initial transmit power to the femto BS.

Figure 1 illustrates the flow of the backhaul attachment phase.

During the radio access configuration phase, the femto BS scans its neighbor and determines its location, determine operational channel, configured by NAP, and optimizes the transmission power.

- Neighbor scanning and location determination: In this stage, the femto BS scans the potential operational channel, establishes neighbor list and report the neighbor list to NAP. Location determination is also performed in this stage. Depending on femto BS design and the positioning technique used, neighbor list establishment and location determination may be performed parallel or in sequence.

- Operational channel assignment: Operational channel for the femto BS is assigned during this stage. A femto BS may negotiate with its NAP the optimal operational channel, or the femto BS simply accepts the NAP’s assignment.

- NAP allocates radio resource to the femto BS: The NAP allocates radio resource, such as cell ID, to the femto BS.

- Transmission power optimization: The femto BS determines the optimal transmission power. The procedure for transmission power optimization is FFS

Figure 2 illustrates the flow of the radio access configuration phase.

Figure 1 Backhaul attachment phase

Figure 2 Radio access configuration phase

Proposed Changes:

[Insert subclause 17.x as indicated:]

17.x Femto BS network entry

Femto BS’s network entry consists of two phases. The first phase is the backhaul attachment phase and the second phase is the radio access configuration phase. During the backhaul attachment phase, the fomto BS performs initial scan, establishes backhaul connection, registers to backhaul network, negotiates basic capability and being configured by NAP.

- Initial scan: The femto BS establishes the list of candidate operational channels. The femto BS also collects information relevant to positioning through the air interface if applicable.

- Backhaul link establishment: The femto BS establishes physical link with backhaul network.

- Backhaul network registration: The femto BS identifies itself to register to backhaul network. AAA for the femto BS is performed in this stage.

- Basic capability negotiation and initial configuration: The femto BS negotiates basic capabilities, such as applicable frequency band, maximum transmit power, multicarrier support, and MIMO, with NAP. The femto BS also reports the outcomes of initial scan to NAP. The NAP then assigns initial configurations such as potential operational channels and initial transmit power to the femto BS.

Figure 1 illustrates the flow of the backhaul attachment phase.

During the radio access configuration phase, the femto BS scans its neighbor and determines its location, determine operational channel, configured by NAP, and optimizes the transmission power.

- Neighbor scanning and location determination: In this stage, the femto BS scans the potential operational channel, establishes neighbor list and report the neighbor list to NAP. Location determination is also performed in this stage. Depending on femto BS design and the positioning technique used, neighbor list establishment and location determination may be performed parallel or in sequence.

- Operational channel assignment: Operational channel for the femto BS is assigned during this stage. A femto BS may negotiate with its NAP the optimal operational channel, or the femto BS simply accepts the NAP’s assignment.

- NAP allocates radio resource to the femto BS: The NAP allocates radio resource, such as cell ID, to the femto BS.

- Transmission power optimization: The femto BS determines the optimal transmission power. The procedure for transmission power optimization is FFS

Figure 2 illustrates the flow of the radio access configuration phase.