| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Scanning femtocells for macro-to-femto HO in IEEE 802.16m |  |
| Date Submitted | 2008-11-04 |  |
| Source(s) | Won-Ik Kim, Kwangjae Lim, Sungcheol Chang, Jung-Im Kim, Chulsik Yoon ETRI 138 Gajeongno, Yuseong-gu, Daejeon, KOREA Namgi Kim Kyonggi Univ. 94-6 Yiui-dong, Yeongtong-gu, Suwon, Kyonggi-do, KOREA | Voice: +82-42-860-6471 E-mail: {woniks, csyoon}@etri.re.kr Voice: +82-16-470-7414 E-mail: ngkim@kyonggi.ac.kr |
| Re: | TGm SDD: Femtocells; in response to the TGm Call from Contributions and Comments 802.16m-08/040 for Session 58 |  |
| Abstract | This contribution provides a method that an MS in a macrocell scans neighbor (overlaid) femtocells for macro-to-femto handover |  |
| Purpose | To discuss and adopt the proposed text in the next revision of the 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Scanning femtocells for macro-to-femto HO in IEEE 802.16m

Won-Ik Kim, Kwangjae Lim, Sungcheol Chang, Jung-Im Kim, Chulsik Yoon

ETRI

Namgi Kim

Kyonggi Univ.

# 1. Introduction

This contribution provides a method that an MS in a macrocell scans neighbor (overlaid) femtocells for macro-to-femto handover in IEEE 802.16m. In the macro-femto overlay system, the size of neighbor cell list and the scanning overhead will be drastically increased because a macrocell can include more than hundreds of neighbor femtocells. In such an environment, when an MS in a macrocell is located away from available femto region, it is inefficient that an MS receives overall macro/femto list and scans all femto BSs in the list. The main objective of this proposal is to reduce the MS scanning overhead for searching a target femtocell and to optimize the size of neighbor femto list. In order to achieve this objective, we propose a two-step scanning scheme that an MS in a macrocell can efficiently scan neighbor femto BSs.

# 2. Handover from macrocell to femtocell

# 2.1. Configuration of femto group

IEEE 802.16e defines three types of preamble segments (carrier-set) according to the allocation of different sub-carriers for each one of them. In order to support femtocells more efficiently in IEEE 802.16m, each segment may be divided into several sub-segments and some sub-segments may be dedicated for the preamble of femto BS. In our proposal, femto BSs in a macrocell is divided into several femto groups according to the preamble sub-segment used by femto BSs. This implies that the number of femto groups is equal to the number of preamble sub-segments and all the femto BSs in a femto group have the same sub-segment index. If the number of preamble sub-segments is large, the number of femto group is also large, but the number of femto BSs included in a femto group is small. During self-organization process, a new sub-segment index, which is different from that of overlaid macro BS (or those of neighboring femto BSs), is assigned for a femto BS’s preamble in order to avoid interference between the macrocell and the femtocell. The method for allocating the sub-segment index of preamble of a femto BS is FFS.

# 2.2. Femto network topology acquisition

# 2.2.1 Femto network topology advertisement

A macro BS shall periodically broadcast the information of the neighbor femto groups as well as the neighbor macro BSs using the MOB_NBR-ADV message. The information for neighbor macro BSs contains detailed BS information same as shown in legacy system (802.16e). The information includes the network identification information and the characteristics of neighbor macro BS information for a potential MS to perform initial network entry procedure or macro-to-macro HO. On the other hand, the information for neighbor femto groups is composed of only the preamble sub-segment indices of femto groups. If the macrocell frequency is different from the femtocell frequency(that is, the dedicated carrier femtocell), the frequency indices of femto BSs can be added in the information field for neighbor femto group.

# 2.2.2 Two-step scanning procedure

An MS in a macrocell should scan not only neighbor macro BSs but also neighbor femto BSs before performing of macro-to-femto HO. The MS performs two-step scanning in order to reduce the scanning overhead and the size of neighbor cell list advertised to all MS in a macrocell. Figure 1 shows the procedure of two-step scanning for the femtocell search.

Figure 1. Two-step scanning procedure for macro-to-femto HO

In the first step, an MS in a macrocell periodically scans neighbor femto groups as well as neighbor macro BSs by using the information obtained from MOB_NBR-ADV broadcast message or the information cached in the MS. By scanning neighbor macro BSs based on the measurement from the preamble, the MS tries to find available BSs and determines their suitability as targets for HO. On the other hand, by acquiring the sub-segment index from the femto preamble, the MS is aware of a candidate femto group. The number of neighbor femto groups is possibly much smaller than that of neighbor femto BS. Moreover, the acquisition process to get the sub-segment index from the preamble is simple because it only requires energy detection without cross-correlation.

In the second step, when the MS in a macrocell detects a sub-segment index from a femto preamble, it requests the serving macro BS by sending an unicast message (e.g., MOB_NBR-REQ) to get the information of neighbor femto BSs belonging to the detected femto group. This message includes an MS identifier and a sub-segment index of the femto group. Then, the serving macro BS receiving this message selects the information of femto BSs in the femto group from the overall neighbor femto list. In addition, the neighbor femto list is more optimized by considering femto access type (e.g., public access or private access) or MS’s authority type (e,g., authorized MS or non-authorized MS). And, the serving macro BS provides the optimized neighbor femto list for the MS by sending unicast message (e.g., MOB_NBR-RSP).

# 4. Text Proposal for the 802.16m SDD

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Adopt the following text in the 802.16m System Description Document (SDD)]

10.3.3 Handover support for femto BSs

10.3.3.x Handover from macrocell to femtocell

10.3.3.x.1 Configuration of femto group

Femto BSs in a macrocell is divided into several femto groups.

10.3.3.x.2 Femto network topology acquisition

10.3.x.2.1 Femto network topology advertisement

A macro BS shall periodically broadcast the information of the neighbor femto groups as well as the neighbor macro BSs using the MOB_NBR-ADV message.

10.3.x.2.2 Scanning procedure

An MS in a macrocell periodically scans neighbor femto groups as well as neighbor macro BSs by using the information obtained from MOB_NBR-ADV broadcast message or the information cached in the MS. When the MS selects a candidate femto group, it requests the serving macro BS by using an unicast message (e.g., MOB_NBR-REQ) to get the information of femto BSs belonging to the selected femto group. The serving macro BS receiving this message selects the information of femto BSs in the selected femto group from the overall neighbor femto list. In addition, the information of femto BSs in the selected femto group is more optimized by considering femto access type (e.g., public access or private access) or MS’s authority type (e.g., authorized MS or non-authorized MS). And, the serving macro BS provides the optimized femto list for the MS by sending an unicast message (e.g., MOB_NBR-RSP).

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

[1] IEEE 802.16m-07/002r6, “IEEE 802.16m System Requirements”