| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on MS Idle for Femto Cells in Project 802.16m System Description Document (Draft 5) |  |
| Date Submitted | 2008-10-31 |  |
| Source(s) | Yang Liu, Tricci So, Yuqin Chen, Jerry Chow,Mary Chion, Hongyun Qu ZTE Corporation 706/4, Liantang Pengji Industrial Park, Shenzhen, P.R.China 518004 | Voice: +86-755-26773000-6630 E-mail: liu.yang8@zte.com.cn |
| Re: | [TGm SDD] [Femtocells] IEEE 8022.16m-08/040: Call for Comments and Contributions on Project 802.16m System Description Document (SDD) |  |
| Abstract | This contribution proposes modifications on 16m protocol architecture to enable MS idle for Femtocells. |  |
| Purpose | For acceptance into the SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Comments on MS Idle for Femto Cells in Project 802.16m System Description Document (Draft 5)

Yang Liu, Tricci So, Yuqin Chen, Jerry Chow, Mary Chion, Hongyun Qu

ZTE Corporation

# Introduction

The addition of many Femto cells into the traditional macro-cellular network introduces some new challenges to the traditional way of designing paging groups.  For one, a MS will typically stay within the coverage of one of these cells for a fairly lengthy period due to the femto cell generally locates home area or office area.  In addition, access to a home-area or office-area cell is typically restricted to a small subset of the users of the macro-cellular network. Due to these characteristics of the home-area and office-area cells which differ substantially from the expected characteristics of mobile terminals in the macro-cellular network, the traditional way of assigning cells to paging groups may become ineffective.  Specifically, the following methods of assigning cells to paging groups need to be reconsidered with Femto Cells:

- The assignment of adjacent cells to a paging group

  - This method works well for the macro-cellular network since the MS is expected to be moving and so will generally be traversing adjacent cells

  - Femto Cells tend to be scattered and do not typically provide access to the same set of users; therefore, grouping Femto Cells in general vicinity of each other into a paging group is not effective since paging a user in a cell where it is certain the user cannot be reached is a waste of both backhaul and over-the-air resources

  - The same drawback applies if Femto Cells are grouped in a paging group with macro-cells in the same vicinity.  In addition, it would also be a waste of over-the-air and backhaul resources of the macro-cells to be paged there if the user is currently located in a Femto Cell since the user is typically expected to stay within the coverage area of the Femto Cell for an extended period of time.

- The freedom to assign any cell with another other cell to a paging group

  - This method works fine for the macro-cellular network since a macro-cell provides access to the full set of subscribers to the network

  - Since the Femto Cell generally admits only a small selective of users (i.e. Closed Subscriber Group), including a Femto Cell with macro-cells in the same paging group is inefficient for the Femto Cell since it will receive pages for many users that cannot be reached via the Femto Cell.

  - Similarly, including Femto Cells in the same paging group with other Femto Cells is generally inefficient since they generally support different users and so the cells will receive pages for users that cannot be reached through that cell.

- The use of preamble alone to differentiate between the non-accessible Femto Cell and the Macro Cell, and to identify the appropriate Femto Cell or the Macro Cell for the MS to perform the location update (LU) or to respond to paging

  - This approach may not always be practical because there are only a small limited number of “fixed” preambles to be used.  The mapping between the Cell ID and the Preamble Index could be replicated across different geographical locations, not to mention the additional complexity to plan in advance and to coordinate the assignment of those preambles between the Femto Cells and the Macro Cells.

The challenges described above imply that the traditional techniques for assigning paging groups for Femto Cells require new solutions to be considered.  The proposed solutions are described in this contribution.

# Proposed Changes

[Add 17.x as indicated:]

17 Support For Femto Cells

17.x MS Idle for Femto Cells

The MS idle procedure for a private femto cell follows the definitions in 10.4.2 with underlying updates in paging.

(NOTE: There is a basic requirement to the femto cell of which the access privilege is explicitly configured for a specific group of subscribers.  Such “controlled” access list is referred as “Closed Subscriber Group” (CSG).  A femto cell may also be opened for public access by subscriber who has the same privilege as the macro cell.  In addition, a femto cell may be configured to support only CSG, or may be configured to support both CSG as well as to allow for public access.  For the prior case, the femto cell is referred as CSG-Closed.  For the latter case, the femto cell is considered as CSG-Open.  Similar to the macro cell, there is no need for subscriber configuration at the femto cell to support the public access. The access control is managed by the backend femto cell gateway or server and is out-of-scope of the discussion here.)

- The CSG subscribers of the femto cells are segregated into one or more paging groups that are “reserved” for CSG subscribers in order to indicate to the paging controller (PC) to apply a more intelligent paging operation to the target MS.   Note that, in the case when the femto cell is CSG-Opened, this implies the given femto cell will support minimum of two paging groups – one for the CSG subscribers; and one for the non-CSG subscribers.

- A method of grouping CSG subscribers from different CSG-Closed femto cells into more than one paging group, if required, via the use of geographic information on the location of the femto cells, and defining a geographic boundary for a paging group.   However, the non-CSG subscribers can either be grouped together using the similar strategy as the CSG subscribers to have their own paging group(s), or the non-CSG subscribers can be grouped together with the macro cells.  The latter approach is recommended since it reduces the number of paging groups assigned.

- A multi-level paging procedure of which the first attempt is to page one (i.e. via unicast) or more femto cells (i.e. via broadcast) for a given paging group that serves the MS during the last idle mode re-entry or during the last successful location update (LU) operation. If no response is received from the femto cell(s) for the last serving paging group, then the PC will expand the paging to the neighbor macro cells or other neighbor CSG-Opened femto cells within the same geographic area as the last serving paging group.

In the case of failure attempt for the MS to access an non-accessible femto cell (i.e. CSG-Closed femto cell), in order to provide a more efficient scanning for the MS to select the appropriate new target macro cells and/or CSG-Open femto cells, the current non-accessible femto cell (i.e. CSG-Closed) shall re-direct the MS to the new potential target macro cells and/or CSG-Open femto cell.  Each non-accessible femto cell shall be pre-configured or auto-discovered or auto-configured with a list of neighbor macro cells and/or CSG-Open femto cells parameter (e.g. a set of the preamble indices together with the corresponding cell-ids, and may be even the cell type) to support the MS handover re-direction. By performing the redirection from the current non-accessible femto cell to the accessible macro cell and/or CSG-Open femto cell, the MS can proceed with the desired operation as soon as possible while minimizing the airlink overhead to trial for another target cell which may also fail.  In addition, the redirection can also allow the possibility to apply the load balancing to distribute the MSs across multiple public femto/macro cells.

Finally, once the MS exits from the Idle Mode and is attached to macro cell or CSG-Open femto cell, and if the MS is within the neighborhood of the CSG-Closed femto cell that the MS has the access privilege, dependent on the network policy, it might be more preferable for the MS to be served by the CSG-Closed femto cell.  As the result, the network may initiate the handover procedures to the new target CSG-Closed femto to off-load the macro cell.  However, the network shall prevent the ping-pong HO effect as the result of such decision.