| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Interference Mitigation for Closed User Group Femtocells |  |
| Date Submitted | 2008-10-31 |  |
| Source(s) | Luciano Sarperi, Yanling Lu Fujitsu | E-mail:{luciano.sarperi;yanling.lu}@uk.fujitsu.com |
| Re: | TGm SDD: Femtocells |  |
| Abstract | This contribution proposes inter-BS coordination for interference mitigation for closed user group femtocells. |  |
| Purpose | Discussion and approval by TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Interference Mitigation for Closed User Group Femtocells

Luciano Sarperi and Yanling Lu

Fujitsu

# 1. Introduction

In cellular wireless communication systems the location and configuration of the Base Stations (BSs) is normally carefully planned in order to obtain good coverage and low interference between cells. However, the installation of femto BSs by customers in their own premises can result in significant interference between existing cells and femto cells, since the deployment of femto cells is unplanned. Therefore, it is proposed to actively manage the interference in case of femto BSs deployments.

This contribution focuses on the interference problem which arises in the case of closed user group femto BSs. Support for closed user group femto BSs has been specified in the SRD 802.16m-07/002r6 in Section 9.5. Closed user group femto BSs can only be accessed by a pre-defined set of MSs, usually consisting of the customer’s own MSs. In this case, a severe interference problem can occur when an MS not belonging to the closed user group is operated in the vicinity of the femto BS. Because the MS is not allowed to handover to the femto BS, it has to be served by a more distant macro base station. Therefore, the MS may cause large interference towards the femto BS in the uplink (UL) and receive large interference from the femto BS in the downlink (DL).

This contribution proposes to perform inter-BS coordination between BSs under the network operator’s control and closed user group femto BSs for the purpose of interference reduction/avoidance.

# 2. Interference Mitigation for Closed User Group Femtocells

# 2.1 Interference Scenarios

The interference problem introduced above is shown in Figure 1 for the DL case, where the signal received by MS2 from the macro BS2 has low power due to the high path loss. Therefore, MS2 will experience high interference from the closed user group femto BS1 if both BS1 and BS2 use non-orthogonal radio resources to serve MS1 and MS2.

- Figure 1 DL Interference Scenario

Similarly, in the UL case shown in Figure 2, the MS2 is likely to transmit at high power, due to the high path loss to the macro BS2. Therefore, the closed user group femto BS1 will experience high interference from MS2, if both MS1 and MS2 are assigned non-orthogonal radio resources.

- Figure 2 UL Interference Scenario

# 2.1 Proposed Interference Mitigation

In order to reduce/avoid interference between closed user group femtocells and cells under the network operator’s control, it is proposed that BSs under the network operator’s control can impose scheduling restrictions on closed user group femto BSs (which could include power control information or FFR related information) whenever the interference levels are too high. The use of scheduling restrictions results in interference avoidance, which allows MSs which do not belong to the closed user group to be operated indoors even within a short range from the femto BS.

Related concepts regarding inter-BS coordination for interference mitigation are defined in Section “20.3 Interference Mitigation using Power Control and Scheduling” in the current SDD 802.16m-08/003r5, however, in the case of femto BS, inter-BS coordination using the exchange of scheduling restrictions should be mandatory.

Since the femto BS may be connected to the core network via a DSL link having potentially high latency, real-time interference mitigation signaling via the backbone (e.g. exchanging CSI or PMI information) may not be feasible. Furthermore, the interference mitigation methods should be simple in order to keep the costs of femto BSs low.

The advantages of the proposed interference mitigation approach are:

- Simple interference reduction/avoidance scheme

- Lower complexity than interference reduction methods at the receiver side

- Provides interference protection even when MSs not belonging to the closed user group are operated in close proximity to the femto BS

# 3. Proposed SDD Text

[Insert new subclause]

17  Support for Femtocells

17.X  Interference Mitigation for Femtocells

In order to support interference mitigation for closed user group femtocells, BSs under the network operator’s control can impose scheduling restrictions on closed user group femto BSs over the backbone. The restrictions may include power control information or FFR related information.