| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Self-Organizing Network Operations to support Efficient Multicarrier and Handover |  |
| Date Submitted | 2008-10-31 |  |
| Source(s) | Ranga Reddy D. J. Shyy | ranga.reddy@us.army.mil djshyy@mitre.org |
| Re: | TGm SDD: SON In response to IEEE 802.16m-08/040, Call for Comments and Contributions on Project 802.16m System Description Document (SDD) (2008-10-07) |  |
| Abstract | The document describes SON Operations to support Efficient Multicarrier and Handover in 802.16m |  |
| Purpose | Request consideration of SON operations contained herein for the 802.16m SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Self-Organizing Network Operations to support Efficient Multicarrier and Handover

Ranga Reddy

US ARMY

D. J. Shyy

MITRE

# Introduction

In multi-carrier or single-carrier systems, BS can use backhaul signaling or scanning procedures to find incumbents or other BSs on adjacent carriers.  BS also instructs RS/SS to do perform scanning on primary and adjacent carriers to find incumbents or other BSs as well.  This process is use to develop a diversity set for [cell-edge] RS/SSs in the cell.  Adjacent channel scanning can be driven by configuration of local spectrum policy allocation information.  This diversity set is maintained by inter-BS signaling over back-haul and periodic RS/SS scanning procedures.  The diversity set consists of a list of available neighbor BSs and the carrier they are available on.  In single-carrier operation, the BS will schedule quiet periods for cell-edge RS/SS to perform association and maintain communications with a suitable neighbor BS.  For multi-carrier operation, BSs can negotiate shared use of adjacent carriers for cell-edge RS/SS to operate on or allow for RS/SS to maintain operation on more than carrier simultaneously.  Once cell-edge RS/SS is capable of maintaining communications with 1 or more distinct BSs, they can be selectively used to relay time-sensitive control signaling and data traffic between cells.

# SDD Text proposal

[---------------------------------------------------Start of Text Proposal--------------------------------------------------]

Section x Self Organizing Network (SON)

In multi-carrier or single-carrier systems, BS can use backhaul signaling or scanning procedures to find incumbents or other BSs on adjacent carriers.  BS also instructs RS/SS to do perform scanning on primary and adjacent carriers to find incumbents or other BSs as well.  This process is use to develop a diversity set for [cell-edge] RS/SSs in the cell.  Adjacent channel scanning can be driven by configuration of local spectrum policy allocation information.  This diversity set is maintained by inter-BS signaling over back-haul and periodic RS/SS scanning procedures.  The diversity set consists of a list of available neighbor BSs and the carrier they are available on.  In single-carrier operation, the BS will schedule quiet periods for cell-edge RS/SS to perform association and maintain communications with a suitable neighbor BS.  For multi-carrier operation, BSs can negotiate shared use of adjacent carriers for cell-edge RS/SS to operate on or allow for RS/SS to maintain operation on more than carrier simultaneously.  Once cell-edge RS/SS is capable of maintaining communications with 1 or more distinct BSs, they can be selectively used to relay time-sensitive control signaling and data traffic between cells.

[---------------------------------------------------End of Text Proposal--------------------------------------------------]