| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Precoder comparison for Open-Loop Single-User MIMO |  |
| Date Submitted | 2008-10-31 |  |
| Source(s) | Senjie Zhang Guangjie Li Hongming Zheng Shanshan Zheng Feng Zhou Yang-Seok Choi Minnie Ho Intel Corp. | senjie.zhang@intel.com |
| Re: | The TGm Call for Contributions and Comments 802.16m-08/040 |  |
| Abstract | Precoder comparison for Open-Loop Single-User MIMO |  |
| Purpose | Discussion and Approval in 16m |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at: <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Choice of Precoding Matricesfor Open-Loop Single-User MIMO

Senjie Zhang, Guangjie Li, Hongming Zheng,

Shanshan Zheng, Feng Zhou, Yang-Seok Choi and Minnie Ho

Intel Corporation

# Introduction

In section 11.8.2.1.1 of SDD (IEEE 802.16m-08/003r5), the precoding matrix P for Open-loop SU-MIMO is defined using the following equation:

P(k) = W(k),

where the matrix W(k) is an NT × NS matrix, NT is the number of transmit antennas and NS is the numbers of streams. The matrix W(k) is selected from a predefined unitary codebook. The detailed unitary codebook is FFS.

Currently two types of codebook are discussed in the SDD in TGm: Grassmannian codebook (e.g. 802.16e codebook) and DFT codebook. In closed-loop MIMO, the Grassmannian codebook benefits the low correlation scenario and the DFT codebook benefits the high correlation scenario.

However, in high correlation scenario, DFT codebook leads to performance loss in OL SU MIMO (with distributed resource mapping), because it creates sharp spatial beams for a few subcarriers and loss of power in other subcarriers.

As Fig-1 shown, DFT codebook loses about 1.5dB in high correlation scenario.

Fig-1   802.16e codebook vs. DFT codebook

# Proposed SDD text remedy

-------------------------------------Begin of proposed remedy-------------------------------------

In page 80 line 5~6, add the following sentence as shown:

The detailed unitary codebook, and the parameter u and v are FFS. The unitary codebook can be the same or different from the one used for CL SU MIMO.

-------------------------------------End of proposed remedy -------------------------------------