| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Codebook comparison for OL and CL MIMO |  |
| Date Submitted | 2008-10-31 |  |
| Source(s) | Senjie Zhang Guangjie Li Hongming Zheng Shanshan Zheng Feng Zhou Yang-Seok Choi Minnie Ho Intel Corp. | senjie.zhang@intel.com |
| Re: | The TGm Call for Contributions and Comments 802.16m-08/040 |  |
| Abstract | Precoder comparison for Open-Loop Single-User MIMO |  |
| Purpose | Discussion and Approval in 16m |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at: <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Codebook comparison for OL and CL MIMO

Senjie Zhang, Guangjie Li, Hongming Zheng,

Shanshan Zheng, Feng Zhou, Yang-Seok Choi and Minnie Ho

Intel Corporation

# Codebook selection for OL SU MIMO

In section 11.8.2.1.1 of SDD (IEEE 802.16m-08/003r5), the precoding matrix P for Open-loop SU-MIMO is defined using the following equation:

P(k) = W(k),

where the matrix W(k) is an NT × NS matrix, NT is the number of transmit antennas and NS is the numbers of streams. The matrix W(k) is selected from a predefined unitary codebook. The detailed unitary codebook is FFS.

Currently two types of codebook are discussed in the SDD in TGm: Grassmannian codebook (e.g. 802.16e codebook) and DFT codebook. In closed-loop MIMO, the Grassmannian codebook benefits the low correlation scenario and the DFT codebook benefits the high correlation scenario.

However, in high correlation scenario, DFT codebook leads to performance loss in OL SU MIMO (with distributed resource mapping), because it creates sharp spatial beams for a few subcarriers and loss of power in other subcarriers.

In the simulation, the basic assumption are

- Tone based DRU are assumed,

- AoD -60~60, random select for each snapshot.

3) DFT matrix: 4x2 precoder matrix selected from 1,3 or 2,4 vector pair of pure 4x4 DFT Matrix.

4) Non-ideal CE.

5) 120km, vehical A channel with high correlation and low correlation.

As Fig-1 shown, DFT codebook loses about 1.5dB in high correlation scenario.

Fig-1   802.16e codebook vs. DFT codebook

# PAPR comparison for codebook based precoding in DL MIMO

Precoding/beamforming in DL seems to introduce PAPR problem for DL MIMO. And different codebook will have different PAPR, but how serious the problem has not been addressed clearly.

In this contribution, the evaluation of PAPR from different codebook are introduced, and shows that the PAPR from precoding in DL MIMO can be ignored, and PAPR should not be treated as one of the criteria for codebook selection.

DFT codebook is constant module, which seems has smaller PAPR than 16e based codebook. However the evalution shows the difference is smaller than 0.1dB, which can be ignored.

The PAPR definition is

In each Tx antenna, in time domain, the peak power within each OFDM symbol divided by the average power (across enough long time) is calculated.

The CDF is drawn for all the samples (each Tx antenna and each OFDM symbol). 95% and 99% CDF of the PAPR are used for comparison.

In the evaluation, 10MHz system, 48PRU are used, and 4 continuous PRU use one precoder, the precoder and MCS are from the SLS scheduler (MIMO schemes is MUZF). Random data is generated for modulation and precoding, and then IFFT the frequency domain signal to time domain.

In figure 2, the PAPR distribution is shown, and open loop MIMO has the lowest PAPR, while 16e codebook based MUZF precoding has highest PAPR, however the gap is very small, only 0.2 dB, which will not has any impact on the PA design and performance.

In figure 3, the distribution of the power of time domain samples are shown, where the average power is 1.

According to the result, even with 8dB back-off, there are only 0.2% samples will be clipped.

(1) CDF of PAPR

Figure 2, CDF of PAPR for different codebook based precoding

| PAPR | 95% of CDF |  | 99% of CDF |  |
| --- | --- | --- | --- | --- |
| MUOL | 9.97 |  | 10.6 |  |
| Scenario | Uncorrelated | High Correlated | Uncorrelated | High Correlated |
| MUZF 16e (6bits) | 10.22dB | 10.09 dB | 10.93 dB | 10.77 dB |
| MUZF DFT (4bits) | 10.17 dB | 10.03 dB | 10.89 dB | 10.71 dB |
| MUZF Transformed 16e (6.23bits) | 10.20 dB | 10.02 dB | 10.93 dB | 10.71 dB |

(2) The power distribution of time domain samples.

Figure 3 The power distribution of time domain samples

| The ratio of clipping sample | 8dB clipping | 10dB clipping |
| --- | --- | --- |
| Open-loop | 0.0018 | 4.6E-5 |
| MUZF DFT codebook | 0.0023 | 8.8E-5 |
| MUZF 16e codebook | 0.0024 | 9.9E-5 |

# Codebook performance evaluation in SLS

In this section, DFT based codebook [1] and 16e codebook are compared in SLS for CL SU MIMO and MU ZF MIMO. Transformation method [2] is also evaluated.

Both SU and MU result show 16e codebook outperform DFT codebook in uncorrelated and low correlated channel, while in highly correlated channel, DFT is better.

16e codebook with transformation method can show the best performance in any scenarios.

4x2 antenna configuration is used.

The result is shown in figures and listed in the following tables

(1)  Tx 4 SUCL

| Antenna | SUCL Codebook Gain (%) | Uncorrelated Channel (Zero correlation) | Lowly Correlated Channel (4Lambda 3degree) | Highly Correlated Channel (0.5Lambda 3degree) |
| --- | --- | --- | --- | --- |
| [4Tx 2Rx] (bits/sub-band/user/frame) | 802.16e (6 bits) | 0 | 0 | 0 |
|  | DFT (4 bits) | -3.42% | -1.44% | 3.45% |
|  | 3bits 16e Transform (3.23* bits) | -4.76% | 0.14% | 4.14% |
|  | 6bits 16e Transform (6.23* bits) | 2.38% | 3.02% | 4.41% |

(2) Tx 4 MUZF

| Antenna | MUZF Codebook Gain (%) | Uncorrelated Channel (Zero correlation) | Lowly Correlated Channel (4Lambda 3degree) | Highly Correlated Channel (0.5Lambda 3degree) |
| --- | --- | --- | --- | --- |
| [4Tx 2Rx] (bits/sub-band/user/frame) | 802.16e (6 bits) | 0 | 0 | 0 |
|  | DFT (4 bits) | -4.56% | 8.70% | 25.49% |
|  | 3bits 16e Transform (3.23 bits) | -5.03% | 9.42% | 39.53% |
|  | 6bits 16e Transform (6.23 bits) | 2.67% | 14.78% | 48.24% |

(3). Rican Channel SLS of MUZF

| Antenna | MUZF Codebook Gain (%) | Lowly Correlated Channel (4Lambda 3degree) |
| --- | --- | --- |
| [4Tx 2Rx] (bits/sub-band/user/frame) | 802.16e (6 bits) | 0 |
|  | DFT (4 bits) | 4.35% |
|  | 6bits 16e Transform (6.23 bits) | 13.04% |

The performance of DFT drops in Rican channel. The gain of DFT codebook compared to 16e codebook is less than Rayleigh channel.

# Nested structure

The codebook with nested structure means the higher rank codebook cover the information of lower rank codebook, and lower rank codebook can be obtained from higher rank codebook.

The property seems can reduce the complexity for rank search, however the computation complexity of rank search and codebook search is trivial compared with MIMO detection block, because

- The rank/codebook search work on midamble, which is less frequent than data, and the equation is much simpler than MIMO detection module.

- The rank search for rank adaptation is not so frequent as codebook search, for example every frame MS will check the rank adaptation, however every subframe, codebook search is necessary.

Nested structure is not efficient for the codebook design compared with flat structure. For example, with nested structure, rank 2 PMI index contain the rank 1 PMI information, while MS most time don’t need the information of both PMIs (only one CQI is reported ), which introduce the waste of information. Meanwhile, the flat structure will not have such waste.

Nested structure is not necessary for the consideration of codebook.

# Conclusion

The performance and complexity is most important for codebook design and comparison, and the following issue should be considered

- Codebook design should consider the impact on OL SU MIMO, and DFT codebook will degrade the performance of OL SU MIMO.

- Constant module property can be ignored when design codebook.

- Nested structure is not necessary for codebook design

Performance advantage of 16e and DFT is mixed.

The transformation method can improve the codebook performance.

# SLS Simulation parameters

| Parameter | Assumption |
| --- | --- |
| OFDM parameters | 10 MHz (1024 subcarriers) |
| Number of OFDM symbols per subframe | 6 |
| Permutation | Localized |
| Number of total RU in one subframe | 48 |
| Number of RU for PMI and CQI calculation | 4 which is same as in IEEE 802.16e |
| CQI, PMI feedback period | Every 1 frame (5ms) |
| Feedback delay | 1 frame (5ms) |
| Link Adaptation | QPSK 1/2 with repetition 1/2/4/6, QPSK 3/4, 16QAM 1/2, 16QAM 3/4, 64QAM 1/2, 64QAM 2/3, 64QAM 3/4, 64QAM 5/6 |
| MIMO receiver | Linear Minimum Mean Squared Error (LMMSE) |
| Data Channel Estimation | Perfect data channel estimation. |
| Feedback Channel Measurement | Perfect feedback channel measurement. |
| Cellular Layout | Hexagonal grid, 19 cell sites, wrap-around, 3 sectors per site |
| Distance-dependent path loss | L=130.19 + 37.6log10(.R), R in kilometers |
| Inter site distance | 1.5km |
| Shadowing standard deviation | 8 dB |
| Antenna pattern (horizontal) (For 3-sector cell sites with fixed antenna patterns) | Aθ=−min12θθ3dB2,Am θ3dB = 70 degrees, Am = 20 dB |
| Users per sector | 10 |
| Scheduling Criterion | Proportional Fair |

# Appendix

- C802.16m- 08/1187, Samsung, “Evaluation of CL SU and MU-MIMO codebooks.”

[2] C802.16m-08_1345r1 “Transformation method for codebook based precoding”

# Proposed SDD text remedy

-------------------------------------Begin of proposed remedy-------------------------------------

In page 80 line 5~6, add the following sentence as shown:

[The detailed unitary codebook, and the parameter u and v are FFS. The unitary codebook could be the same or different from the one used for CL SU MIMO.]

-------------------------------------End of proposed remedy -------------------------------------