| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on Relay Frame Structure in IEEE 802.16m SDD |  |
| Date Submitted | 2008-10-31 |  |
| Source(s) | Adrian Agustin, Josep Vidal, Olga Muñoz Dep. of Signal Theory and Comm. Technical University of Catalonia (UPC) Campus Nord, Jordi Girona 1-3 08034 Barcelona, SPAIN | Voice: +34 93 401 09 64 E-mail: adrian.agustin@upc.edu josep.vidal@upc.edu olga.munoz@upc.edu |
| Re: | 802.16m-08/040, “Call for Contributions and Comments on Project 802.16m System Description Document (SDD) Call for comments on the material IEEE 802.16m-08/003r5 Section 11.4.4 Relay Support in Frame Structure Ballot id: tgmsdd |  |
| Abstract | Proposition of a flexible frame structure which subsumes both options considered in IEEE 802.16m-08/003r5 and accommodates different relay-assisted transmission techniques with half-duplex relays |  |
| Purpose | For IEEE 802.16m discussion and eventual adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on relay frame Structure in IEEE 802.16m SDD

A.Agustin, J.Vidal, O.Muñoz

Dept. of Signal Theory and Communications
Technical University of Catalonia (UPC)

# Introduction

The introduction of half-duplex relays motivates that the relay-assisted transmission must be carried out in two orthogonal phases: a phase where the relay is receiving data and another phase where the relay is transmitting data.

In a three-terminal network which consists of terminals T1, T2 and T3 (a relay station), several protocols and transmission schemes can be defined depending on the transmission mode of the relay station and which terminals are active in each phase of the transmission. In a cellular network, T1 and T2 may describe a base station (BS) or a mobile station (MS), while T3 is a RS.

The relay may perform a unidirectional transmission (One-Way Relay Channel, OWRC) to the MS or a bidirectional transmission (Two-Way Relay Channel, TWRC) to BS and MS.

Figure 1. Protocols for relay-assisted transmission with unidirectional relay (OWRC). T1, T2 are related to BS, MS, while T3 is the RS, respectively

Figure 1 depicts four protocols defined for the relay-assisted transmission with unidirectional relay: forwarding, protocol I, protocol II and protocol III. The transmission is done over two time slots.

- OWRC-Forwarding.  T1 transmits to T3 in the first time slot (dashed lines). Afterwards, T3 transmits to T2 (solid lines).

- OWRC-Protocol I. T1 transmits to T3 and T2 in the first time slot (dashed lines). Afterwards, T3 transmits to T2 (solid lines).

- OWRC-Protocol II. First, T1 transmits to T3 (dashed lines). In the second time slot, T3 and T1 transmit to T2 (solid lines).

- OWRC-Protocol III. T1 transmits to T3 and T2 in the first time slot (dashed lines). In the second time slot, T1 and T3 transmit to T2 (solid lines).

Protocol I, II and III take into account the T1-T2 link. However, if that link presents a bad quality, the performance of those protocols tend to the performance obtained by the forwarding protocol. Although, the most efficient protocol is protocol III, it also presents the highest complexity. The other protocols also are interesting because they present a lower complexity. For instance protocol II can be easily used for the uplink (UL) transmission (T1 denotes de MS and T2 is the BS), while protocol I for the downlink (DL) transmission (T1 stands for the BS and T2 is the MS),

Figure 2. Two-way Relay Channel (TWRC). Top: Forwarding protocol. Bottom: protocol I.

On the other hand for bidirectional relays (TWRC), two protocols are devised: forwarding and protocol I, both sketched in Figure 2. For the forwarding protocol the transmission is done in two time slots, while protocol I needs of three time slots.

- TWRC-forwarding. First, T1 and T2 transmit to T3 (dashed lines). In the second time slot, T3 transmits to T1 and T2 (solid lines).

- TWRC-protocol I. First, T1 transmits to T3 and T2 (dashed lines). Afterwards, T2 transmits to T3 and T1 (dotted lines). In the third time slot, T3 transmits to T1 and T2 (solid lines).

When those protocols are applied in a cellular network, T1 and T2 denote the BS and MS, respectively.

The evaluation of the previous techniques is carried out in a scenario where both terminals (T1 and T2) are separated by a normalized distance equal to one. The noise power at the different terminals will be the same. An additive white Gaussian noise channel is assumed. Table 1 presents the configuration parameters for the simulations.

Table 1. Configuration parameters for the simulations

| Distance between T1-T2 | 1 |  |
| --- | --- | --- |
| Distance between T1-T3 | d |  |
| Distance between T3-T2 | 1-d |  |
| SNR in the T1-T2 link |  |  |
| SNR in the T1-T3 link |  |  |
| SNR in the T1-T3 link |  |  |
| Frame composition for OWRC transmissions | There are four time slots, 2 devoted for transmission from T1 (BS) to T2 (MS), and two devoted for transmissions from T2 (MS) to T1 (BS) |  |
| Types of OWRC transmissions | opt A | T1->T2 with OWRC-protocol III T2->T1 with OWRC-protocol III |
|  | opt B | T1->T2 with OWRC-protocol I T2->T1 with OWRC-protocol II |
| Frame composition for TWRC-forwarding | There are 2 time slots |  |
| Frame composition for TWRC-protocol I | There are 3 time slots |  |
| Frame duration | Normalized to 1 |  |
| Traffic asymmetry between traffic T2->T1 and T1->T2 | The traffic transmitted by the BS is 3 times higher than the traffic transmitted by the MS |  |

Figure 3 shows the sum-rate for different positions of the RS obtained with different protocols. It has optimized the duration of the time slots for the different phases of the transmission. We can see that opt.B-OWRC attains the best performance for almost all positions of the relay. The TWRC-protocol I get the best sum-rate when the relay is placed at 0.6<d0.75. Finally, Opt.A-OWRC improves the TWRC-protocol I when the RS is placed near of T2 (MS). It should be remarked that the OWRC transmissions are using more power than the TWRC transmissions.

Figure 3. Sum-rate obtained by T1 and T2 for different positions of the relay. Dynamic resource allocation of the time slots. A=10 dB.

If the time slots have a static an equal duration then the performance of the different protocols changes, as it is shown in Figure 4. In such a case, the best protocol for the TWRC depends on the position of the RS, for d0.35 protocol I is the best, while TWRC-forwarding is the best for d>0.35.

Figure 4. Sum-rate obtained by T1 and T2 for different positions of the relay. Static resource allocation of the time slots. A=10 dB.

Finally, Figure 5 shows the sum-rate of the different schemes when the same power is introduced in the three-terminal network (efficiency), and the resources are dynamically optimized (Figure 5-left) or the resources are static (Figure 5-right). In the first case, we observe that TWRC-protocol I get the best performance, while when the resources are static, the best protocol (TWRC-protocol I, opt A-OWRC or TWRC-forwarding) depend on the position of the relay.

Figure 5. Sum-rate obtained by T1 and T2 for different positions of the relay. Left) Static resource allocation of the time slots, Right) Static resource allocation of the time slots. Equal power used by all the schemes. A=10 dB.

With the previous examples we have illustrated the best transmission scheme depends on the topology geometry (position of the RS) and the resource allocation (dynamic or static). Therefore, the frame structure should support different configurations.

Currently, the IEEE 802.16m-08/003r5 considers two options for the Relay frame structure depicted in Figure 6. According to those options, unidirectional transmission (OWRC-protocol III) only can be implemented by means of Option 1. On the other hand, TWRC-forwarding is supported by Option 2 if the MS can be active during the bidirectional zone. Hence, the selection of one option limits the protocols to be introduced in the 802.16m standard.

To this end we propose a flexible relay frame structure which subsumes both options currently considered in the IEEE 802.16m-08/003r5 and allows new transmission schemes. Moreover, in contrast to the previous options where the transmission are predefined (i.e. BS to MS or BS to RS), we envision new control parameters for defining if the RS or MS are in transmitting or receiving mode.

Figure 6. Relay Frame structure defined in IEEE 802.16m-08/003r5. Left) Option 1, Right) Option 2.

# Description

The control parameters (of 1 bit) named RS_mode and MS_mode define if the terminal is receiving data (value equal to 0) or transmitting data (value equal to 1). In such a case, the meaning of DL (downlink) and UL (uplink) denotes if the BS is transmitting or receiving, respectively.

Those control parameters can be placed in the DL_MAP (downlink MAP) and UL_MAP (uplink MAP).

If there are more than one RS active, as is contemplated in the IEEE 802.16m (odd-hop and even-hop RS) we consider the following control parameters:

- RSo_mode:  odd-hop RS

- RSe_mode:  even-hop RS

- MS_mode

Figure 7. Implementation of the frame formats presented in Figure 3 with the proposed frame structure

Figure 7 shows that the relay frame formats considered in Figure 6 can be described by the proposed frame format. It must be noticed that the value of the MS_mode in the second subframe in the DL and UL for the option 2 does not mean anything because there is not any MS active in that subframe. Finally, Figure 8 sketches the implementation of OWRC-protocol III, TWRC-forwarding, and TWRC-protocol I.

Figure 8. Implementation of the unidirectional (OWRC-protocol III) (top) and bidirectional transmission (TWRC-forwarding and TWRC-protocol I) (bottom) with the proposed frame structure and a single relay active

The advantages of the proposed frame structure are:

- It accommodates unidirectional and bidirectional transmissions

- It is open for other possible transmission schemes

# Text for SDD

[line 26 of page 47]

There are two options for the Relay frame structure. These are captured in Figure 25 and Figure 26. Further study is required to distill a single frame structure from among these options.

The relay frame description includes the following control parameters

- RS_mode   (0=receiving,  1=transmitting)

- MS_mode  (0=receiving,  1=transmitting)

Both are placed in the DL_MAP and UL_MAP. Those parameters specify if the RS and MS are in receiving (parameter equal to 0) or transmitting mode (parameter equal to 1) in each subframe (or consecutive subframes). In such a case, at the BS the DL and UL subframes denote where the BS is transmitting or receiving data.

If odd-hop and even-hop RSs are devised the control parameters to be considered are:

- RSo_mode	(odd-hop)

- RSe_mode	(even-hop)

- MS_mode

[Delete Figure 25 and text between lines: 5 of page 48 and 14 of page 49]

[Delete Figure 26 and text between lines: 1 of page 50 and 2 of page 51]