| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | New Zone with High-Mobility Support for TGm Frame Structure |  |
| Date Submitted | 2008-03-10 |  |
| Source(s) | Yu-Tao Hsieh, Lai-Huei Wang, Jen-Yuan Hsu, Rong-Terng Juang, Pang-An Ting, Ren-Jr Chen, Yang-Xiu Zheng, Richard Li ITRI 195,Sec. 4, Chung Hsing Rd. Chutung, Hsinchu, Taiwan 310, R.O.C Hsi-Pin Ma, Yeong-Luh Ueng NTHU | Voice: + 886 3 5915642 E-mail: richard929@itri.org.tw |
| Re: | Call for Comments on C802.16m-08/118r1 |  |
| Abstract | This contribution proposes a mechanism to improve system performance in high mobility environments with high-mobility zones in the new 802.16m frame structure. |  |
| Purpose | For 802.16m discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

New Zone with High-Mobility Support for TGm Frame Structure

Yu-Tao Hsieh, Lai-Huei, Wang, Jen-Yuan Hsu, Rong-Terng Juang,
Pang-An Ting ,Ren-Jr Chen, Yang-Xiu Zheng, Richard Li

ITRI

Hsin-Pin Ma, Yeong-Luh Ueng

NTHU

# Introduction

The 802.16m system requirements [1] describes the system performance demand for high speed environments However, the WirelessMAN-OFDMA Reference system can support limited link performance under moderate to high speed scenarios. In order to support improved performance in higher speed (e.g., 350 Km/hr), we proposed to create a new type of transmission format with two major changes. Firstly, a new zone with high-mobility support in which the symbol duration was suggested to change [2]. The second revision is the extended cyclic prefix (CP) length, which can greatly help cancel the inter-carrier interference (ICI) in high mobility environments [3]. In this contribution more detailed performance comparisons for the proposed high-mobility zone are proposed. Simulation results show that the new zone with high-mobility support can provide the enhanced system throughput even under 350 km/hr and outperform the legacy OFDMA system.

# Numerology for the New Zone with High-Mobility Support

The TGm Frame Structure Rapporteur Group has drafted an initial version of frame structure [4]. OFDMA parameters for the frame structure were shown in Table 1. Here we propose another table of parameters suitable for the high-mobility zone in Table 2. When the high-mobility zone is assigned in the frame, the symbol duration was decreased to one half of the normal symbol, and the CP ratio is extended to 1/2 the new symbol duration. Note that the proposed architecture fits the length of the original sub-frame. Although the system overhead is higher than the original system design, however, the resistance to severe ICI of the proposed architecture will bring less packet errors and hence result in more system throughput.

Table 1. Regular OFDMA parameters

| Nominal Channel Bandwidth (MHz) |  | 5 | 7 | 8.75 | 10 | 20 |
| --- | --- | --- | --- | --- | --- | --- |
| Over-sampling Factor |  | 28/25 | 8/7 | 8/7 | 28/25 | 28/25 |
| Sampling Frequency (MHz) |  | 5.6 | 8 | 10 | 11.2 | 22.4 |
| FFT Size |  | 512 | 1024 | 1024 | 1024 | 2048 |
| Sub-Carrier Spacing (kHz) |  | 10.94 | 7.81 | 9.77 | 10.94 | 10.94 |
| Tu (us) |  | 91.4 | 128 | 102.4 | 91.4 | 91.4 |
| Cyclic Prefix (CP) | Ts (us) | Number of OFDM Symbols per Frame |  |  |  | Idle Time (us) |
| Tg=1/8 Tu | 91.4 +11.42= 102.82 (for 5, 10, 20 MHz) | 48 (for 5, 10, 20 MHz) |  |  |  | 62.86 |
|  | 128+16=144 (for 7 MHz) | 34 (for 7 MHz) |  |  |  | 104 |
|  | 102.4+12.8=115.2 (for 8.75 MHz) | 43 (for 8.75 MHz) |  |  |  | 46.40 |

Table 2. OFDMA parameters for the high-mobility zone

| Nominal Channel Bandwidth (MHz) |  | 5 | 7 | 8.75 | 10 | 20 |
| --- | --- | --- | --- | --- | --- | --- |
| Over-sampling Factor |  | 28/25 | 8/7 | 8/7 | 28/25 | 28/25 |
| Sampling Frequency (MHz) |  | 5.6 | 8 | 10 | 11.2 | 22.4 |
| FFT Size |  | 256 | 512 | 512 | 512 | 1024 |
| Sub-Carrier Spacing (kHz) |  | 21.88 | 15.62 | 19.54 | 21.88 | 21.88 |
| Tu (us) |  | 45.7 | 64 | 51.2 | 45.7 | 45.7 |
| Cyclic Prefix (CP) | Ts (us) | Number of OFDM Symbols per Frame |  |  |  | Idle Time per Frame (us) |
| Tg=1/2 Tu | 45.71+22.86= 68.57 (for 5, 10, 20 MHz) | 72 (for 5, 10, 20 MHz) |  |  |  | 62.86 |
|  | 64+32=96 (for 7 MHz) | 52 (51) (for 7 MHz) |  |  |  | 8 (104) |
|  | 51.2+25.6=76.8 (for 8.75 MHz) | 65 (64) (for 8.75 MHz) |  |  |  | 8 (84.8) |

# Simulation Results

In the following we compare the results of packet error rate (PER) and single-user throughput with PUSC permutation zone. The simulation setup is tabulated in Table 3. In the simulations the user is allocated 5 packets in a DL subframe, where the subframe durations for the WirelessMAN OFDMA system and the proposed system are set as similar for fairness of comparison. The overhead of 6 control symbols as indicated in the Evaluation methodology document is not accounted for in the calculation of throughput. The subcarrier allocation for the high-mobility zone can be found in Figure 1. The subcarrier permutation and allocation for 5 MHz was applied to the proposed high-mobility zone for channel bandwidth of 10 MHz. The effective subcarrier spacing is thus doubled to be 2x10.94 kHz. In Figure 2 and Figure 3 the system PER and throughput performances for the WirelessMAN OFDMA system are represented with user velocity of 350 km/hr. It can be seen that the link performance of 16QAM-1/2 is better than that of 64QAM-1/2. The PER deteriorates severely in the case of 64QAM-1/2 due to the high moving speed. The superior performance of 16QAM-1/2 may be attributed to its better PER and thus less packet re-transmission. Since the system throughput with 16-QAM is higher than QPSK, it can be conclude that the peak throughput of around 13.5 Mbps can be obtained with code rate of 16QAM-1/2. However, when the proposed zone is applied to the high speed environments, the severe ICI can be significantly suppressed such that the PER can be decreased by more than one order with modulation of 64-QAM. Please note the system performance expressed in Figure 4 and Figure 5. The PER is greatly enhanced with modulation of 64-QAM, and the throughput of more than 23Mbps using 64QAM-3/4 can be achieved.

Table 3. Simulation setup for the proposed new zone

| Channel Bandwidth | 10 MHz |
| --- | --- |
| Number of DL symbols | 22 (WirelessMAN OFDMA reference system) 32 (high-mobility zone) |
| FFT size | 512 |
| Number of data subcarriers | 360 |
| Number of pilots | 60 |
| Number of clusters | 30 |
| Permutation | PUSC |
| Number of packets per frame | 5 |
| Channel model | Modified Vehicular-A |
| HARQ type | Chase Combining |
| Maximum HARQ re-transmission | 4 |
| User velocity | 350 km/hr |
| Channel estimation | MMSE |

14

1

Pilot subcarrier

Data subcarrier

Figure 1. Subcarrier allocation for the proposed zone

Figure 2. PER performance of the Wireless-MAN OFDMA reference system

Figure 3. Single-user throughput of the Wireless-MAN OFDMA reference system

Figure 4. PER performance of the proposed system

Figure 5. Single-user throughput of the proposed system

# Proposed Text

------------------------------------------------------- Start of the proposed text ------------------------------------------------

11.3 OFDMA parameters

The proposed table for OFDMA parameters of the new zone with high-mobility support is given in Table xx.

Table xx. OFDMA parameters for the high-mobility zone

| Nominal Channel Bandwidth (MHz) |  | 5 | 7 | 8.75 | 10 | 20 |
| --- | --- | --- | --- | --- | --- | --- |
| Over-sampling Factor |  | 28/25 | 8/7 | 8/7 | 28/25 | 28/25 |
| Sampling Frequency (MHz) |  | 5.6 | 8 | 10 | 11.2 | 22.4 |
| FFT Size |  | 256 | 512 | 512 | 512 | 1024 |
| Sub-Carrier Spacing (kHz) |  | 21.88 | 15.62 | 19.54 | 21.88 | 21.88 |
| Tu (us) |  | 45.7 | 64 | 51.2 | 45.7 | 45.7 |
| Cyclic Prefix (CP) | Ts (us) | Number of OFDM Symbols per Frame |  |  |  | Idle Time per Frame (us) |
| Tg=1/2 Tu | 45.71+22.86= 68.57 (for 5, 10, 20 MHz) | 72 (for 5, 10, 20 MHz) |  |  |  | 62.86 |
|  | 64+32=96 (for 7 MHz) | 52 (51) (for 7 MHz) |  |  |  | 8 (104) |
|  | 51.2+25.6=76.8 (for 8.75 MHz) | 65 (64) (for 8.75 MHz) |  |  |  | 8 (84.8) |

------------------------------------------------------- End of the proposed text ------------------------------------------------

# References

[1] M. Cudak, “802.16m System Requirements, ” IEEE 802.16m-07/002r4

[2] R.-J. Chen, et al, “OFDMA Frame Structures with Scalable Bandwidth and High-Mobility Support for IEEE 802.16m, ” IEEE 802.16m-07/295r1.

[3] Y.-T. Hsieh et al, “ Extended Cyclic Prefix for High-Mobility Support of IEEE 802.16m Frame Structure,” IEEE 802.16m-08/025r3

[4] S. Ahmadi et al, “Initial Document output of the IEEE 802.16 TGm Frame Structure Rapporteur Group”, IEEE 802.16m-08/118r1

[5] R. Srinivasan, et al, “Draft IEEE 802.16m Evaluation Methodology”, IEEE 802.16m-07/037r2.